Home Network Transport

Standards Overview and Work Plan

December 2021 Q1/15 meeting

Contact persons for project updates:

| Study Group 15 Advisor: Mr. Hiroshi Ota International Telecommunication Union (ITU) Place des Nations 1211 Geneva 20 Switzerland Tel.: +41 22 730 6356 E-mail: hiroshi.ota@itu.int | Study Group 15 Chairman Dr. Stephen J. Trowbridge Nokia 630 Terrace Avenue, Unit F Boulder, CO 80304-4843 USA Tel: +1 303 809 7423 E-mail: steve.trowbridge@nokia.com | Question 1/15 Rapporteur Mr. Jean-Marie Fromenteau Corning Incorporated Corning, NY 14831 USA Tel: +49 9561 42 74 20 E-mail: fromentejm@corning.com |
| --- | --- | --- |

Home Network Transport is an ITU-T Project dealing with studies and Recommendations on the Home Network.

# Home Network Transport Standards Overview and Work Plan

Issue 13, December 2021

# Revision Status Report: Major Updates of Version 13, December 2021

In this version 13, following changes, additions and updates have been made. 1) Section 1.1 - Existing Recommendations within ITU-T SG15 – WP1/15 Table 1 – “Published” Recommendations related to Home Network within ITU-T SG15 Question 18 - ITU-T SG15 Recommendations G.9903 (2017) Amd.1 (05/2021); G.9961 (2018) Amd.3 (04/2021); G.9963 (2018) Amd.1 (04/2021); G.9991 (2019) Amd.2 (04/2021 have been added. - ITU-T SG15 Technical Papers GSTP-FTTR (04/2021) and GSTP-HNAFS (04/2021) have been added. ITU-T SG15 Technical Paper TP G.HN SG Appl. (06/2010) has been deleted. 2) Section 1.2 - Recommendations “Under Study” within ITU-T SG15 – WP1 Table 2– Recommendations “Under Study” related to Home Network - The list of items “Under Study” has been updated with the latest Q18/15 activities resulting from December 2021 SG15 WP1 plenary meeting. 3) Section 1.3 - Home Network Activities within other ITU-T Study Groups Table 3 - “ITU-T study groups working on Home Network related topics” - ITU-T SG5: new Recommendation L.1383 (10/2021); Supplements K Suppl. 3 (10/2015); K suppl. 15 (09/2018); K Suppl. 21 (05/2021) and K Suppl. 25 (05/2021) have been added. - ITU-T SG9 Recommendation list related to Home Network have been reviewed and updated: J.1612 (consented on 11/2021) and J.198.1 (consented on 11/2021) have been added. - ITU-T SG16: Recommendations H.720 (10/2008); H.721 (04/2015); H.722 (01/2014); H.723 (07/2016); H.724 (12/2017) and H.772 (11/2015) have been added. - ITU-T SG17: Recommendations X.1113 (11/2007) and X.1332 (03/2020) have been added. 4) Section 2 - Other groups within ITU and other Standard Developing Organizations Table 4 - Home Network related activities in other groups within ITU-R & other SDOs - ETSI TC BRAN: ETSI EN 302 567 V2.2.1 (2021-07) and TR 103 583 V1.1.1 (2019-08) been updated. - ETSI TC DECT: ETSI TS 103 636-1 V1.2.1 (2021-04); TS 103 636-2 V1.2.1 (2021-04); TS 103 636-3 V1.2.1 (2021-04) and TS 103 636-4 V1.2.1 (2021-04) have been added. - ETSI ISG F5G: ETSI GS F5G 009 V1.1.1 (2021-06) and GS F5G 003 V1.1.1 (2021-09) have been added. - CENELEC CLC/TR 50682:2018 has been added. CENELEC EN 50173-1:2018 and EN 50173-4:2018 have been added. CENELEC EN 50090-1:2011; EN 50491-6-1:2014; EN 63044-1:2017; EN 63044-1:2017/A1:2021; EN IEC 63044-6:2021 have been added. - IEC 62807-1:2017; IEC 60825-2:2021; IEC 60825-2:2021/COR1:2021; IEC 60793-2-50:2018; IEC 60794-2:2017; IEC TR 62000:2021 and IEC TR 62362:2020 have been added. - IEEE 802.3 has been updated. - IEEE 802.11ax-2021 and IEEE 802.11ay-2021 have been updated/added. - IEEE 802.15.7-2018 and IEEE P802.15.13/D5.0, Sept 2021 have been added - DRAFT IEEE 1901b Sept. 2021 has been added. - IEEE 1905.1-2013 and IEEE 1905.1a-2014 have been added. - ISO-IEC JTC 1/SC 25 has been reviewed and updated. - Zigbee Alliance rebranded as Connectivity Standards Alliance has been updated. 5) Section 3 - Web-Based Home Network Transport (HNT) Standards Overview has been updated. 6) Annex 1 “Overview of Home Network applications” and Annex 2 “List of Contacts” have been updated.

# Home Network Transport Standards Overview and Work Plan

# Issue 13, December 2021

General…	5

Introduction	5

1. Home Network Activities in ITU-T	6

1.1 Existing Recommendations within ITU-T SG15 – WP1/15 “Transport aspects of       
access, home and smart grid networks”………………………………………………….6

1.2. Recommendations “Under Study” within ITU-T SG15 – WP1 “Transport aspects of
 access, home and smart grid   networks”………………………………………………..11

1.3. Home Network Activities within other ITU-T Study Groups…………………………...13
[ITU-T SG5; SG9; SG13; WP2 of SG15; SG16; SG17; SG20]

2. Home Network Activities within ITU-R and in other Standards Development
Organizations	23
[ITU-R SG1; SG5; SG6]
[Broadband Forum; Bluetooth®; ETSI; HomeGrid Forum, CENELEC; IEC; IEEE 802.3; 
IEEE 802.11; IEEE 802.15; IEEE 1901; IEEE 1905; ISO/IEC; MoCA®; TIA; TTC;
CSA (ex. Zigbee); Z-Wave]

3. Web-Based Home Network Transport (HNT) Standards Overview………………………59

Annex 1 - Overview of Home Networks applications………………………………………..62

Annex 2 - List of Contacts	64

HOME NETWORK TRANSPORT STANDARDS OVERVIEW AND WORK PLAN

Issue 13, DECEMBER 2021

# General

The HNT Standards Overview and Work Plan is a living document. The actual version is available at https://www.itu.int/en/ITU-T/studygroups/Pages/sg15-hnt.aspx .
A web-based HNT Standards Overview completes the present document and is available at
https://www.itu.int/net4/ITU-T/landscape#?topic=0.132&workgroup=1&searchValue=&page=1&sort=Revelance .

# Introduction

The continuing customer demand for ever higher bit rate data services, high-speed Internet access and other innovative services, and the ongoing needs of network operators to leverage in-premises connectivity for distributing within the home IPTV and other services and applications, require the development of new Recommendations and enhancements to existing Recommendations covering all aspects of in-premises networking transceivers. These studies include, but are not limited to, the transport of higher layer protocols, the management and test of the in-premises systems, spectral management aspects and energy saving techniques.

Given the interdisciplinary nature of Home Network applications, it is expected that a high degree of cooperation with other ITU Sectors (ITU-R, ITU-D), ITU-T Study Groups, Questions, Focus Groups (FGs), Joint Coordination Activities (JCAs), Global Strategic Initiatives (GSIs), as well as other international standards bodies will be required.

Within the ITU-T, the study and development of Recommendations related to transport in the Home Network is being carried out in a number of different Study Groups - see in Section 1 Tables 1, 2 and 3 below - e.g.  SGs 5, 9, 13, 15, 16, 17 and 20. Also ITU-R and other standards bodies, forums and consortia are active in this area - see in Section 2 Table 4 below - for a list of such activities.

Recognizing that without a strong coordination effort there is the danger of duplication of work as well as the development of incompatible and non-interoperable standards, the ITU-T designated Study Group 15 as the Lead Study Group on Home Network.

To represent the multitude of home networking technologies and services offered to the customer,  the Table 5 in Annex 1 provides an overview of Home Network applications and related Home Network technologies.

A critical part of the HNT standardization work is the network of contacts for the development of relevant standards and the tracking of correspondence, usually in the form of liaisons, to guide the work. The list of contacts in Annex 2 provides an overview of the other relevant Study Groups in the ITU and other Standardization Groups corresponding through liaisons with Study Group 15 about matters related to the HNT Standards Overview and Work Plan.

# 1.   Home Network Activities within ITU-T

# 1.1 Existing Recommendations within ITU-T SG15 – WP1/15 “Transport aspects of access, home and smart grid   networks”

“Published” Recommendations related to “Technologies for in-premises networking and related access applications” are listed here for convenience in Table 1.
Note: During its 27 January – 7 February 2020 meeting, ITU-T Study Group 15 agreed to merge Question 15 “Communications for smart grid” into Question18 “Broadband in-premises networking” with the new title “Technologies for in-premises networking and related access applications”.

Status:  December  2021

Table 1 – “Published” Recommendations related to Home Network within ITU-T SG15 Question 18

| Rec. No. | Title | Status | Date |
| --- | --- | --- | --- |
| G.9901 (06/2017) | Narrowband orthogonal frequency division multiplexing power line communication transceivers - Power spectral density specification | Published | 06/2017 |
| G.9902 (10/2012) | Narrowband orthogonal frequency division multiplexing power line communication transceivers for ITU-T G.hnem networks | Published | 10/2012 |
| G.9902 (2012) Amd.1 (03/2013) | Narrowband orthogonal frequency division multiplexing power line communication transceivers for ITU-T G.hnem networks Amendment 1 | Published | 03/2013 |
| G.9902 (2012) Amd.2 (08/2013) | Narrowband orthogonal frequency division multiplexing power line communication transceivers for ITU-T G.hnem networks Amendment 2: Clarifications on payload encoder and addition of a network admission procedure | Published | 08/2013 |
| G.9903 (08/2017) | Narrow-band OFDM power line communication transceivers for G3-PLC networks | Published | 08/2017 |
| G.9903 (2017) Amd.1 (05/2021) | Narrowband orthogonal frequency division multiplexing power line communication transceivers for G3-PLC networks Amendment 1 | Published | 05/2021 |
| G.9904 (10/2012) | Narrowband orthogonal frequency division multiplexing power line communication transceivers for PRIME networks | Published | 10/2012 |
| G.9905 (08/2013) | Centralized metric-based source routing | Published | 08/2013 |
| G.9905 (2013) Amd.1 (11/2016) | Centralized metric-based source routing Amendment 1 | Published | 11/2016 |
| G.9951 (02/2001) | Phoneline networking transceivers - Foundation | Published | 02/2001 |
| G.9952 (11/2001) | Phoneline networking transceivers - Payload format and link layer requirements | Published | 11/2001 |
| G.9953 (03/2003) | Phoneline networking transceivers - Isolation function | Published | 03/2003 |
| G.9954 (01/2007) | Home networking transceivers - Enhanced physical, media access, and link layer specifications | Published | 01/2007 |
| G.9958 (03/2018) | Generic architecture of home networks for energy management | Published | 03/2018 |
| G.9959 (01/2015) | Short range narrow-band digital radiocommunication transceivers - PHY and MAC layer specifications | Published | 01/2015 |
| G.9960 (11/2018) | Unified high-speed wireline-based home networking transceivers - System architecture and physical layer specification | Published | 11/2018 |
| G.9960 (2018) Cor.1 (09/2019) | Unified high-speed wire-line based home networking transceivers - System architecture and physical layer specification Corrigendum 1 | Published | 09/2019 |
| G.9960 (2018) Amd.1 (02/2020) | Unified high-speed wire-line based home networking transceivers - System architecture and physical layer specification Amendment 1 | Published | 02/2020 |
| G.9960 (2018) Amd.2 (07/2020) | Unified high-speed wireline-based home networking transceivers - System architecture and physical layer specification Amendment 2 | Published | 07/2020 |
| G.9960 (2018) Cor.2 (10/2020) | Unified high-speed wire-line based home networking transceivers - System architecture and physical layer specification Corrigendum 2 | Published | 10/2020 |
| G.9961 (11/2018) | Unified high-speed wireline-based home networking transceivers - Data link layer specification | Published | 11/2018 |
| G.9961 (2018) Cor.1 (09/2019) | Unified high-speed wireline-based home networking transceivers - Data link layer specification Corrigendum 1 | Published | 09/2019 |
| G.9961 (2018) Amd.1 (02/2020) | Unified high-speed wireline-based home networking transceivers - Data link layer specification Amendment 1 | Published | 02/2020 |
| G.9961 (2018) Cor.2 (03/2020) | Unified high-speed wireline-based home networking transceivers - Data link layer specification Corrigendum 2 | Published | 03/2020 |
| G.9961 (2018) Amd.2 (07/2020) | Unified high-speed wireline-based home networking transceivers - Data link layer specification Amendment 2 | Published | 07/2020 |
| G.9961 (2018) Amd.3 (04/2021) | Unified high-speed wireline-based home networking transceivers - Data link layer specification Amendment 3 | Published | 04/2021 |
| G.9962 (11/2018) | Unified high-speed wire-line based home networking transceivers - Management specification | Published | 11/2018 |
| G.9962 (2018) Cor.1 (03/2020) | Unified high-speed wire-line based home networking transceivers - Management specification Corrigendum 1 | Published | 03/2020 |
| G.9962 (2018) Amd.1 (07/2020) | Unified high-speed wire-line based home networking transceivers - Management specification Amendment 1 | Published | 07/2020 |
| G.9963 (11/2018) | Unified high-speed wireline-based home networking transceivers - Multiple input/multiple output specification | Published | 11/2018 |
| G.9963 (2018) Amd.1 (04/2021) | Unified high-speed wireline-based home networking transceivers - Multiple input/multiple output specification Amendment 1 | Published | 04/2021 |
| G.9964 (12/2011) | Unified high-speed wireline-based home networking transceivers - Power spectral density specification | Published | 12/2011 |
| G.9964 (2011) Amd.1 (02/2016) | Unified high-speed wire-line based home networking transceivers - Power spectral density specification Amendment 1 | Published | 02/2016 |
| G.9964 (2011) Amd.2 (09/2016) | Unified high-speed wire-line based home networking transceivers - Power spectral density specification Amendment 2 | Published | 09/2016 |
| G.9964 (2011) Amd.3 (02/2020) | Unified high-speed wireline-based home networking transceivers - Power spectral density specification Amendment 3 | Published | 02/2020 |
| G.9970 (01/2009) | Generic home network transport architecture | Published | 01/2009 |
| G.9971 (07/2010) | Requirements of transport functions in IP home network | Published | 07/2010 |
| G.9972 (06/2010) | Coexistence mechanism for wireline home networking transceivers | Published | 06/2010 |
| G.9972 (2010) Cor.1 (04/2014) | Coexistence mechanism for wireline home networking transceivers Corrigendum 1: Revised definition of coexisting systems categories | Published | 04/2014 |
| G.9973 (08/2017) | Protocol for identifying home network topology | Published | 08/2017 |
| G.9977 (02/2016) | Mitigation of interference between DSL and PLC | Published | 02/2016 |
| G.9977 (2016) Cor.1 (08/2017) | Mitigation of interference between DSL and PLC: Corrigendum 1 | Published | 08/2017 |
| G.9978 (11/2018) | Secure admission in a G.hn network | Published | 11/2018 |
| G.9979 (11/2018) | Implementation of the generic mechanism in the IEEE 1905.1a-2014 standard to include applicable ITU-T Recommendations | Published | 11/2018 |
| G.9980 (11/2012) | Remote management of customer premises equipment over broadband networks – Customer premises equipment WAN management protocol | Published | 11/2012 |
| G.9991 (03/2019) | High speed indoor visible light communication transceiver - System architecture, physical layer and data link layer specification | Published | 03/2019 |
| G.9991 (2019) Amd.1 (07/2020) | High-speed indoor visible light communication transceiver - System architecture, physical layer and data link layer specification Amendment 1 | Published | 07/2020 |
| G.9991 (2019) Cor.1 (10/2020) | High-speed indoor visible light communication transceiver - System architecture, physical layer and data link layer specification Corrigendum 1 | Published | 10/2020 |
| G.9991 (2019) Amd.2 (04/2021) | High-speed indoor visible light communication transceiver - System architecture, physical layer and data link layer specification Amendment 2 | Published | 04/2021 |
| G.9992 (03/2019) | Indoor optical camera communication transceivers - System architecture, physical layer and data link layer specification | Published | 03/2019 |
| G Suppl. 57 (07/2015) | Smart home profiles for 6LoWPAN devices | Published | 07/2015 |
| ITU-T SG15 Technical Paper TPLS.G-HN (07/2015) | Operation of G.hn technology over access and in-premises phone line medium | Published | 07/2015 |
| ITU-T SG15 Technical Paper GSTP-HNIA (02/2020) | Use of G.hn in Industrial Applications | Published | 02/2020 |
| ITU-T SG15 Technical Paper GSTP-HNSG (09/2020) | Technical paper on the use of G.hn technology for smart grid | Published | 09/2020 |
| ITU-T SG15 Technical Paper Wireline Broadband (12/2011) | Wireline broadband access networks and home networking | Published | 12/2011 |
| ITU-T SG15 Technical Paper GSTP-FTTR (04/2021) | Use cases and requirements of fibre-to-the-room (FTTR) | Published | 04/2021 |
| ITU-T SG15 Technical Paper GSTP-HNAFS (04/2021) | Architecture, functions and services of home network | Published | 04/2021 |
| J.190 (07/2007) | Architecture of MediaHomeNet | Published | 07/2007 |
| J.191 (03/2004) | IP feature package to enhance cable modems | Published | 03/2004 |
| J.192 (11/2005) | A residential gateway to support the delivery of cable data services | Published | 11/2005 |

# 1.2. Recommendations “Under Study” within ITU-T SG15 – WP1 “Transport aspects of access, home and smart grid   networks”

# Recommendations “Under Study” related to “Technologies for in-premises networking and related access applications” are listed here for convenience in Table 2.
Note: During its 27 January – 7 February 2020 meeting, ITU-T Study Group 15 agreed to merge Question 15 “Communications for smart grid” into Question 18“Broadband in-premises networking” with the new title “Technologies for in-premises networking and related access applications”.

Status:  December  2021

Table 2 – Recommendations “Under Study” and related to Home Network within ITU-T SG15 Question 18

Table updated with the latest Q18 activities resulting from December 2021 SG15 WP1 plenary meeting

| Work item | Question | Status | Timing | Approval process | Subject / Title | Base text(s) | Editor(s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| G.9960 Amd.3 | Q18/15 | Consented | 2021-12 | AAP | Unified high-speed wire-line based home networking transceivers - System architecture and physical layer specification - Amendment3 | SG15-TD825/PLEN (2021-12) | Les Brown |
| G.9961 Amd.4 | Q18/15 | Consented | 2021-12 | AAP | Unified high-speed wireline-based home networking transceivers Data link layer specification: Amendment 4 | SG15-TD827/PLEN (2021-12) | Les Brown |
| G.9962 Amd2 | Q18/15 | Under Study | 2022-09 | AAP | Unified high-speed wireline-based home networking transceivers Management Specification: Amendment 2 | TD-697/WP1 | Marcos Martínez (MaxLinear) |
| G.9976 (ex G.uvs) | Q18/15 | Approved | 2021-12 | AAP | Support UHD video service over G.hn | SG15-TD790R1/PLEN (2021-12) | Wu Jia, Tony Zeng |
| G.9978 Amd.1 | Q18/15 | Consented | 2021-12 | AAP | Secure admission in G.hn network - Amendment 1 | SG15-TD826/PLEN (2021-12) | Les Brown |
| G.fin-DLL | Q18/15 | Under study | 2022-09 | AAP | High speed fibre-based in-premises transceivers data link layer | TD731/WP1 | Xinrui Shi (China Telecom), Wu Jia (China Unicom) |
| G.fin-NM | Q18/15 | Under study | 2022-09 | AAP | High speed fibre-based in-premises transceivers network management | No draft yet | Yue Sun (China Unicom), Shuzhi Gou (CAICT) |
| G.fin-PHY | Q18/15 | Under study | 2022-09 | AAP | High speed fibre-based in-premises transceivers physical layer | No draft yet | Junwei Li (China Mobile), Xuming Wu (Huawei) |
| G.fin-SA | Q18/15 | Under study | 2022-09 | AAP | High speed fibre-based in-premises transceivers - system architecture | No draft yet | Qiang Cheng (CAICT), Xinrui Shi (China Telecom) |
| G.hetnet | Q18/15 | Under study | 2022-09 | AAP | Terminology & overview of the architecture of Heterogeneous Home Networks | TD698/WP1 | Marcos Martinez, Tony Zeng |
| G.hn2 | Q18/15 | Under study | TBD | AAP | Evolution of unified high-speed wire-line based home networking transceivers | TD714/WP1 | Tony Zeng |
| G.IoT | Q18/15 | Under study | TBD | AAP | System architecture, PHY layer and DLL layer for IoT Smart Home over PLC | SG15-TD614/WP1 (2021-04) | Yue Sun, Tony Zeng |
| G.uvs-xR | Q18/15 | Under study | TBD | AAP | Technical requirements of extended reality service over in-premises networks | TD730/WP1 | Wu Jia (China Unicom) |
| TP-GHN | Q18/15 | Agreed | 2021-12 | Agreement | Technical paper: Overview of the G.hn technology | SG15-TD822R1/PLEN (2021-12) | Tony Zeng |
| TP-UC-HN | Q18/15 | Under study | 2022-09 | Agreement | Technical paper on the use of ITU-T G.hn technology for in-home networking | SG15-TD700/WP1 (2021-12) | Marcos Martinez |
| TP-VLC | Q18/15 | Under study | 2022-09 | Agreement | Technical paper on the use of ITU-T Visible Light Communication technology | SG15-TD701/WP1 (2021-12) | Marcos Martinez |
| SUP-FTTR-4B | Q18/15 | Under study | 2022-09 | Agreement | Supplement on uses cases for FTTR 4B | C2638 (2021-12) | Dezhi Zhang (China Telecom), Tony Zeng (Huawei) |

ITU-T Q18/15 Project FTTR (Fibre to the room): Question 18 of ITU-T SG15 organised a Joint ETSI ISG F5G (European Telecommunications Standards Institute – Industry Specification Group - Fifth Generation Fixed Network), BBF (Broadband Forum), CCSA TC6 (China Communications Standards Association – Technical Committee 6) and ITU-T SG15 (International Telecommunication Union – Telecommunication Standardization Sector – Study Group 15) Workshop on “FTTR” (Fibre to the room) on 14 June 2021.
For more information see workshop programme and Q18/15 presentation at
https://www.itu.int/en/ITU-T/Workshops-and-Seminars/2021/0614/Pages/default.aspx .

# 1.3. Home Network Activities within other ITU-T Study Groups

This section provides an overview of the existing Home Network related activities within other ITU-T Study Groups.

Table 3 below gives an updated overview of such activities.
Status:  December  2021 
                                                       Table 3 – Home Network related activities within other ITU-T Study Groups

| Items | SGs and aspects |  |
| --- | --- | --- |
| Environment, climate change and circular economy continuation of Environment, climate change and circular economy continuation of Environment, climate change and circular economy | SG5 SG5 SG5 | SG5 “Environment, climate change and circular economy” SG15 TD 548 WP1 April 2021 ITU-T SG5 informs ITU-T SG15 that it has reviewed the Roadmap and for the time being, ITU-T SG5 does not have any update to provide regarding the Home Network Transport (HNT) Standards Overview and Work Plan. WP1/5 “EMC, lightning protection, EMF” Published Recommendations related to Home Network - ITU-T K.66 (11/2019): Protection of customer premises from overvoltages - ITU-T K.146 (06/2020): Management of interferences on telecommunication transmissions on copper other than speech. This Recommendation deals with the management of electromagnetic interference produced by electrified railways traction systems on telecommunication systems in DSL frequency band. - ITU-T K.21 (07/2019) and Amd.1 (06/2020): Resistibility of telecommunication equipment installed in customer premises to overvoltages and overcurrents. This Recommendation has enhanced test level values for Ethernet and PoE ports where the IEEE 802.3 LAN A conditions are not met. - ITU-T K Suppl. 3 (10/2015): ITU-T K.20, K.21, K.45, K.82 – Additional criteria to protect telecommunication cabling during a power cross event - ITU-T K Suppl. 15 (09/2018): ITU-T K.20, K.21 and K.44 – Internal DC powering interface surge testing factors - New Supplement ITU-T K Suppl. 21 (05/2021): ITU-T K.21 – Rationale for setting resistibility requirements of telecommunication equipment installed in customer premises against lightning - ITU-T K.98 (08/2014) and Cor.1 (03/2015), Cor.2 (06/2020): Overvoltage protection guide for telecommunication equipment installed in customer premises. This Recommendation analyses equipment port surge voltages for a.c. mains configuration types of TN-S, TN-C, TN-C-S, TT and IT resulting from a lightning strike to either the telecommunications line or the a.c. mains supply. - ITU-T K.147 (06/2020) and Cor.1 (01/2021): Ethernet port resistibility testing for overvoltages and overcurrents. This Recommendation covers the different “IEEE 802.3:2018” Ethernet twisted pair implementations, their configurations, how surges are coupled into the system and what surge mitigation measures are used. Both Power over Ethernet, PoE, and Power over Data Line, PoDL, are covered. Following the overview, the rational is given for the Recommendation ITU-T K series surge and power fault test circuit approaches. - ITU-T K.117 (12/2016): Primary protector parameters for the surge protection of equipment Ethernet ports. This Recommendation specifies the common-mode, differential mode and common mode to differential mode conversion surge parameters and test circuit requirements of an Ethernet port primary protector. The preferred surge generator voltage levels are 2.5 kV, 6 kV and 12 kV, but the test circuits can be used for any surge voltage environmental. Power over Ethernet (PoE) feed requirements are also given. Ethernet signal performance parameters are not covered. - New Supplement ITU-T K Suppl. 25 (05/2021): ITU-T K.117 – Long reach single twisted-pair Ethernet resistibility testing - ITU-T K.34 (12/2020): Classification of electromagnetic environmental conditions for telecommunication equipment – Basic EMC Recommendation - ITU-T K.74 (03/2015): Electromagnetic compatibility, resistibility and safety requirements for home network devices. This Recommendation provides the description and references to harmonize the newly developed Recommendation and EMC standards. - ITU-T K.85 (11/2011): Requirements for the mitigation of lightning effects on home networks installed in customer premises - ITU-T K.92 (05/2012): Conducted and radiated electromagnetic environment in home networking - ITU-T K.93 (12/2016): Immunity of home network devices to electromagnetic disturbances. This Recommendation provides test method for home network equipment against broadband noises. - ITU-T K.106 (03/2015): Techniques to mitigate interference between radio devices and cable or equipment connected to wired broadband networks and cable television networks. This Recommendation provides guidance to solve interference problems in home networking environments between radio devices and the cable or equipment connected to wired broadband networks and/or cable television networks. WP2/5 “Environment, Energy Efficiency and the Circular Economy” Published Recommendations related to Home Network - ITU-T L.1310 (09/2020): Energy efficiency metrics and measurement for telecommunication equipment. This Recommendation contains Clause 12 dedicated to energy efficiency metric for small networking devices. - ITU-T L.1340 (02/2014): Energy efficiency reference values for telecommunication equipment and infrastructure. This Recommendation contains Clause 9.1 on informative values for small networking devices. - ITU-T L.1383 (10/2021) “Smart energy solutions for cities and home applications” includes specific smart energy applications in cities and homes such as energy sources, energy management functions, etc. - ITU-T L.1001 (11/2012): External universal power adapter solutions for stationary information and communication technology devices - ITU-T L.1006 (12/2016): Test suites for assessment of the external universal power adapter solutions for stationary information and communication technology - ITU-T L.1410 (12/2014): Methodology for environmental life cycle assessments of information and communication technology goods, networks and services Work programme of ITU-T SG5 can be found at following URL https://www.itu.int/itu-t/workprog/wp_search.aspx?sg=05 More information about ITU-T SG5 can be found at following URL https://www.itu.int/en/ITU-T/studygroups/2017-2020/05/Pages/default.aspx |
| Broadband cable and TV continuation of Broadband cable and TV | SG9 SG9 | SG9: Broadband cable and TV WP1/9 “Cable transport and terminals, including video and data” SG15 TD 705 WP1 December 2021 ITU-T SG9 informs that the draft new Recommendation ITU-T J.1612 “The Architecture for Smart Home Gateway” has been AAP consented during its ITU-T SG9 meeting, 15-24 November 2021. This new Recommendation proposes the architecture for a Smart Home Gateway which may co-exist with a STB or a residential gateway, it may also exist as a standalone device in a home environment. SG15 TD 721 WP1 December 2021 ITU-T SG19 provides a review and updates of the listed Recommendations below. Published Recommendations related to Home Network (cable modem, cable set-up box, gateway) - ITU-T J.126 (12/2007): Embedded Cable Modem device specification - ITU-T J.128 (10/2008): Set-top Gateway specification for transmission systems for interactive cable television services - ITU-T J.190 (07/2007): Architecture of MediaHomeNet - ITU-T J.191 (03/2004): IP feature package to enhance cable modems - ITU-T J.192 (11/2005): A residential gateway to support the delivery of cable data services - ITU-T J.193 (06/2004): Requirements for the next generation of set-top-boxes - ITU-T J.195.1 (03/2016): Functional requirements for high speed transmission over coaxial networks connected with fibre to the building - ITU-T J.195.2 (10/2014): Physical layer specification for high speed transmission over coaxial networks - ITU-T J.195.3 (10/2014): Medium Access Control layer specification for high speed transmission over coaxial networks - ITU-T J.196.1 (03/2016): Functional requirements for second-generation HiNoC - ITU-T J.196.2 (10/2016): Physical layer specification of second generation HiNoC - ITU-T J.196.3 (10/2016): Media access control layer specification of second generation HiNoC - ITU-T J.198.1: Functional requirements for third-generation HiNoC (consented on 11/2021) - ITU-T J.290 (11/2006): Next generation set-top-box core architecture - ITU-T J.291 (11/2006): Next generation set-top-box cable architecture - ITU-T J.292 (11/2006): Next generation set-top-box media independent architecture - ITU-T J.293 (06/2008): Component definition and interface specification for the next generation set-top box - ITU-T J.294 (09/2010): Residential gateway requirements for the support of broadcast and IP-based interactive services over cable television networks - ITU-T J.295 (01/2012): Functional requirements for a hybrid cable set-top box - ITU-T J.296 (06/2012): Specification for hybrid cable set-top box - ITU-T J.297 (03/2018): Requirements and functional specification of cable set-top box for 4K ultra high definition television - ITU-T J.298 (03/2019): Requirements and technical specifications of a cable TV hybrid set-top box compatible with terrestrial and satellite TV transport - ITU-T J.1611 (01/2021): Functional requirements for Smart Home Gateway - ITU-T J.1612: The Architecture for Smart Home Gateway (consented on 11/2021) Work programme of ITU-T SG9 can be found at following URL https://www.itu.int/ITU-T/workprog/wp_search.aspx?sg=9 More information about ITU-T SG9 can be found at following URL https://www.itu.int/en/ITU-T/studygroups/2017-2020/09/Pages/default.aspx |
| Protocols and test specifications | SG11 | SG11: Signalling requirements, protocols, test specifications and combating counterfeit products SG15 TD 506 GEN December 2021 ITU-T Study Group 11 informs ITU-T SG15 about the reference table and list of ongoing pilot projects on Conformance and Interoperability testing (see at www.itu.int/go/reference-table and www.itu.int/go/pilot-projects) and invites to update the tables. Refer to the ANT Standards Work Plan (version 34) - section 6 “Basic field of activities related to the ANT Standards Overview” – item 5 – regarding the update from ITU-T SG15 related to technologies based on ITU-T Recs. from WP1/15. Work Programme of ITU-T SG11 can be found at following URL https://www.itu.int/ITU-T/workprog/wp_search.aspx?sg=11 More information about ITU-T SG9 can be found at following URL https://www.itu.int/en/ITU-T/studygroups/2017-2020/11/Pages/default.aspx |
| Future networks (& cloud) | SG13 | SG13: Future networks, with focus on IMT-2020, cloud computing and trusted network infrastructures WP3/13 “Network Evolution, Trust and Quantum Enhanced Networking” More information about ITU-T SG13 can be found at following URL https://www.itu.int/en/ITU-T/studygroups/2017-2020/13/Pages/default.aspx |
| Transport, access and home | SG15 | SG15: Transport, Access and Home WP1/15 “Transport aspects of access, home and smart grid networks” - Refer to Table 1 and Table 2 of present HNT Standards Overview and Work Plan document. WP2/15 “Optical technologies and physical infrastructures” Q5/15 ”Characteristics and test methods of optical fibres and cables, and installation guidance” Published Recommendations related to Home Network - ITU T G.657 (11/2016): Characteristics of a bending-loss insensitive single-mode optical fibre and cable - ITU-T L.103 (04/2016): Optical fibre cables for indoor applications - ITU-T L.104/L.67 (10/2006): Small count optical fibre cables for indoor applications - ITU-T L.111 (10/2020): Optical fibre cables for in-home applications Work programme of ITU-T SG15 WP2/15 can be found at URL https://www.itu.int/ITU-T/workprog/wp_search.aspx?sg=15 - Revision of ITU-T L.400 “Optical fibre splices” and new Recommendation L.209 “Requirements for fibre optic network terminal box” have been consented at the ITU-T SG15 6-17 December 2021 meeting. More information about ITU-T SG15 can be found at following URL https://www.itu.int/en/ITU-T/studygroups/2017-2020/15/Pages/default.aspx |
| Multimedia | SG16 | SG16: Multimedia WP1/16 “Multimedia content delivery” SG15 TD 642 WP1 December 2021 ITU-T SG16 informs ITU-T SG15 that it has reviewed the published Home Network Transport Standards Overview and Work plan and did not identify any issues, gaps or overlaps. Further ITU-T SG16 informs that following approved Recommendations are applicable to Home Networks: - ITU-T H.720 (10/2008): Overview of IPTV terminal devices and end systems - ITU-T H.721 (04/2015): IPTV terminal devices: Basic model - ITU-T H.722 (01/2014): IPTV terminal device: Full-fledged model - ITU-T H.723 (07/2016): IPTV terminal device: Mobile model - ITU-T H.724 (12/2017): IPTV terminal device: Interworking-enabled model of multiple devices - ITU-T H.772 (11/2015): IPTV terminal device discovery ITU-T SG16 also informs ITU-T SG15 on its ongoing work and recommends to awaits the finalization of two recent draft recommendations related to HNT: - H.IPTV-TDES.6 "IPTV Terminal Device: Virtualized model" - H.IPTV-TDES.7 "IPTV Terminal Device: Home gateway integrated model" Published Recommendations related to Home Network - ITU-T H.622 (06/2008): A generic home network architecture with support for multimedia services - ITU-T H.622.1 (10/2008): Architecture and functional requirements for home networks supporting IPTV services - ITU-T H.610 (07/2003): Full service VDSL – System architecture and customer premises equipment - ITU-T H.622.2 (11/2015): Service capabilities and framework for virtual home networks Work programme of ITU-T SG16 can be found at following URL https://www.itu.int/itu-t/workprog/wp_search.aspx?sg=16 More information about ITU-T SG16 can be found at following URL https://www.itu.int/en/ITU-T/studygroups/2017-2020/16/Pages/default.aspx |
| Security | SG17 | SG17: Security WP2/17 “5G, IoT and ITS security” Published Recommendations related to Home Network - ITU-T X.1111 (02/2007): Framework of security technologies for home network - ITU-T X.1112 (11/2007): Device certificate profile for the home network - ITU-T X.1113 (11/2007): Guideline on user authentication mechanisms for home network services - ITU-T X.1114 (11/2008): Authorization framework for home networks - ITU-T X.1331 (03/2018): Security guidelines for home area network (HAN) devices in smart grid systems - ITU-T X.1197 (2012), Amd.1 (09/2019) and Erratum 1 (03/2020): Guidelines on criteria for selecting cryptographic algorithms for IPTV service and content protection - ITU-T X.1332 (03/2020): Security guidelines for smart metering services in smart grids Work programme of ITU-T SG17 can be found at following URL https://www.itu.int/itu-t/workprog/wp_search.aspx?sg=17 More information about ITU-T SG17 can be found at following URL https://www.itu.int/en/ITU-T/studygroups/2017-2020/17/Pages/default.aspx |
| Internet of things (IoT) and smart cities and communities (SC&C) | SG20 | SG20: Internet of things (IoT) and smart cities and communities (SC&C) WP1/20 Published Recommendations related to Home Network - ITU-T Y.4410/Y.2291 (01/2011): Architectural overview of next generation home networks - ITU-T Y.4415 (06/2018):Architecture of web of objects-based virtual home network - ITU-T Y.4469 (08/2020): Reference architecture of spare computational capability exposure of IoT devices for smart home - ITU-T Y.4465 (01/2020): Framework of Internet of things services based on visible light communications - ITU-T Y.4474 (08/2020): Functional architecture for Internet of things services based on visible light Communications Work programme of ITU-T SG20 can be found at following URL https://www.itu.int/ITU-T/workprog/wp_search.aspx?sg=20 More information about SG20 can be found at following URL https://www.itu.int/en/ITU-T/studygroups/2017-2020/20/Pages/default.aspx |

# 2. Home Network Activities within ITU-R and in other Standards Development Organizations

This following list is intended to improve understanding and communication of the on-going work related to Home Network  within ITU-R and in other Standardization Developing Organizations and may help identify possible gaps or overlaps.
Table 4 below gives an updated overview of such activities.

Status:  December  2021  
                                      Table 4 – Home Network related activities within ITU-R and in other Standard Developing Organizations

| Standards Org. | SGs, Committees, Aspects, Activities |  |
| --- | --- | --- |
| ITU-R continuation of ITU-R continuation of ITU-R continuation of ITU-R | SG1 WP1A SG1 WP1B | SG1 “Spectrum Management” WP1A “Spectrum engineering techniques” SG15 TD 704 WP1 December 2021 In its liaison TD 704 WP1 from December 2021, ITU-R WP1A informs that it has started work on a new Recommendation ITU-R SM.[Optical Wireless]. SG15 TD 554 WP1 April 2021 In its liaison in TD 554 WP1 from April 2021, ITU-R WP1A reports that it has noted several areas of current interest in the work plan of ITU-T SG15 Question 18 (Table 3 in the “Home Network Transport Standards Overview and Work Plan- Issue 11, September 2020”) particularly on the use of power wirelines for various narrow-band communication projects supporting smart grid utility management, smart metering and the internet of things. These can suffer disruption from various forms of electromagnetic interference and radio frequency noise, as do radiocommunication systems. ITU-R WP1A informs that it has started work on studies directed towards the “Evaluation of radiated electromagnetic disturbances of household appliances and their interferences over an IoT network” - (see attached document below “Preliminary draft report ITU-R SM.EMI-IoT). More information about ITU-R WP1A can be found at following URL https://www.itu.int/en/ITU-R/study-groups/rsg1/rwp1a/Pages/default.aspx WP1B “Spectrum management methodologies and economic strategies” - Report SM.2351: Smart grid utility management systems. This report provides an overview of smart grid communications and features, communication network technologies, reference architecture, wireline and wireless standards and references of deployments in several countries. More information on ITU-R WP1B can be found at following URL https://www.itu.int/en/ITU-R/study-groups/rsg1/rwp1b/Pages/default.aspx More information about ITU-R SG1 can be found at following URL https://www.itu.int/en/ITU-R/study-groups/rsg1/Pages/default.aspx |
|  | SG5 WP5A continuation of SG5 WP5A | SG5 “Terrestrial Services” WP5A “Land mobile service excluding IMT; amateur and amateur-satellite service” - Guide to the use of ITU-R texts relating to the land mobile service, including wireless access in the fixe service (last update: 29 July 2021): see at https://www.itu.int/oth/R0A06000001/en Radio Local Area Networks (RLANs) and Multiple Gigabit Wireless Systems (MGWS) WP 5A conducts work on radio local area networks (RLANs) and Multiple Gigabit Wireless Systems (MGWS) under Question ITU-R 212-4/5 . LANs/MGWS could be used to implement a home network. For a description of this work refer to: - section 8.2 “Nomadic Wireless Access” of the Guide to the use of ITU-R texts relating to the land mobile service, including wireless access in the fixed service, and in particular Recommendation ITU-R M.1450-5, which recommends characteristics of broadband radio local area networks, - section 8.4 “Multiple Gigabit Wireless Systems” of the Guide to the use of ITU-R texts relating to the land mobile service, including wireless access in the fixed service, and in particular Recommendation ITU-R M.2003-2, which provides general characteristics and radio interface standards for MGWS in frequencies around 60 GHz and ITU-R Report M.2227-2 which describes applications, deployment scenarios and technical characteristics of MGWS. - section 11: Cordless Telecommunication Systems of the Guide, and in particular Recommendation ITU-R M.1033-1 and Report ITU-R M.1025 which provides technical and operational characteristics of cordless telephones and cordless telecommunication systems. More information about ITU-R WP5A can be found at following URL https://www.itu.int/en/ITU-R/study-groups/rsg5/rwp5a/Pages/default.aspx More information about ITU-R SG5 can be found at following URL https://www.itu.int/en/ITU-R/study-groups/rsg5/Pages/default.aspx |
|  | SG6 WP6A | SG6 “Broadcasting Service” WP6A “Terrestrial broadcasting delivery” SG15 TD 547 WP1 April 2021 In its liaison in TD 547 WP1 from April 2021, ITU-R WP6A Rapporteur on “Power Line Telecommunications (PLT) and General EMC-Related Potential Interference Issues” reports that it has analysed the work plan of ITU-T SG15 Question 18 (Table 3 in the “Home Network Transport Standards Overview and Work Plan- Issue 11, September 2020”) and provides a compilation of its work in the document “Update on recent EMC related issues” attached below. ITU-R WP6A Rapporteur notes that no issues have been identified that would need an immediate response on possible concerns, but issues with PLT (in Q18 projects G.9904.1, G.Iot and G.uvs) have been identified where further monitoring, supported by studies by the broadcasting community, is essential. More information about ITU-R WP6A can be found at following URL https://www.itu.int/en/ITU-R/study-groups/rsg6/rwp6a/Pages/default.aspx More information about ITU-R SG6 can be found at following URL https://www.itu.int/en/ITU-R/study-groups/rsg6/Pages/default.aspx |
| continuation of ITU-R | SG6 WP6B | SG6 “Broadcasting Service” WP6B “Broadcast service assembly and access” More information about ITU-R WP6B can be found at following URL https://www.itu.int/en/ITU-R/study-groups/rsg6/rwp6b/Pages/default.aspx More information about ITU-R SG6 can be found at following URL https://www.itu.int/en/ITU-R/study-groups/rsg6/Pages/default.aspx |
| Broadband Forum | Broadband Forum https://www.broadband-forum.org SG15 TD 334 GEN Jan-Feb 2020 Broadband Forum Technical Committee Chair informs that on October 14, 2019, Broadband Forum took the final step to become ‘open’, adopting new Bylaws, which resulted in the Forum considering all its activities to be ‘open’ for the purposes of the U.S. Export Administration Regulations. The new Bylaws eliminate any restrictions on sharing Forum documentation and allow for anyone with interest to have access to Forum deliberations and documents if they apply for it. Information about Broadband Forum Projects “Connected Home” can be found at following URL https://www.broadband-forum.org/projects/connected-home Published Broadband Forum Technical Reports related to HNT can be found at following URL https://www.broadband-forum.org/technical-reports Broadband Forum Test Plans related to HNT can be found at following URL https://www.broadband-forum.org/test-plans Broadband Forum Work in Progress related to HNT can be found at following URL https://www.broadband-forum.org/broadband-forum-resources/work-in-progress ITU-T Q18/15 Project FTTR (Fibre to the Room): for more information see workshop programme and Broadband Forum presentation at https://www.itu.int/en/ITU-T/Workshops-and-Seminars/2021/0614/Pages/default.aspx |  |
| Bluetooth® | Bluetooth® Technology https://www.bluetooth.com/ - Bluetooth Core Specification – Revision: v5.2 - Revision Date: 2019-12-31 - Supplement to the Bluetooth Core Specification - Revision: v9 - Revision Date: 2019-12-31 The Bluetooth® Core Specification defines the technology building blocks that developers use to create the interoperable devices that make up the thriving Bluetooth ecosystem. The Bluetooth specification is overseen by the Bluetooth Special Interest Group (SIG) and is regularly updated and enhanced by Bluetooth SIG Working Groups to meet evolving technology and market needs. Bluetooth devices Power Class 1 have a transmit power of 100 mW and an operation range of typically ~ 100m. Devices power classes 2 and 3 have an operation range of typically ~10 m and ~1 m respectively and are regarded as WPAN devices. Bluetooth is a IEEE 802.15.1 based wireless system. More information about Bluetooth technology can be found at following URL https://www.bluetooth.com/specifications/bluetooth-core-specification/ and https://www.bluetooth.com/specifications/specs/ |  |
| ETSI TC ATTM continuation of ETSI TC ATTM | ETSI TC ATTM “Access, Terminals, Transmission and Multiplexing” https://www.etsi.org/committee/1390-attm SG15 TD 580 WP1 April 2021 ETSI ATTM AT2 informs that it will study the new version of the HNT Standards Overview and Work Plan in its next AT2 meeting. Published standards, technical specifications and reports related to Home Network - TS 105 175-1 V2.0.0 (2011-10): Access, Terminals, Transmission and Multiplexing (ATTM); Plastic Optical Fibre System Specifications for 100 Mbit/s and 1 Gbit/s - TS 105 175-1-1 V1.1.1 (2015-10): Access, Terminals, Transmission and Multiplexing (ATTM); Plastic Optical Fibres; Part 1: Plastic Optical Fibre System Specifications for 100 Mbit/s and 1 Gbit/s; Sub-part 1: Application requirements for physical layer specifications for high-speed operations over Plastic Optical Fibres - TS 105 175-1-2 V1.1.1 (2015-04): Access, Terminals, Transmission and Multiplexing (ATTM); Plastic Optical Fibres; Part 1: Plastic Optical Fibre System Specifications for 100 Mbit/s and 1 Gbit/s; Sub-part 2: 1 Gbit/s and 100 Mbit/s physical layer for Plastic Optical Fibres - TS 103 247 V1.2.1 (2018-11) : Access, Terminals, Transmission and Multiplexing (ATTM); Singlemode Optical Fibre System Specifications for Home Cabling - TR 102 994 V1.1.1 (2012-08) : Access, Terminals, Transmission and Multiplexing (ATTM); In Home Cabling for Integrated Broadband Cable and Television Services - TS 102 220 V1.1.1 (2014-04): Access and Terminals (AT); Technical Specification: Delivery of Cable based services across a home access to the devices in the home - TS 101 573 V1.1.1 (2012-09): Access, Terminals, Transmission and Multiplexing (ATTM); General engineering of optical building cabling - TS 102 873 V1.1.1 (2010-10): Access, Terminals, Transmission and Multiplexing (ATTM); Optical External Network Testing Interface Technical specifications related to Smart Grid - TS 104 001 V2.2.1 (2019-01): Open Smart Grid Protocol (OSGP); Smart Metering/Smart Grid Communication Protocol More information about ETSI ATTM can be found at following URL https://portal.etsi.org/TB-SiteMap/ATTM/ATTM-ToR Work Programme of ETSI ATTM can be found at following URL https://portal.etsi.org/tb.aspx?tbid=689&SubTB=689,693,851,706,694,695#/ |  |
| ETSI TC BRAN continuation of ETSI TC BRAN | ETSI TC BRAN “Broadband Radio Access Networks” https://www.etsi.org/committee/1389-bran Published standards, technical specifications and reports related to Home Network - EN 300 328 V2.2.2 (2019-07): Wideband transmission systems; Data transmission equipment operating in the 2,4 GHz band; Harmonised Standard for access to radio spectrum. This document covers Wideband Data Transmission equipment. Examples of Wideband Data Transmission equipment are equipment such as IEEE 802.11™ RLANs, Bluetooth® wireless technologies, Zigbee™, etc. - EN 301 893 V2.1.1 (2017-05): 5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU. 5 GHz wireless access systems (WAS) including RLAN equipment are used in wireless local area networks which provide high speed data communications in between devices connected to the wireless infrastructure. This document also addresses ad-hoc networking where devices communicate directly with each other, without the use of a wireless infrastructure. - TR 103 631 V1.1.1 (2019-03): Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) in the band 6 725 MHz to 7 125 MHz. License exempt Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) represent the primary broadband wireless access technologies used for wireless internet access. This document provides information on the intended applications, the technical parameters, mitigation techniques, the relation to the existing spectrum regulation and additional new radio spectrum requirements for technology neutral wireless access systems including radio local area networks (WAS/RLANs) capable of operating in the 6 725 MHz to 7 125 MHz range. - EN 302 567 V2.2.1 (2021-07): : Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard for access to radio spectrum. This document specifies technical characteristics and methods of measurements for radio equipment with integral antennas operating indoor or outdoor at data rates of multiple-gigabit per second in the 60 GHz frequency range - ETSI TR 103 583 V1.1.1 (2019-08): System Reference document (SRdoc); Technical characteristics of Multiple Gigabit Wireless Systems (MGWS) in radio spectrum between 57 GHz and 71 GHz Broadband Radio Access Networks (BRAN); HIPERLAN Type 2 HIPERLAN/2 systems are intended to be operated as private or public systems making use of the 5 GHz frequency range - TR 101 683 V1.1.1 (2000-02): Broadband Radio Access Networks (BRAN); HIPERLAN Type 2; System Overview - TS 101 475 V1.3.1 (2001-12): Broadband Radio Access Networks (BRAN); HIPERLAN Type 2; Physical (PHY) layer - TS 101 761-1 V1.3.1 (2001-12): Broadband Radio Access Networks (BRAN); HIPERLAN Type 2; Data Link Control (DLC) Layer; Part 1: Basic Data Transport Functions - TS 101 493-1 V1.1.1 (2000-04): Broadband Radio Access Networks (BRAN); HIPERLAN Type 2; Packet based Convergence Layer; Part 1: Common Part More information about ETSI BRAN can be found at following URL https://portal.etsi.org/TB-SiteMap/bran/bran-tor Work Programme of ETSI BRAN can be found at following URL https://portal.etsi.org/tb.aspx?tbid=287&SubTB=287#/ |  |
| ETSI TC DECT continuation of ETSI TC DECT continuation of ETSI TC DECT | ETSI TC DECT “Digital Enhanced Cordless Telecommunications” https://www.etsi.org/committee/1394-dect Published standards, technical specifications and reports related to Home Network - EN 300 175-1 V2.8.1 (2019-12): Digital Enhanced Cordless Telecommunications (DECT); Common Interface (CI); Part 1: Overview - EN 300 175-2 V2.8.1 (2019-12): Digital Enhanced Cordless Telecommunications (DECT); Common Interface (CI); Part 2: Physical Layer (PHL) - EN 300 175-3 V2.8.1 (2019-12): Digital Enhanced Cordless Telecommunications (DECT); Common Interface (CI); Part 3: Medium Access Control (MAC) layer - EN 300 175-4 V2.8.1 (2019-12): Digital Enhanced Cordless Telecommunications (DECT); Common Interface (CI); Part 4: Data Link Control (DLC) layer - EN 300 175-5 V2.8.1 (2019-12): Digital Enhanced Cordless Telecommunications (DECT); Common Interface (CI); Part 5: Network (NWK) layer - EN 300 175-6 V2.8.1 (2019-12): Digital Enhanced Cordless Telecommunications (DECT); Common Interface (CI); Part 6: Identities and addressing - EN 300 175-7 V2.8.1 (2019-12): Digital Enhanced Cordless Telecommunications (DECT); Common Interface (CI); Part 7: Security features - EN 300 175-8 V2.8.1 (2019-12): Digital Enhanced Cordless Telecommunications (DECT); Common Interface (CI); Part 8: Speech and audio coding and transmission - EN 300 444 V2.5.1 (2017-10): Digital Enhanced Cordless Telecommunications (DECT); Generic Access Profile (GAP) - TS 102 939-1 V1.3.1 (2017-10): Digital Enhanced Cordless Telecommunications (DECT); Ultra Low Energy (ULE); Machine to Machine Communications; Part 1: Home Automation Network (phase 1) - TS 102 939-2 V1.3.1 (2019-01): Digital Enhanced Cordless Telecommunications (DECT); Ultra Low Energy (ULE); Machine to Machine Communications; Part 2: Home Automation Network (phase 2) - TR 103 445 V1.1.1 (2017-07): Digital Enhanced Cordless Telecommunications (DECT); DECT security technical review; Security review and assessment 2017 - TR 103 422 V1.1.1 (2017-06): Digital Enhanced Cordless Telecommunications (DECT); DECT evolution technical study; Requirements and technical analysis for the further evolution of DECT and DECT ULE DECT-2020 New Radio (NR) DECT-2020 NR is a Radio Interface Technology (RIT) designed to provide a slim but powerful technology foundation for wireless applications deployed in various use cases and markets. This radio technology includes, but is not limited to Cordless Telephony, Audio Streaming Applications, Professional Audio Applications, consumer and industrial applications of Internet of Things (IoT) such as industry and building automation and monitoring, and in general solutions for local area deployments for Ultra-Reliable Low Latency Communication (URLLC) and massive Machine Type Communication (mMTC) as envisioned by ITU-R for IMT2020. - ETSI TS 103 636-1 V1.2.1 (2021-04): DECT-2020 New Radio (NR); Part 1: Overview; Release 1 - ETSI TS 103 636-2 V1.2.1 (2021-04): DECT-2020 New Radio (NR); Part 2: Radio reception and transmission requirements; Release 1 - ETSI TS 103 636-3 V1.2.1 (2021-04): DECT-2020 New Radio (NR); Part 3: Physical layer; Release 1 - ETSI TS 103 636-4 V1.2.1 (2021-04): DECT-2020 New Radio (NR); Part 4: MAC layer; Release 1 Roadmap of DECT can be seen in Technical Report: - TR 103 513 V1.1.1 (2019-11): Digital Enhanced Cordless Telecommunications (DECT); DECT Technology Roadmap More information about ETSI DECT can be found at following URL https://portal.etsi.org/TB-SiteMap/dect/dect-tor Work Programme of ETSI DECT can be found at following URL https://portal.etsi.org/tb.aspx?tbid=19&SubTB=19,894#/ |  |
| ETSI TC EE | ETSI TC EE “Environmental Engineering” https://www.etsi.org/committee/1395-ee Published standards, technical specifications and reports related to Home Network - EN 301 575 V1.1.1 (2012-05): Environmental Engineering (EE); Measurement methods for energy consumption of Customer Premises Equipment (CPE). This document defines the energy consumption measurement methods for Broadband CPE telecommunication equipment. More information about ETSI EE can be found at following URL https://portal.etsi.org/TB-SiteMap/ee/ee-tor Work Programme of ETSI EE can be found at following URL https://portal.etsi.org/tb.aspx?tbid=28&SubTB=28,29,30,635,853#/ |  |
| ETSI ISG F5G | ETSI ISG - Fifth Generation Fixed Network (F5G) https://www.etsi.org/committee/1696-f5g Published standards, technical specifications and reports related to Home Network - GR F5G 001 V1.1.1 (2020-12): Fifth Generation Fixed Network (F5G); F5G Generation Definition Release #1 - GR F5G 002 V1.1.1 (2021-02): Fifth Generation Fixed Network (F5G); F5G Use Cases Release #1 - GS F5G 009 V1.1.1 (2021-06): Fifth Generation Fixed Network (F5G); Proof of Concept Framework - GS F5G 003 V1.1.1 (2021-09): Fifth Generation Fixed Network (F5G); F5G Technology Landscape More information about ETSI ISG F5G can be found at following URL https://portal.etsi.org/Portals/0/TBpages/F5G/ISG_F5G_ToR_D-G_APPROVED_20191210.pdf Work Programme of ETSI F5G can be found at following URL https://portal.etsi.org/tb.aspx?tbid=885&SubTB=885#/ ITU-T Q18/15 Project FTTR (Fibre to the Room): for more information see workshop programme and ETSI ISG F5G presentation at ​​​​​​https://www.itu.int/en/ITU-T/Workshops-and-Seminars/2021/0614/Pages/default.aspx |  |
| HomeGrid Forum | HomeGrid Forum http://www.homegridforum.org/ HomeGrid Forum (HGF) is an industry alliance formed to support the development and deployment of a unified coaxial, phoneline, powerline, and plastic optical fiber home networking technology called G.hn (Gigabit Home Networking). HomeGrid Forum G.hn Certification Testing More information about G.hn Certification can be found at following URL https://homegridforum.org/certification-overview/ SG15 TD 650 WP1 December 2021 In its liaison TD 650 WP1 from December 2021, HomeGrid Forum informs about the recent developments on the GiGAWire VB™ technology for broadband access applications, based on ITU-T G.hn family of Recommendations and provides details on the G.hn Access standardization profiles addressing broadband access network topologies, as well as the acceleration of the deployments with the GiGAWire VB™ open-source code program. Additional information about GiGaWire can be found at following URL https://homegridforum.org/giga-wire-access/ |  |
| CENELEC continuation of CENELEC | TC 86BXA | Fibre optic interconnect, passive and connectorised components https://standards.cencenelec.eu/dyn/www/f?p=305:7:0:25:::FSP_ORG_ID,FSP_LANG_ID:1258371 Business Plan of TC 86BXA: see https://ftp.cencenelec.eu/CENELEC/BP/BP_TC_86BXA.pdf Published documents related to Home Network installation and maintenance - CLC/TR 50682:2018: Consideration on the use of OTDRs to measure return loss of single-mode optical fibre connection |
|  | TC 205 | Home and Building Electronic Systems (HBES) https://standards.cencenelec.eu/dyn/www/f?p=305:7:0:25:::FSP_ORG_ID,FSP_LANG_ID:1258281 Business Plan of TC 205: see https://ftp.cencenelec.eu/CENELEC/BP/BP_TC_205.pdf Main published documents according to TC 205 Business Plan - EN 50090-1:2011: Home and Building Electronic Systems (HBES) - Part 1: Standardization structure - EN 50491-6-1:2014: General requirements for Home and Building Electronic Systems (HBES) and Building Automation and Control Systems (BACS) - Part 6-1: HBES installations - Installation and planning - EN 63044-1:2017: Home and Building Electronic Systems (HBES) and Building Automation and Control Systems (BACS) - Part 1: General requirements - EN 63044-1:2017/A1:2021: Home and Building Electronic Systems (HBES) and Building Automation and Control Systems (BACS) - Part 1: General requirements - EN IEC 63044-6:2021: Home and building electronic systems (HBES) and building automation and control systems (BACS) - Part 6: Requirements for planning and installation |
|  | TC 215 | Electrotechnical aspects of telecommunication equipment https://standards.cencenelec.eu/dyn/www/f?p=305:7:0:25:::FSP_ORG_ID,FSP_LANG_ID:1258297 Business Plan of TC 215: see https://ftp.cencenelec.eu/CENELEC/BP/BP_TC_215.pdf Published documents related to Home Network cabling - EN 50173-1:2018: Information technology - Generic cabling systems - Part 1: General requirements - EN 50173-4:2018: Information technology - Generic cabling systems - Part 4: Homes |
| IEC continuation of IEC | CISPR | International special committee on radio interference - EMC requirements https://www.iec.ch/emc/iec_emc/iec_emc_players_cispr.htm |
|  | TC 46/ SC 46C | Wires and symmetric cables https://www.iec.ch/dyn/www/f?p=103:7:::::FSP_ORG_ID:1366 Published documents related to Home Network cabling - IEC 62807-1:2017: Hybrid telecommunication cables - Part 1: Generic specification |
|  | TC 76 | Optical radiation safety and laser equipment https://www.iec.ch/dyn/www/f?p=103:7:::::FSP_ORG_ID:1264 Published documents related to Home Network safety - IEC 60825-2:2021: Safety of laser products - Part 2: Safety of optical fibre communication systems (OFCSs) - IEC 60825-2:2021/COR1:2021: Corrigendum 1 - Safety of laser products - Part 2: Safety of optical fibre communication systems (OFCSs) |
|  | TC 86/ SC 86A | Fibres and cables https://www.iec.ch/dyn/www/f?p=103:7:::::FSP_ORG_ID:1398 Published documents related to Home Network cabling - IEC 60793-2-40:2021:Optical fibres - Part 2-40: Product specifications - Sectional specification for category A4 multimode fibres - IEC 60793-2-50:2018: Optical fibres - Part 2-50: Product specifications - Sectional specification for class B single-mode fibres - IEC 60794-2:2017: Optical fibre cables - Part 2: Indoor cables - Sectional specification - IEC 60794-6:2020: Optical Fibre Cables Part 6: Indoor-Outdoor cables - Sectional specification for Indoor-Outdoor cables - IEC TR 62000:2021: Guidelines for combining different single-mode fibre sub-categories - IEC TR 62362:2020: Selection of optical fibre cable specifications relative to mechanical, ingress, climatic or electromagnetic characteristics - Guidance |
| IEEE 802.3 continuation of IEEE 802.3 IEEE 802.11 continuation of IEEE 802.11 continuation of IEEE 802.11 IEEE 802.15 IEEE PLCSC IEEE 1901 | IEEE 802.3 Ethernet Working Group http://www.ieee802.org/3/ SG15 TD 720 WP1 December 2021 IEEE 802.3 Working Group communicates following update: IEEE Std 802.3-2018, Standard for Ethernet, is the current revision. https://standards.ieee.org/standard/802_3-2018.html This revision has fourteen approved amendments: IEEE Std 802.3cb-2018, IEEE Std 802.3bt-2018, IEEE Std 802.3cd-2018, IEEE Std 802.3cn-2019, IEEE Std 802.3cg-2019, IEEE Std 802.3cq-2020, IEEE Std 802.3cm-2020, IEEE Std 802.3ch-2020, IEEE Std 802.3ca-2020, IEEE Std 802.3cr-2021, IEEE Std 802.3cu-2021, IEEE Std 802.3cv-2021, IEEE Std 802.3ct-2021, and IEEE Std 802.3cp-2021. All of these amendments are being merged into IEEE Std 802.3 in a currently active amendment project, IEEE P802.3 (also referred to as IEEE 802.3dc). The following are example HNT applicable technologies in IEEE Std 802.3-2018 (including its amendments): - The 10BASE-T, 100BASE-TX and 1000BASE-T specifications for operation over various grades of twisted pair cabling have long been used as a home networking technology, and they continue to be applicable. - Home gateways typically include both IEEE Std 802.11 specified capabilities and 10/100/1000 Mb/s Ethernet ports. - 2.5GBASE-T, 5GBASE-T and 10GBASE-T provide a migration path for higher bandwidth home networks. - 1000BASE-RHA is a plastic optical fiber port type targeted for home networks. - Fiber optic Ethernet port types would be applicable to HNT especially in cases where a non-conductive medium is required. It is appropriate to note that BASE-T port types are not specified for outdoor cable installations. - For access to the home, the approved standard includes various speeds of operation for Ethernet Passive Optical Networks. - The standard also includes DTE Power via the MDI (also called Power over Ethernet) capabilities applicable to HNT (e.g., to provide power to security equipment). These specifications include multiple options for BASE-T cabling with options for amount of power provided to the Powered Device. Other optional Ethernet capabilities have relevance to HNT including: - Time Sensitive Networking related functions appropriate to support applications running over HNT, and Energy-Efficient Ethernet specifications for many port types to reduce energy consumption. - IEEE Std 802.3.1-2013 specifies SNMP management modules for various Ethernet port types and capabilities. IEEE Std 802.3.2-2018 YANG Data Model(s) specifies YANG data models for selected Ethernet port types. Much of the current work within the IEEE 802.3 Working Group (current activities are listed on the 802.3 home page http://ieee802.org/3) may not be applicable to HNT, but a few recent and current activities are highlighted below as possibly related. - The IEEE P802.3cx Improved PTP Timestamping Accuracy Task Force draft is currently in WG ballot. This amendment will improve the precision of delay and jitter measurements, for data carried over Ethernet, capabilities that are leveraged by some time sensitive HNT applications. - Single pair Ethernet port types recently added to IEEE Std 802.3 are also being enhanced by IEEE P802.3da. 10 Mb/s Single Pair Multidrop Segments Enhancement, with additional capabilities under consideration in the Greater than 10 Mb/s Long Reach Single Pair Ethernet Study Group. These single pair port types though not targeted for HNT use may nevertheless find HNT use. |  |
|  | IEEE 802.11 Working Group for Wireless Local Area Networks http://www.ieee802.org/11/ IEEE Std 802.11-2020 - IEEE Standard for Information technology--Telecommunications and information exchange between systems Local and metropolitan area networks--Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications. More information about 802.11-202 can be found at following URL https://standards.ieee.org/standard/802_11-2020.html Wi-Fi 4, 5, 6 and 6E - IEEE Std 802.11-2020 - Clause 19 (Supersedes IEEE 802.11n-2009 Amendment 5 of IEEE 802.11-2007): IEEE Standard for Information technology--Telecommunications and information exchange between systems Local and metropolitan area networks--Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications - Clause 19: High-throughput (HT) PHY specification Branded as Wi-Fi 4 by the Wi-Fi Alliance. - IEEE Std 802.11-2020 - Clause 21 (Supersedes IEEE 802.11ac-2013 Amendment 4 of IEEE 802.11-2012): IEEE Standard for Information technology--Telecommunications and information exchange between systems Local and metropolitan area networks--Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications - Clause 21 - Very high throughput (VHT) PHY specification Branded as Wi-Fi 5 by the Wi-Fi Alliance. - IEEE Std 802.11ax-2021 - IEEE Standard for Information Technology--Telecommunications and Information Exchange Between Systems Local and Metropolitan Area Networks--Specific Requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications Amendment 1: Enhancements for High Efficiency WLAN Branded as Wi-Fi 6 and Wi-Fi 6E by the Wi-Fi Alliance. WiGig (mmWave Wi-Fi) - IEEE Std 802.11-2020 - Clause 20 (Supersedes IEEE 802.11ad-2012 Amendment 3 of IEEE 802.11-2012): IEEE Standard for Information technology--Telecommunications and information exchange between systems Local and metropolitan area networks--Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications - Clause 20 - Directional multi-gigabit (DMG) PHY specification - IEEE Std 802.11-2020 – Clause 25 (Supersedes IEEE Std 802.11aj-2018 Amendment 3 of IEEE 802.11-2016: Enhancements for Very High Throughput to Support Chinese Millimeter Wave Frequency Bands (60 GHz and 45 GHz): IEEE Standard for Information Technology--Telecommunications and information exchange between systems Local and metropolitan area networks--Specific requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications - Clause 25 - China millimeter-wave multi-gigabit (CMMG) PHY specification - IEEE Std 802.11ay-2021 - IEEE Standard for Information Technology--Telecommunications and Information Exchange between Systems Local and Metropolitan Area Networks--Specific Requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications Amendment 2: Enhanced Throughput for Operation in License-exempt Bands above 45 GHz Sub 1 GHz (Low-power Wi-Fi) - IEEE Std 802.11-2020 – Clause 23 (Supersedes IEEE Std 802.11ah-2016 Amendment 2 of 802.11-2016: Sub 1 GHz License Exempt Operation): IEEE Standard for Information technology--Telecommunications and information exchange between systems - Local and metropolitan area networks--Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications – Clause 23 - Sub 1 GHz (S1G) PHY specification (Note: 80.2.11ah-2016 is branded as “Wi-Fi HaLow™” by the Wi-Fi Alliance) New IEEE 802.11 standardization projects applicable to Home Network IEEE 802.11 has approved the creation of a new task group to work on a major amendment, IEEE P802.11be, for next generation wireless LAN: IEEE P802.11be to Enable Extremely High Throughput (EHT) and Low Latency for Wi-Fi. Branded as future Wi-Fi 7 by the Wi-Fi Alliance For more information about IEEE P802.11be can be found at following URL https://beyondstandards.ieee.org/ieee-p802-11be-to-enable-extremely-high-throughput-eht-and-low-latency-for-wi-fi/ and at: https://www.ieee802.org/11/Reports/tgbe_update.htm IEEE P802.11bf is a new Task Group about WLAN sensing within the IEEE 802.11 working group More information about IEEE P802.11bf can be found at following URL https://www.ieee802.org/11/Reports/tgbf_update.htm Additional information about Wi-Fi can be found on the Wi-Fi Alliance® website at following URL https://www.wi-fi.org/ Information about the Wi-Fi generation names and user interface visuals can be found at following URL Generational Wi-Fi User Guide Information about Wi-Fi Certification can be found at following URL https://www.wi-fi.org/certification |  |
|  | IEEE 802.15 Working Group for Wireless Specialty Networks (WSN) http://www.ieee802.org/15/ Short-Range Optical Wireless Communications https://standards.ieee.org/standard/802_15_7-2018.html - IEEE Std 802.15.7-2018 - IEEE Standard for Local and metropolitan area networks--Part 15.7: Short-Range Optical Wireless Communications Project of Task Group 13 (TG13) Multi-Gigabit/s Optical Wireless Communications https://ieeexplore.ieee.org/document/9540818 - P802.15.13/D5.0, Sept 2021 - IEEE Draft Standard for Multi-Gigabit per Second Optical Wireless Communications (OWC), with Ranges up to 200 meters, for both stationary and mobile devices |  |
|  | IEEE Power Line Communications Standards Committee https://sagroups.ieee.org/plcsc/ IEEE 1901 Working Group on Power Line Communications (COM/PLC) https://sagroups.ieee.org/1901/ The scope of the IEEE 1901 Working Group is to maintain and advance the IEEE 1901 standard “IEEE Standard for Broadband over Power Line Networks: Medium Access Control and Physical Layer Specifications” originally approved in 2010. Recent activities included the development of IEEE 1901a-2019 “Amendment 1: Enhancement for Internet of Things Applications” and the revision of IEEE 1901-2010. This has resulted in the publication of IEEE Std 1901-2020 - IEEE Standard for Broadband over Power Line Networks: Medium Access Control and Physical Layer Specifications. Most recent activities of the Working Group include the work on the P1901b project “Standard for Broadband over Power Line Networks: Medium Access Control and Physical Layer Specifications - Amendment 2: Enhancements for Authentication and Authorization”. The Draft Standard has been approved by SA ballot on 2 October 2021. See at https://ieeexplore.ieee.org/document/9530471 IEEE 1901b - IEEE Draft Standard for Broadband over Power Line Networks: Medium Access Control and Physical Layer Specifications Amendment 2: Enhancements for Authentication and Authorization (published on 2 September 2021). For more information about IEEE 1901-2020, see at https://standards.ieee.org/standard/1901-2020.html |  |
| IEEE PLCSC IEEE 1901.1 | IEEE Power Line Communications Standards Committee https://sagroups.ieee.org/plcsc/ IEEE Standard for Medium Frequency (less than 12 MHz) Power Line Communications for Smart Grid Applications https://standards.ieee.org/standard/1901_1-2018.html - IEEE Std 1901.1-2018 - IEEE Standard for Medium Frequency (less than 12 MHz) Power Line Communications for Smart Grid Applications |  |
| IEEE PLCSC IEEE 1901.2 | IEEE Power Line Communications Standards Committee https://sagroups.ieee.org/plcsc/ IEEE Standard for Low-Frequency (less than 500 kHz) Narrowband Power Line Communications for Smart Grid Applications https://standards.ieee.org/standard/1901_2-2013.html - IEEE Std 1901.2-2013 - IEEE Standard for Low-Frequency (less than 500 kHz) Narrowband Power Line Communications for Smart Grid Applications and amendment - IEEE Std 1901.2a-2015 - IEEE Standard for Low-Frequency (less than 500 kHz) Narrowband Power Line Communications for Smart Grid Applications - Amendment 1 |  |
| IEEE PLCSC IEEE 1905.1 | IEEE Power Line Communications Standards Committee https://sagroups.ieee.org/plcsc/ IEEE Standard for a Convergent Digital Home Network for Heterogeneous Technologies https://standards.ieee.org/standard/1905_1-2013.html - IEEE Std 1905.1-2013 - IEEE Standard for a Convergent Digital Home Network for Heterogeneous Technologies - IEEE Std 1905.1a-2014 - IEEE Standard for a Convergent Digital Home Network for Heterogeneous Technologies Amendment 1: Support of New MAC/PHYs and Enhancements |  |
| ISO/IEC continuation of ISO/IEC continuation of ISO/IEC continuation of ISO/IEC continuation of ISO/IEC continuation of ISO/IEC continuation of ISO/IEC continuation of ISO/IEC | JTC 1/SC 6 | JTC 1/SC 6 “Telecommunications and information exchange between systems” https://www.iso.org/committee/45072.html Published documents related to Home Network - ISO/IEC 12139-1:2009 - Information technology — Telecommunications and information exchange between systems — Powerline communication (PLC) — High speed PLC medium access control (MAC) and physical layer (PHY) — Part 1: General requirements - ISO/IEC 12139-1:2009/COR 1:2010 - Information technology — Telecommunications and information exchange between systems — Powerline communication (PLC) — High speed PLC medium access control (MAC) and physical layer (PHY) — Part 1: General requirements — Technical Corrigendum 1 ISO/IEC 12139-1:2009 is a physical and medium access control layer specification with respect to the connectivity for In-home and Access network high speed powerline communication stations. |
|  | JTC 1/SC 25 continuation of JTC 1/SC 25 continuation of JTC 1/SC 25 continuation of JTC 1/SC 25 continuation of JTC 1/SC 25 continuation of JTC 1/SC 25 continuation of JTC 1/SC 25 | JTC 1/SC 25 “Interconnection of Information Technology Equipment” https://www.iso.org/committee/45270.html All information related to ISO/IEC JTC 1/SC 25 is available on the IEC web site Scope of ISO/IEC JTC 1/SC 25 The scope of SC 25 is to provide technologies for interconnection of information technology equipment in Customer premises – see also ISO/IEC JTC 1/SC 25 Scope ISO/IEC JTC 1/SC 25 Subcommittee(s) and/or Working Group(s) responsible for home networks: - SC 25/WG 1 “Home Electronic System (HES)” develops IoT (Internet of Things) standards for the interconnection of electrical and electronic equipment and products for homes and small buildings. - SC 25/WG 3 “Customer Premises Cabling” develops standards for customer premises cabling systems including test procedures, planning and installation guide. The list of standards published by ISO/IEC JTC 1/SC 25 is available using the URL below ISO/IEC JTC 1/SC 25 Publications The work programme of ISO/IEC JTC 1/SC 25 is available using the URL below ISO-IEC JTC 1/SC 25 Work Programme ISO/IEC JTC 1/SC 25/WG 1 “Home Electronic System (HES)” SG15 TD 546 WP1 April 2021 In its liaison ISO/IEC JTC 1/SC 25 notes that ITU-T SG15 has projects in the field of home networks (“domotics”) such as: - Technical paper on architecture, function, and service of home network - Technologies for in-premises networking and related access applications ISO/IEC JTC 1/SC25 recommends that ITU-T SG15 references standards developed by SC25 that are relevant to SG15 projects. The scope of SC25 covers areas important for home networks and applications, as stated in the following except from the scope: - Standards for home and building electronic systems in residential and commercial environments to support interworking devices (IoT-related) and applications such as energy management, environmental control, lighting, and security. In its liaison ISO/IEC JTC 1/SC 25 attaches a summary “Home Electronic System (HES) Overview” dated September 2020 prepared by Dr. Kenneth Wacks - Convener of ISO/IEC JTC 1/SC 25/WG 1 – see attached below. This summary provides an overview of the more than 50 standards and technical reports published and under development originated in SC25/WG1. This document served as reference for the update of the “List of HES standards and technical reports” below as well as Part 6 of the “Web-Based Home Network Transport (HNT) Standards Overview” - see Section 3 - of the present document. Published HES standards and technical reports HES standards specify the architecture and protocols of the Home Electronic System. These are found in the ISO/IEC 14543 series. Presently three sets of protocols are specified, namely the ISO/IEC 14543-3 series, the ISO/IEC 14543-4 series and the ISO/IEC 14543-5 series. They all conform to a common architecture specified in ISO/IEC 14543-2-1: see below the list of standards and technical reports for HES Architecture. - TS 15044:2000 - Information technology - Terminology for the Home Electronic System (HES) - 14543-2-1:2006 - HES Architecture Part 2-1: Introduction and device modularity - 14543-3-1: 2006 - HES Architecture Part 3-1: Communication layers - Application layer for network based control of HES Class 1 - 14543-3-2:2006 - HES Architecture Part 3-2: Communication layers - Transport, network and general parts of data link layer for network based control of HES Class 1 - 14543-3-3:2007 - HES Architecture Part 3-3: User process for network based control of HES Class 1 - 14543-3-4:2007 - HES Architecture Part 3-4: System management - Management procedures for network based control of HES Class 1 - 14543-3-5:2007 - HES Architecture Part 3-5: Media and media dependent layers - Powerline for network based control of HES Class 1 - 14543-3-6:2007 - HES Architecture Part 3-6: Media and media dependent layers - Twisted pair for network based control of HES Class 1 - 14543-3-7:2007 - HES Architecture Part 3-7: Media and media dependent layers - Radio frequency for network based control of HES Class 1 - TR 14543-4:2002 - HES Architecture Part 4: Home and building automation in a mixed-use building - 14543-4-1:2008 - HES Architecture Part 4-1: Communication layers - Application layer for network enhanced control devices of HES Class 1 - 14543-4-2:2008 - HES Architecture Part 4-2: Communication layers - Transport, network and general parts of data link layer for network enhanced control devices of HES Class 1 - 14543-4-3:2015 - HES Architecture Part 4-3: Application layer interface to lower communications layers for network enhanced control devices of HES Class 1 - 14543-4-301:2020 - HES Architecture Part 4-301: Application protocols for home air conditioners and controllers - 14543-5-1:2010 - HES Architecture Part 5-1: Intelligent grouping and resource sharing for Class 2 and Class 3 - Core protocol - 14543-5-3:2012 - HES Architecture Part 5-3: Intelligent grouping and resource sharing for HES Class 2 and Class 3 - Basic application - 14543-5-4:2010 - HES Architecture Part 5-4: Intelligent grouping and resource sharing for HES Class 2 and Class 3 - Device validation - 14543-5-5:2012 - HES Architecture Part 5-5: Intelligent grouping and resource sharing for HES Class 2 and Class 3 - Device type - 14543-5-6:2012 - HES Architecture Part 5-6: Intelligent grouping and resource sharing for HES Class 2 and Class 3 - Service type - 14543-5-7:2015 - HES Architecture - Part 5-7: Intelligent grouping and resource sharing for HES Class 2 and Class 3 - Remote access system architecture - 14543-5-8:2017 - HES Architecture - Part 5-8: Intelligent grouping and resource sharing for HES Class 2 and Class 3 - Remote access core protocol - 14543-5-9:2017 - HES Architecture - Part 5-9: Intelligent grouping and resource sharing for HES Class 2 and Class 3 - Remote access service platform - 14543-5-11:2018 - HES Architecture - Part 5-11: Intelligent grouping and resource sharing for HES Class 2 and Class 3 - Remote user interface - 14543-5-12:2019 - HES Architecture - Part 5-12: Intelligent grouping and resource sharing for HES Class 2 and Class 3 - Remote access test and verification - 14543-5-21:2012 - HES Architecture - Part 5-21: Intelligent grouping and resource sharing for HES Class 2 and Class 3 - Application profile - AV profile -14543-5-22:2010 - HES Architecture - Part 5-22: Intelligent grouping and resource sharing for HES Class 2 and Class 3 - Application profile - File profile - 14543-5-101:2019 - HES Architecture - Part 5-101: Intelligent grouping and resource sharing for HES Class 2 and Class 3 - Remote media access profile - 14543-5-102:2020 - HES Architecture - Part 5-102: Intelligent grouping and resource sharing for HES Class 2 and Class 3 - Remote universal management profile Since these three protocols cannot directly communicate with each other, SC 25/WG 1 has also specified a series of interoperability and gateway standards, the ISO/IEC 18012 and ISO/IEC 15045 series: see below the list of standards and technical reports for product interoperability, gateway and interfaces. - 18012-1:2004 - Guidelines for product interoperability - Part 1: Introduction - 18012-2:2012 - Guidelines for product interoperability - Part 2: Taxonomy and application interoperability model - 15045-1:2004 - HES Gateway - Part 1: A residential gateway model for HES - 15045-2:2012 - HES Gateway - Part 2: Modularity and protocol - 10192-1:2002 - HES Interfaces - Part 1: Universal Interface (UI) Class 1 - TR 10192-2:2000 - HES Interfaces - Part 2: Simple Interface Type 1 - 10192-3:2017 - HES Interfaces - Part 3: Modular communications interface for energy management For the protection of the home and related communications, a set of standards have been specified. ISO/IEC 24767-1 specifies the security requirements. For protection of the communications between devices in the home that do not have the capability to support IP, a simpler security protocol has been specified in ISO/IEC 24767-2: see below the list of standards for security. - 24767-1:2008 - Home network security - Part 1: Security requirements - 24767-2:2009 - Home network security - Part 2: Internal security services - Secure communication protocol for middleware (SCPM) In order to support energy harvesting devices, i.e. devices that do not depend on batteries or mains power, very energy-efficient wireless communication protocols have been developed (ISO/IEC 14543-3-10 and ISO/IEC 14543-3-11): see below the list of standards for wireless communication. - 14543-3-10:2020 - HES Architecture Part 3-10: Amplitude modulated wireless short-packet (AMWSP) protocol optimized for energy harvesting - Architecture and lower layer protocols - 14543-3-11:2016 - HES Architecture Part 3-11: Frequency modulated wireless short-packet (FMWSP) protocol optimised for energy harvesting - Architecture and lower layer protocols The ISO/IEC 29145 series specifies an efficient wireless mesh network - 29145-1:2014 - Wireless beacon-enabled energy efficient mesh network (WiBEEM) for wireless home network services - Part 1: PHY layer - 29145-2:2014 - Wireless beacon-enabled energy efficient mesh network (WiBEEM) for wireless home network services - Part 2: MAC layer - 29145-3:2014 - Wireless beacon-enabled energy efficient mesh network (WiBEEM) for wireless home network services - Part 3: NWK layer In addition to these communication protocol specifications, SC 25/WG 1 published a set of standards to manage energy usage and generation in the home, GridWise, lighting systems and residential security systems. These are found in the ISO/IEC 15067-2, ISO/IEC 15067-3 and ISO/IEC 15067-4 series: see below the list of standards and technical reports for energy management, lighting and residential security systems. - TR 15067-2:1997 - HES Application model - Part 2: Lighting model for HES - 15067-3:2012 - HES Application model - Part 3: Model of a demand-response energy management system for HES - TR 15067-3-2:2016 - HES Application model - Part 3-2: GridWise - Interoperability context-setting framework - 15067-3-3:2019 - HES Application model - Part 3-3: Model of a system of interacting energy management agents (EMAs) for demand-response energy management - TR 15067-3-7:2020 - HES Application model - GridWise transactive energy systems research, development and deployment roadmap - TR 15067-3-8:2020 - HES Application model - Part 3-8: GridWise transactive energy framework - TR 15067-4:2001 - HES Application model - Part 4: Security system for HES SC 25/WG 1 published also a set of standards for the home network resource management - these are found in the ISO/IEC 30100 series – and a standard ISO/IEC 14762 for the functional safety requirements: see below the list for standards for home network resource management and safety - 30100-1:2016 - Home network resource management - Part 1: Requirements - 30100-2:2016 - Home network resource management - Part 2: Architecture - 30100-3:2016 - Home network resource management - Part 3: Management application - 14762:2009 - Functional safety requirements for home and building electronic systems (HBES) See also the ISO/IEC Technical Reports on Intelligent homes: - TR 29107-1:2010 - Information technology - Intelligent homes - Taxonomy of specifications — Part 1: The scheme - TR 29108:2013 - Information technology - Terminology for intelligent homes ISO/IEC JTC 1/SC 25/WG 3 “Customer Premises Cabling” The cabling standards ISO/IEC 11801 series specify a cabling infrastructure supporting the transport of all kinds of information on customer premises with help of primarily balanced cable and optical fibre cable, and in some cases also of correctional cables. Up to now standards have been published for offices, homes, industrial premises and data centres: see below the list of major standards related to home cabling. - 11801-1:2017 - Generic cabling for customer premises – Part 1: General requirements - 11801-1- Corrigendum 1:2018 - Generic cabling for customer premises — Part 1: General requirements — Technical Corrigendum 1 - 11801-4:2017 - Generic cabling for customer premises – Part 4: Single-tenant homes - 11801-4 – Corrigendum 1:2018 - Generic cabling for customer premises — Part 4: Single-tenant homes — Technical Corrigendum 1 - 14763-2:2019 - Implementation and operation of customer premises cabling - Part 2: Planning and installation - TR 14763-2-1:2011 - Implementation and operation of customer premises cabling - Part 2-1: Planning and installation - Identifiers within administration systems - 14763-3:2014+AMD1:2018 CSV - Information technology - Implementation and operation of customer premises cabling - Part 3: Testing of optical fibre cabling - TS 29125:2017 and Amd.1:2020 - Telecommunications cabling requirements for remote powering of terminal equipment |
| MoCA® continuation of MoCA® continuation of MoCA® | MoCA® Multimedia over Coax Alliance http://www.mocalliance.org/ SG15 TD 578 WP1 April 2021 MoCa provided a complete update of the section “MoCA home networking MAC/PHY specifications” as below. The Multimedia over Coax Alliance, MoCA®, supports three versions of its MoCA home networking MAC/PHY specifications (MoCA Home™ 2.0/2.0 Bonded, MoCA Home™ 2.5 and MoCA Home™ 3.0). MoCA 1.0 and 1.1 specifications are sunset and the Alliance no longer certifies products using these protocols. MoCA Home 2.0/2.0 Bonded - Up to 1 Gbps MAC data rate - Deterministic (scheduled) media access with 3.6 ms average latency - Two packet error modes: Very low Packet Error Rate (1e-8) and Nominal Packet Error Rate (1e-6) - Single or dual 100 MHz channels operating in the frequency range of 400…1650MHz - Power states: active, low power, standby and sleep mode - Backward interoperable with MoCA 1.1 MoCA Home 2.5 - Up to 2.5 Gbps MAC data rate - MoCA protected setup (MPS) - Management Proxy - Enhanced Privacy - Network wide Beacon Power - Bridge detection - MoCA Home 2.5 is backward interoperable with MoCA Home 2.0/2.0 Bonded. NOTE: MoCASec™ is a peer-to-peer security feature accessible for MoCA Home 2.0/2.0 Bonded and MoCA Home 2.5 via firmware download. Peer-to-peer security is embedded in MoCA Home 3.0. MoCA Home 3.0 - Up to 10 Gbps MAC data rate with a channel bandwidth of up to 1600 MHz and 5 Gbps with a channel bandwidth of up to 800 MHz - Less channel bandwidth required to achieve 2.5 Gbps of MAC data rate (300 MHz ~ 400 MHz) - Low latency (as low as 1.4 ms) - Provides peer-to-peer security capability - Strong security and privacy features (support for IEEE Std 802.1X, 128-bit AES-CTR) - Up to four 400 MHz channels operating in the frequency range of 200…2075MHz - Support for network-wide reduced power mode - Support for remote password provisioning without user intervention - MoCA Home 3.0 is backward interoperable with MoCA Home 2.0/2.0 Bonded and MoCA Home 2.5. MoCA Access™ 2.5 MAC/PHY Specification is a network access standard capable of 2.5 Gbps data rates over coaxial cable. For more details can be found at following URL http://www.mocalliance.org/access/index.htm. The Alliance emphasizes true and realized data rates and has conducted numerous field tests to verify MAC rates. |  |
| TIA | TIA (Telecommunications Industry Association) http://standards.tiaonline.org/ TIA-1113 - 2008 Edition, May 2008: Medium-Speed (up to 14 Mbps) Power Line Communications (PLC) Modems using Windowed OFDM |  |
| TTC | TTC (The Telecommunication Technology Committee) https://www.ttc.or.jp/e - TTC JJ-300.00 – Version 3.0 – May 25 , 2017: Home-network Topology Identifying Protocol (HIP) |  |
| CSA (ex. Zigbee) | Connectivity Standards Alliance (CSA) https://csa-iot.org/ In May 2021 Zigbee Alliance rebranded as Connectivity Standards Alliance see https://zigbeealliance.org/news_and_articles/connectivity-standards-alliance/ Zigbee is a IEEE 802.15.4 based wireless system. More information about the Zigbee solution and Technical Specification can be found at following URL https://zigbeealliance.org/solution/zigbee/ |  |
| Z-Wave | Z-Wave Alliance https://z-wavealliance.org/ Z-Wave PHY and MAC layers are defined by ITU-T Recommendation G.9959 . Z-Wave specifications can be found at following URL https://z-wavealliance.org/z-wave-specifications/ More information about Z-Wave technology can be found at following URL https://z-wavealliance.org/about_z-wave_technology/ More information on Z-Wave technology for the Internet of Things and connected objects can be found at following URL https://www.z-wave.com/ |  |

# 3. Web-Based Home Network Transport (HNT) Standards Overview

A web-based HNT Standards Overview has been set up in order to make the standards lists in Section 1.1 (Table 1), Section 1.2 (Table 2), Section 1.3 (Table 3) , Section 2 (Table 4) and related applications in Annex 1 (Table 5) more manageable and user-friendly. The web-based overview is organized by Home Network Transport mediums and the various systems and technologies. It also provides examples of Home Network applications as described in Annex 1. The web-based HNT Standards Overview enables an easy identification and download of the publicly available Standards, Recommendations, Technical Specifications and Reports related to each system, technology and associated technical matters. The web-based HNT Standards Overview focuses on the most relevant published and updated documents related to Home Network. The taxonomy used for these web-based HNT Standards Overview is described in the table below. 
The items (i.e. titles of Standards, Recommendations, etc.) are available by topics & sub topics, responsible ITU Working Groups and other SDOs. They are also searchable on number, title and description. The topics, subtopics and items of the web-based HNT Standards Overview appear alphanumerically listed; this is inherent in the data base system.  

The web-based HNT Standards Overview is hosted on the ITU-T SG15 website page “Documentation” and available at following URL

https://www.itu.int/net4/ITU-T/landscape#?topic=0.132&workgroup=1&searchValue=&page=1&sort=Revelance

| Taxonomy of the Web-based Home Network Transport Standards Overview |  |  |  |
| --- | --- | --- | --- |
| Part 1: General on Home Network |  |  |  |
| Home Network architecture and functions | Home Network security |  |  |
| Part 2: G.hn home networking transceivers |  |  |  |
| Part 3: Wireline home networking |  |  |  |
| Over phone line - HomePNA transceivers - Phoneline Enhanced HomePNA transceivers - Phoneline G.hn unified transceivers | Over powerline – Power Line Communications (PLC) - Narrowband PLC transceivers - Broadband PLC transceivers - Powerline G.hn unified transceivers - Mitigation of interference | Over coaxial cable - Multimedia over Coax Alliance (MoCA) - Coax Enhanced HomePNA transceivers - Coax G.hn unified transceivers | Over Ethernet twisted-pair cable - IEEE 802.3 transceivers over Catx cable - Catx G.hn unified transceivers |
| Over plastic optical fibre (POF) - ETSI POF based home networking - IEEE 802.3 transceivers over POF - POF G.hn unified transceivers | Over glass optical fibre |  |  |
| Part 4: Wireless home networking |  |  |  |
| General on WLAN and WPAN Wireless Access | IEEE 802.11 Wireless LAN / Wi-Fi | Sub 1 GHz Wireless LAN | ETSI HiperLAN Type 2 |
| IEEE 802.15 based Wireless PAN - Bluetooth - Zigbee | Optical wireless communications - Visible Light Communication / Li-Fi | Z-Wave |  |
| Part 5: Home Network applications |  |  |  |
| General on Smart Home Network | Cable-based video services and IPTV | Telecommunications services - Digital Enhanced Cordless Telecommunications (DECT) | Internet connectivity - Internet connectivity over wireline networking - Internet connectivity over wireless networking |
| In-home Access Points connectivity - In-home Access Points connectivity over wireline networking - In-home Access Points connectivity over wireless networking | Personal connectivity | IoT / Domotics applications - IoT / Domotics over wireline networking - IoT/ Domotics over wireless networking |  |
| Part 6: Smart Grid applications |  |  |  |
| Part 7: Home and Building Automation Systems |  |  |  |
| Home and Building Electronic Systems (HBES) - HBES general requirements | Home Electronic System (HES) - HES terminology and architecture - HES interoperability, gateways and interfaces - HES security - HES wireless communication – AM and FMWSP, WiBEEM - HES home network resource management - HBES safety requirements | HES Applications - HES energy management, GridWise and lighting - HES residential security systems |  |
| Part 8: Use of Home Network transceivers in industrial applications |  |  |  |
| Part 9: Home Network cabling |  |  |  |
| Part 10: Environment and power supply of Home Network equipment |  |  |  |
| Part 11: Safety and protection of Home Network equipment |  |  |  |
| Part 12: Operation and Maintenance (OAM) of Home Network |  |  |  |
| Management of Home Network devices | Data models for Home Network | Performance testing of Home Network equipment | Conformance testing of Home Network equipment |

Annex 1 - Overview of Home Networks applications

Table 5 – Overview of Home Network applications and related Home Network technologies

| Technology | Standards | Internet connectivity | In-home Access Points connectivity | Personal connectivity | IoT / Domotics applications | Smart Grid / Energy Management |
| --- | --- | --- | --- | --- | --- | --- |
| Wireline home networking |  |  |  |  |  |  |
| HomePNA (Phoneline) | G.9951 G.9952 G.9953 G.9954 | X |  |  |  |  |
| G.hn (Phoneline) | G.9960 G.9961 G.9962 G.9963 G.9964 | X | X |  | X |  |
| Narrowband PLC (Powerline) | G.9901 G.9902 G.9903 G.9904 G.9905 |  |  |  | X | X |
|  | IEEE 1901.2 IEEE 1901.2a |  |  |  |  | X |
| G.hn Broadband PLC (Powerline) | G.9960 G.9961 G.9962 G.9963 G.9964 | X | X |  | X | X |
| Other Broadband PLC (Powerline) | IEEE 1901 Draft IEEE 1901b | X | X |  | X | X |
|  | IEEE 1901.1 |  |  |  |  | X |
| MoCA (Coax) | MoCA Alliance | X | X |  |  |  |
| HomePNA (Coax) | G.9954 | X | X |  |  |  |
| G.hn (Coax) | G.9960 G.9961 G.9962 G.9963 G.9964 | X | X |  | X |  |
| IEEE 802.3 Ethernet (Catx) | 1000BASE-T 2.5GBASE-T 5GBASE-T 10GBASE-T | X | X |  |  |  |
|  | IEEE 802.3cg |  |  |  | X |  |
| G.hn (Catx) G.hn (Catx) | G.9960 G.9961 G.9962 G.9963 G.9964 | X | X |  | X |  |
| ETSI (PoF) |  | X | X |  |  |  |
| IEEE 802.3 (PoF) | 1000BASE-RHA | X | X |  |  |  |
| G.hn (PoF) | G.9960 G.9961 G.9962 G.9963; G.9964 | X | X |  | X |  |
| Over glass optical fibre | Under study |  |  |  |  |  |
| Wireless home networking |  |  |  |  |  |  |
| IEEE 802.11 | Wi-Fi 4, 5, 6, 6E | X | X |  | X |  |
| IEEE 802.11 | WiGig (mmWave Wi-Fi) | X | X |  |  |  |
| Bluetooth | Bluetooth specifications |  |  | X | X |  |
| Zigbee | Zigbee specifications |  |  |  | X | X |
| LiFi | G.9991 IEEE 802.15.7 | X |  | X | X |  |
| Sub-1 GHz and Z-Wave | IEEE 802.11ah G.9959 Z-Wave specifications |  |  |  | X | X |
| DECT ULE |  |  |  |  | X |  |
| DECT-2020 NR |  |  |  | X | X |  |
| ISO/IEC JTC 1/SC 25 “Home Electronic System ” |  |  |  |  |  |  |
| HES |  |  |  |  | X | X |

Annex 2  - List of Contacts

| Body | Contact person |  | Link to the Web-Site | Status of contact Notes Liaison Tracking |
| --- | --- | --- | --- | --- |
| Broadband Forum | Lincoln Lavoie Broadband Forum Technical Committee Chair lylavoie@iol.unh.edu |  | www.broadband-forum.org/ | SG15 TD 334 GEN Jan-Feb 2020 Liaison Rapporteur Frank Van der Putten frank.van_der_putten@nokia.com |
| CENELEC CLC/TC 205, "Home and Building Electronic Systems (HBES)" |  |  | https://standards.cencenelec.eu/dyn/www/f?p=305:7:0:25:::FSP_ORG_ID,FSP_LANG_ID:1258281 |  |
| CENELEC CLC/TC215, "Electrotechnical aspects of telecommunication equipment” |  |  | https://standards.cencenelec.eu/dyn/www/f?p=305:7:0:25:::FSP_ORG_ID,FSP_LANG_ID:1258297 |  |
| IEC TC 86 SC 86A Fibres and cables |  |  | https://www.iec.ch/dyn/www/f?p=103:7:0::::FSP_ORG_ID:1398 | SG15 TD 476 GEN and SG15 TD 477 GEN April 2021 Liaison Rapporteur Peter Pondillo Corning Incorporated pondillopl@corning.com |
| ETSI TC ATTM Access Terminals, Transmission and Multiplexing | Chairman: ATTM Dominique Roche eG4U dominique.roche@eg4u.org ATTM Technical Secretary Pat O’Keeffe eG4U Pat.okeeffe@eg4u.ie ATTM AT2 Olivier Bouffant ATTM/AT2 Chairman olivier.bouffant@orange.com |  | https://www.etsi.org/committee/1390-attm | SG15 TD 580 WP1 April 2021 |
| ETSI TC DECT Digital Enhanced Cordless Telecommunications |  |  | https://www.etsi.org/committee/1394-dect |  |
| ETSI ISG F5G 5th Generation Fixed Network |  |  | https://www.etsi.org/committee/1696-f5g | SG15 TD 575 WP1 April 2021 Liaison Rapporteur Tony Zeng Huawei China tony.zengyan@huawei.com |
| ETSI TC EE Environmental Engineering |  |  | https://www.etsi.org/committee/1395-ee |  |
| ETSI BRAN Broadband Radio Access Networks |  |  | https://www.etsi.org/committee/1389-bran |  |
| IEEE 802.3 | David Law Chair, IEEE 802.3 Ethernet Working Group dlaw@hpe.com |  | http://www.ieee802.org/3/ | SG15 TD 720 WP1 December 20221 SG15 TD 466 GEN April 2021 Liaison Rapporteur Peter Stassar peter.stassar@huawei.com |
| IEEE 802.11 |  |  | https://www.ieee802.org/11/ |  |
| IEEE 802.15 |  |  | http://www.ieee802.org/15/ |  |
| IEEE 1901 Working Group on Power Line Communications (COM/PLC) | Chair, Jean-Philippe Faure jean-philippe.faure@progilon.com |  | https://sagroups.ieee.org/1901/ | SG15 TD 639 WP1 April 2021 |
| MoCA® Multimedia over Coax Alliance | Roberta Silverstein, Managing Director Multimedia over Coax Alliance, MoCA (R) robertas@mocalliance.org |  | www.mocalliance.org www.mocainyourhouse.com | SG15 TD 578 WP1 April 2021 |
| HomeGrid Forum |  |  | http://www.homegridforum.org/ | SG15 TD 650 WP1 December 2021 |
| ITU-R WP1A | John Shaw Chairman, Correspondence Group on EMC Related Interference and Coexistence of wired telecommunication systems with radiocommunication systems shawzone@gmail.com |  | www.itu.int/en/ITU-R/study-groups/Pages/default.aspx | SG15 TD 704 WP1 December 2021 SG15 TD 554 WP1 April 2021 |
| ITU-R WP5C |  |  | www.itu.int/en/ITU-R/study-groups/rsg5/rwp5c/Pages/default.aspx |  |
| ITU-R WP5D |  |  | www.itu.int/en/ITU-R/study-groups/rsg5/rwp5d/Pages/default.aspx |  |
| ITU-R WP6A | John Shaw Rapporteur on Power Line Telecommunication (PLT) and general EMC-related potential shawzone@gmail.com |  | http://www.itu.int/en/ITU-R/study-groups/rsg6/Pages/default.aspx | SG15 TD 547 WP1 April 2021 |
| ITU-R WP6B | Paul Gardiner Chairman, WP6B paul.gardiner@eu.sony.com |  | https://www.itu.int/en/ITU-R/study-groups/rsg6/rwp6b/Pages/default.aspx |  |
| ITU-T SG5 | Fryderyk Lewicki WP1/5 Chairman Orange Polska fryderyk.lewicki@orange.com |  | http://www.itu.int/en/ITU-T/studygroups/2017-2020/05/Pages/default.aspx | SG15 TD 548 WP1 April 2021 |
| ITU-T SG9 | Jingyi Xue Rapporteur of Q10/9 ABP, NRTA China xuejingyi@abp2003.cn Shizhu Long Shenzhen Skyworth Digital Technology Co. Ltd China Rapporteur of Q6/9 longshizhu@skyworth.com |  | http://www.itu.int/en/ITU-T/studygroups/2017-2020/09/Pages/default.aspx | SG15 TD 7012 WP1 December 2021 SG15 TD 705 WP1 December 2021 |
| ITU-T SG11 | Andrey Kucheryavy SG11 Chairman akouch@mail.ru |  | http://www.itu.int/en/ITU-T/studygroups/2017-2020/11/Pages/default.aspx | SG15 TD 471 GEN April 2021 |
| ITU-T SG13 |  |  | http://www.itu.int/en/ITU-T/studygroups/2017-2020/13/Pages/default.aspx |  |
| ITU-T SG16 | Sarra Rebhi Office National de la télédiffusion Tunisia rebhi.sarra@telediffusion.net.tn |  | http://www.itu.int/en/ITU-T/studygroups/2017-2020/16/Pages/default.aspx | SG15 TD 642 WP1 December 2021 |
| ITU-T SG17 | Jonghyun Baek Rapporteur of Q6/17 jhbaek@kisa.or.kr |  | http://www.itu.int/en/ITU-T/studygroups/2017-2020/17/Pages/default.aspx | SG15 TD 483 WP1 September 2020 |
| ITU-T SG20 |  |  | http://www.itu.int/en/ITU-T/studygroups/2017-2020/20/Pages/default.aspx |  |
| ITU-D SG1 | Mr Vadym Kaptur Co-Rapporteur for Question 1/1 A.S. Popov Odessa National Academy of Telecommunications Ukraine vadim.kaptur@onat.edu.ua Mr Fred Onchoka Ong'aro Co-Rapporteur for Question 1/1 Communications Authority of Kenya (CA) Kenya ongaro@ca.go.ke |  | https://www.itu.int/net4/ITU-D/CDS/sg/index.asp?lg=1&sp=2018&stg=1 | SG15 TD 653 WP1 December 2021 |
| ITU-D SG2 |  |  | https://www.itu.int/net4/ITU-D/CDS/sg/index.asp?lg=1&sp=2018&stg=2 |  |
| ISO/IEC JTC 1/SC 25 | Marco Peter Secretary ISO/IEC JTC 1/SC 25 peter@zvei.org |  | https://www.iso.org/committee/45270.html | SG15 TD 546 WP1 April 2021 |
| TIA = Telecommunication Industry Association TR-41, TR-42 |  |  | https://standards.tiaonline.org/ |  |

_________________________________

____________________