MOD	IAP/5/187

ANNEX  4     (WRC-03)

Need for coordination of a transmitting space station in the fixed-satellite service or in the broadcasting-satellite service where this service is not
subject to a Plan: in Region 2 (11.7-12.2 GHz) with respect to the
 Plan, the List or proposed new or modified 
assignments in the List for Regions 1 and 3; in Region 1 
(12.5-12.7 GHz) and in Region 3 (12.2-12.7 GHz) with 
respect to the Plan or proposed modifications 
to the Plan in Region 2; in Region 3 (12.2-12.5 GHz) 
with respect to the Plan, List or proposed new or 
modified assignments in the List for Region 1

(See Article 7)

With respect to § 7.1 and 7.2 of Article 7, coordination of a space station in the fixed-satellite service of Region 2 or Region 3 is required with another administration when, under assumed free-space propagation conditions, the power flux-density over the service area of a space station in the broadcasting-satellite service of Region 1 or Region 3 with overlapping frequency assignments exceeds the  following:

–147 	 	dB(W/(m2  27 MHz))			for	0°	  < 0.23°

–135.7 + 17.74 log 	dB(W/(m2  27 MHz))		for	0.23°	  < 2.0°

–136.7 + 1.66 2	dB(W/(m2  27 MHz)) 		for	2.0°	  < 3.59°

–129.2 + 25 log 		dB(W/(m2  27 MHz))		for	3.59°	  < 10.57°

–103.6 		dB(W/(m2  27 MHz))			for	10.57	 

where  is

the difference in degrees between the longitude of the interfering fixed-satellite service space station in Region 2 and the longitude of the affected broadcasting-satellite service space station in Regions 1 and 3.

With respect to § 7.1 and 7.2 of Article 7, coordination of a space station in the fixed-satellite service (space-to-Earth) in Region 1 or 3 or broadcasting-satellite service not subject to a Plan in Region 3 is required when, under assumed free-space propagation conditions, the power flux-density over the service area of a space station in the broadcasting-satellite service of Region 2 with overlapping frequency assignments exceeds the following:

–147 	 	dB(W/(m2  27 MHz))			for	0°	  < 0.23°

–135.7 + 17.74 log 	dB(W/(m2  27 MHz))		for	0.23°	  < 2.0°

–136.7 + 1.66 2	dB(W/(m2  27 MHz)) 		for	2.0°	  < 3.59°

–129.2 + 25 log 		dB(W/(m2  27 MHz))		for	3.59°	  < 10.57°

–103.6 		dB(W/(m2  27 MHz))			for	10.57	 

where  is

the difference in degrees between the longitude of the interfering fixed-satellite service space station in Region 1 or 3 or the interfering broadcasting-satellite service space station in Region 3 and the longitude of the affected broadcasting-satellite service space station in Region 2.

Reasons:	Based on results of ITU-R studies provides protection to Region 2 BSS networks employing antenna diameters in the range 45-240 cm and protection of Region 1 & 3 BSS networks employing antenna diameters in the range 60-240 cm.