MOD	IAP/5/209

### 1.4.4	Earth stations operating in bidirectionally allocated frequency bands

For earth stations operating in some frequency bands there may be allocations to space services operating with equal rights in both the Earth-to-space and space-to-Earth directions. In this case, where two earth stations are operating in opposite directions of transmission it is only necessary to establish the coordination area for the transmitting earth station, as receiving earth stations will automatically be taken into consideration. Hence, a receiving earth station operating in a bidirectionally allocated frequency band will only be involved in coordination with a transmitting earth station if it is located within the transmitting earth station’s coordination area.

For a transmitting earth station operating with either geostationary or non-geostationary satellites in a bidirectionally allocated frequency band, the coordination area is determined using the procedures described in § 3.

Reasons:	Brings the text in line with existing No. 9.17A and Appendix 5 provisions.