MOD	IAP/5/215

TABLE  5-1     (WRC-03)

Technical conditions for coordination
(see Article 9)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO | A station in a satellite network using the geostationary-satellite orbit (GSO), in any space radiocommunication service, in a frequency band and in a Region where this service is not subject to a Plan, in respect of any other satellite network using that orbit, in any space radiocommunication service in a frequency band and in a Region where this service is not subject to a Plan, with the exception of the coordination between earth stations operating in the opposite direction of transmission | 1) 3 400-4 200 MHz 5 725-5 850 MHz (Region 1) and 5 850-6 725 MHz | i) Bandwidth overlap, and ii) any network in the fixed-satellite service (FSS) with a space station within an orbital arc of  10° of the nominal orbital position of a proposed network in the FSS |  | With respect to thespace services listed in the threshold/condition column in the bands in 1), 2), 3), 4) and 5), an administration may request, pursuant to No. 9.41, to be included in requests for coordination, indicating the networks for which the value of T/T calculated by the method in § 2.2.1.2 and 3.2 of Appendix 8 exceeds 6%. When the Bureau, on request by an affected administration, studies this information pursuant to No. 9.42, the calculation method given in § 2.2.1.2 and 3.2 of Appendix 8 shall be used. |
|  |  | 2) 10.95-11.2 GHz 11.45-11.7 GHz 11.7-12.2 GHz (Region 2) 12.2-12.5 GHz (Region 3) 12.5-12.75 GHz (Regions 1 and 3) 12.7-12.75 GHz (Region 2) and 13.75-14.5 GHz | i) Bandwidth overlap, and ii) any network in the FSS with a space station within an orbital arc of  9° of the nominal orbital position of a proposed network in the FSS |  |  |

TABLE  5-1 (continued)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 3) 17.7-20.2 GHz, and 27.5-30 GHz 4) Bands above 17.3 GHz, except those defined in § 3) 5) Bands above 17.3 GHz | i) Bandwidth overlap, and ii) any network in the FSS with a space station within an orbital arc of  8° of the nominal orbital position of a proposed network in the FSS i) Bandwidth overlap, and ii) any network in the FSS with a space station within an orbital arc of  8° of the nominal orbital position of a proposed network in the FSS (see also Resolution [Coord_Arc]) (WRC-03) i) Bandwidth overlap, and ii) any network in the FSS or BSS not subject to a plan with a space station within an orbital arc of  16° of the nominal orbital position of a proposed network in the FSS or BSS not subject to a plan, except for cases covered in § 4) above (see also Resolution [Coord_Arc]) (WRC-03) |  | With respect to thespace services listed in the threshold/condition column in the bands in 1), 2), 3), 4) and 5), an administration may request, pursuant to No. 9.41, that an administration be excluded from requests for coordination, giving as reason that the network of this administration will not be affected because value of T/T calculated by the method in § 2.2.1.2 and 3.2 of Appendix 8 do not exceed 6%. When the Bureau, at the request of an administration, studies this information pursuant to No. 9.42, the calculation method given in § 2.2.1.2 and 3.2 of Appendix 8 shall be used |

TABLE  5-1 (continued)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.7 GSO/GSO (cont.) |  | 6) All frequency bands, other than those in § 1), 2), 3), 4) and 5), allocated to a space service, and the bands in § 1), 2), 3), 4) and 5) where the radio service of the proposed network or affected networks is other than the space services listed in the threshold/ condition column, or in the case of coordination of space stations operating in the opposite direction of transmission | i) Bandwidth overlap, and ii) Value of T/T exceeds 6% | Appendix 8 |  |

Reasons:	Extend the coordination arc concept to all bands above 17.3 GHz. CITEL is further considering the appropriate interim value for the coordination arc in § 5) above and may have additional proposals for WRC-03. The final value(s) for the coordination arc will be determined by the ITU-R studies to be conducted under Resolution [coord_arc] (see the following proposal). A value for the BSS coordination arc of  16° was selected on the basis that the spacing between BSS satellites is normally larger than the FSS such that the relative impact of another BSS satellite at 16° spacing will be comparable to the FSS case for an 8° spacing.