ADD	IAP/5/216

Draft  RESOLUTION  [COORD_ARC]  (WRC-03)

Determination of orbital arc separation for which coordination would be required between two satellite networks operating in a space 
service not subject to a plan

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that WRC-2000 adopted a coordination arc approach in Appendix 5 to simplify the coordination between fixed-satellite service (FSS) networks in certain frequency bands below 30 GHz;

b)	that the application of such a concept was limited to the frequency ranges where very large numbers of FSS satellite filings had been received by ITU-R;

c)	that many satellite networks and systems are now proposing to use higher frequency bands for which the coordination arc does not yet apply;

d)	that the RRB adopted a Rule of Procedure on No. 9.36 that extended the coordination arc concept to the FSS and broadcasting-satellite service (BSS), not subject to a plan, and in all bands above 3.4 GHz until a review by WRC-03;

e)	that the use of the coordination arc considerably reduces the volume of data that needs to be supplied to the Radiocommunication Bureau under Section D of Annex 2 to Appendix 4;

f)	that the coordination arc concept has the potential to greatly reduce the workload of the Bureau in identifying affected administrations;

g)	that the coordination arc concept is valid for all geostationary space stations operating in any space radiocommunication service that is not subject to a plan;

h)	that the ITU-R studies for other services and for frequency bands above 17.3 GHz, except the 17.7-20.2 GHz and 29.5-30 GHz ranges for the FSS, have not been completed;

i)	that the coordination arc concept could facilitate introduction of satellite services above 17.3 GHz after the studies conclude on the appropriate value(s) of the coordination arc,

recognizing

that there have been no difficulties resulting from the application of the coordination arc concept in the bands where it applies,

noting

a)	that ITU-R is under considerable budgetary constraints and that the adoption of a coordination arc for other bands and services should reduce the workload for the Radiocommunication Bureau;

b)	that WRC-03 extended the coordination arc of  8°for the FSS in bands above 17.3 GHz on a provisional basis and adopted an alternative value of  16° on a provisional basis for the coordination arc applicable for the BSS in these bands in Table 5-1 of Appendix 5,

resolves

1	that in the bands above 17.3 GHz, an interim value for the coordination arc for the FSS and BSS as indicated in items 4) and 5) of No. 9.7 in Table 5-1 of Appendix 5 shall apply until revised by a future competent conference;

2	that a future competent conference review the results of ITU-R studies for the application of the coordination arc value(s) to other frequency bands and other services, as applicable, and consider their inclusion in Appendix 5 to the Radio Regulations,

invites ITU-R

to continue studies on the appropriate value(s) for the coordination arc for space radiocommunication services in bands above 17.3 GHz, except for the FSS in the 17.7-20.2 GHz and 29.5-30 GHz bands,

instructs the Radiocommunication Bureau

to modify the current Rule of Procedure on No. 9.36 with respect to the coordination arc approach in bands above 17.3 GHz in order to use the provisional values described in resolves 1 until such time as ITU-R finalizes its work on the coordination arc value(s) to be used.

Reasons:	Allow for the continuation of studies with a view of incorporating the appropriate coordination arc value(s) to other services and other frequency bands, and adopt the interim coordination arc values until studies are completed.