MOD	IAP/5/218

D  –  Overall link characteristics

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |  | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Items in Appendix | Data Description | Advance publication of a geostationary-satellite network | Advance publication of a non-geostationary-satellite network subject to coordination under Section II of Article 9 | Advance publication of a non-geostationary-satellite network not subject to coordination under Section II of Article 9 | Notification or coordination of a geostationary-satellite network (including Appendix 30B) (75) | Notification or coordination of a non-geostationary-satellite network | Notification or coordination of an earth station (including notification under Appendices 30A and 30B (76) | Notice for a satellite network in the broadcasting-satellite service under Appendix 30 (77) | Notice for a satellite network (feeder-link) stations under Appendix 30A (78) | Notice for a satellite network in the fixed- satellite service under Appendix 30B (Articles 6 and 8) (75) | Items in Appendix | Radio astronomy |
| D | Overall link characteristics |  |  |  |  |  |  |  |  |  |  |  |
|  | May be provided by administrations that so desire but only when simple frequency-changing transponders are used on the space station on board a geostationary satellite. In the case of satellite networks listed in the threshold/condition column specified in No. 9.7 (GSO/GSO) of Table 5-1 of Appendix 5, § 1), 2) and 3) and using the frequencies listed in the frequency band column, the data specified in this section of the Appendix should not be submitted to the Bureau. |  |  |  |  |  |  |  |  |  |  |  |
| D.1 | Connection between Earth-to-space and space-to-Earth frequencies in the network |  |  |  | O |  |  |  |  |  | D.1 |  |
| D.1 | The connection between uplink and downlink frequency assignments in each transponder for each intended combination of receiving and transmitting beams. For the case of Appendices 30 and 30A required only for systems in Region 2. |  |  |  |  |  |  | X | X |  |  |  |
| D.2 | Transmission gains and associated equivalent satellite link noise temperatures |  |  |  |  |  |  |  |  |  |  |  |
| D.2 | For each entry under D.1: |  |  |  |  |  |  |  |  |  |  |  |
| D.2.a.1 | The lowest equivalent satellite link noise temperature. These values shall be indicated for the nominal value of the angle of elevation. |  |  |  | O |  |  |  |  |  | D.2.a |  |
| D.2.a.2 | The associated transmission gain of the lowest equivalent satellite link noise temperature. These values shall be indicated for the nominal value of the angle of elevation. The transmission gain is evaluated from the output of the receiving antenna of the space station to the output of the receiving antenna of the earth station. |  |  |  | O |  |  |  |  |  |  |  |
| D.2.b.1 | The values of transmission gain that correspond to the highest ratio of transmission gain to equivalent satellite link noise temperature. |  |  |  | O |  |  |  |  |  | D.2.b |  |
| D.2.b.2 | The values of associated equivalent satellite link noise temperature that corresponds to the highest ratio of transmission gain to equivalent satellite link noise temperature. |  |  |  | O |  |  |  |  |  |  |  |

X	Mandatory information	O  Optional information	C  Mandatory if used as a basis to effect coordination with another administration

Reasons:

1)	The addition of item D1 in Appendix 30/30A: currently not listed in RR Appendix 4, but the requirement for this data item was added at WRC-97 – see BR Circular Letter CR/158.

2)	To simplify the submission of AP 4 information, the overall link characteristics is made to be an optional field to be provided if desired for satellite systems not subject to the Appendix 30/30A plans. If administrations desire, they can provide this information so that the total increase in noise temperature can be computed.