MOD	IAP/5/220

## 3.2	Cases requiring independent treatment of the uplink and the downlink

a)	In the case of interference into only one link, the uplink or the downlink, the value  Te /Te or  Ts /Ts, expressed as a percentage, shall be compared with the threshold value of 6%3.

b)	In the case of interference into both the uplink and the downlink, between which there is a change of modulation on board the satellite, or in cases where the optional values for transmission gain and equivalent link noise temperature have not been supplied, the values of  Te /Te and  Ts /Ts, expressed as a percentage, shall each be compared with the threshold value of 6%3.

Reasons:	Add the case where the transmission gain and equivalent link noise temperatures have not been supplied (see proposal IAP/5/215 under item D of Appendix 4).