MOD	IAP/5/246

RESOLUTION  221  (Rev.WRC-03)

Use of high altitude platform stations as IMT-2000 base stations in the bands 1 885-1 980 MHz, 2 010-2 025 MHz and 2 110-2 170 MHz in Regions 1 and 3 and 1 885-1 980 MHz and 2 110-2 160 MHz in Region 2

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the bands 1 885-2 025 MHz and 2 110-2 200 MHz are identified in No. 5.388 as intended for use on a worldwide basis IMT-2000, including the bands 1 980-2 010 MHz and 2 170-2 200 MHz for both the terrestrial and the satellite component of IMT-2000;

b)	that a high altitude platform station (HAPS) is defined in No. 1.66A as “a station located on an object at an altitude of 20 to 50 km and at a specified, nominal, fixed point relative to the Earth”;

c)	that HAPS may offer a new means of providing IMT-2000 services with minimal network infrastructure as they are capable of providing service to a large footprint together with a dense coverage;

d)	that the use of HAPS as base stations within the terrestrial component of IMT-2000 is optional for administrations, and that such use should not have any priority over other terrestrial IMT-2000 use;

e)	that, in accordance with No. 5.388 and Resolution 212 (Rev.WRC-97), administrations may use the bands identified for IMT-2000, including the bands referred to in this Resolution, for stations of other primary services to which they are allocated;

f)	that these bands are allocated to the fixed and mobile services on a co-primary basis;

-

g)	that, in accordance with No. 5.388A, HAPS may be used as base stations within the terrestrial component of IMT-2000 in the bands 1 885-1 980 MHz, 2 010-2 025 MHz and 2 110-2 170 MHz in Regions 1 and 3 and 1 885-1 980 MHz and 2 110-2 160 MHz in Region 2; the use by IMT-2000 applications using HAPS as base stations does not preclude the use of these bands by any station in the services to which they are allocated and does not establish priority in the Radio Regulations;

h)	that ITU-R has studied sharing between HAPS and other stations within IMT-2000, has considered compatibility of HAPS within IMT-2000 with some services having allocations in the adjacent bands, and has established Recommendation ITU-R M.1456;

i)	that radio interfaces of IMT-2000 HAPS are compliant with Recommendation ITU-R M.1457;

j)	that ITU-R has addressed sharing between systems using HAPS and some existing systems, particularly PCS (personal communications system), MMDS (multichannel multipoint distribution system) and systems in the fixed service, which are currently operating in some countries in the bands 1 885-2 025 MHz and 2 110-2 200 MHz;

k)	that HAPS stations are intended to transmit in the band 2 110-2 170 MHz in Regions 1 and 3 and in the band 2 110-2 160 MHz in Region 2,

resolves

1	that:

1.1	for the purpose of protecting IMT-2000 mobile stations -in neighbouring countries from co-channel interference, a HAPS operating as an IMT-2000 base station shall not exceed a co-channel power flux-density (pfd) of _117 dB(W/(m2 · MHz)) at the Earth’s surface outside a country’s borders unless explicit agreement of the affected administration is provided at the time of the notification of the HAPS station;

--

1.2	a HAPS operating as an IMT-2000 base station shall not transmit outside the frequency bands 2 110-2 170 MHz in Regions 1 and 3 and 2 110-2 160 MHz in Region 2;

1.3	in Region 2, for the purpose of protecting MMDS stations in some neighbouring countries in the band 2 150-2 160 MHz from co-channel interference, a HAPS operating as an IMT-2000 base station shall not exceed the following co-channel pfd at the Earth’s surface outside an country’s borders unless explicit agreement of the affected administration is provided at the time of the notification of the HAPS station:

–	–127 dB(W/(m2 · MHz)) for angles of arrival (q) less than 7° above the horizontal plane;

–	–127 + 0.666 (q – 7) dB(W/(m2 · MHz)) for angles of arrival between 7° and 22° above the horizontal plane; and

–	–117 dB(W/(m2 · MHz)) for angles of arrival between 22° and 90° above the horizontal plane;

1.4	a HAPS operating as a IMT-2000 base station, in order to protect fixed stations from interference, shall not exceed the following limits of out-of-band pfd at the Earth’s surface in the bands 2 025-2 110 MHz:

	–165 dB(W/(m2 · MHz)) for angles of arrival () less than 5 above the horizontal plane;

	–165 + 1.75 ( – 5) dB(W/(m2 · MHz)) for angles of arrival between 5 and 25 above the horizontal plane; and

	–130 dB(W/(m2 · MHz)) for angles of arrival between 25 and 90 above the horizontal plane;

2	that all the limits in this Resolution shall apply to all HAPS stations operating in accordance with No. 5.388A as of 1 January 2002;

3	that administrations wishing to implement HAPS within a terrestrial IMT-2000 system shall comply with the following:

3.1	for the purpose of protecting IMT-2000 stations operating in neighbouring countries from co-channel interference, a HAPS operating as a base stations within IMT-2000 shall use antennas that comply with the following antenna pattern:

G(y)  Gm – 3(y/yb)2 	dBi		for	 0°	£ y £ y1

G(y)  Gm  LN	dBi		for	y1	< y £ y2

G(y)  X – 60 log (y)	dBi		for	y2	< y £ y3

G(y)  LF		dBi		for	y3	< y £ 90°

where:

G(y)  :	gain at the angle y from the main beam direction (dBi)

Gm  :	maximum gain in the main lobe (dBi)

yb  : 	one-half of the 3 dB beamwidth in the plane considered (3 dB below Gm) (degrees)

LN  : 	near side-lobe level in dB relative to the peak gain required by the system design, and has a maximum value of –25 dB

LF  :	far side-lobe level, Gm – 73 dBi

y1  yb −LN3		degrees

y2  3.745 yb    	degrees

X  Gm  LN + 60 log (y2)	dBi

y3 = 10(X−LF)60	degrees

The 3 dB beamwidth (2yb) is again estimated by:

(yb)2  7 442/(100.1Gm)	degrees2

where Gm is the peak aperture gain (dBi);

3.2	for the purpose of protecting mobile earth stations within the satellite component of IMT-2000 from interference, a HAPS operating as an IMT-2000 base station, shall not exceed an out-of-band pfd of –165 dB(W/(m2 ·  4 kHz)) at the Earth’s surface in the bands 2 160-2 200 MHz in Region 2 and 2 170-2 200 MHz in Regions 1 and 3,

invites ITU-R

-

to develop an ITU-R Recommendation providing technical guidance to facilitate consideration with neighbouring administrations,

instructs the Bureau

to review the findings made under No. 11.31 with respect to the conformity of HAPS stations operating in accordance with No. 5.388A, and notified to the Bureau after 1 January 2002.

Reasons:	ITU-R has addressed sharing and coordination between HAPS and existing systems, particularly IMT-2000 stations operating in adjacent countries and MMDS (multichannel multipoint distribution service), which are currently operating in the bands 1 885-2 025 MHz and 2 110-2 200 MHz. The Resolution is being modified to reflect the conclusions of the studies, including the update of the pfd thresholds.