MOD	IAP/5/263

RESOLUTION  77  (Rev.WRC-03)

Protection of terrestrial services in all Regions from geostationary-satellite networks in the fixed-satellite service in Region 2 
using the frequency band 11.7-12.2 GHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that, in Regions 1 and 3, the band 11.7-12.2 GHz is allocated on a co-primary basis to terrestrial services and to the broadcasting-satellite service (BSS);

b)	that, in Region 2, the band 11.7-12.1 GHz is allocated on a co-primary basis to terrestrial services (except in the countries listed in No. 5.486) and to the fixed-satellite service (FSS);

c)	that, in Region 2, the band 12.1-12.2 GHz is allocated on a co-primary basis to terrestrial services in Peru (see No. 5.489) and to the FSS;

d)	that protection of the BSS in Regions 1 and 3 from the FSS in Region 2 is assured by Article 7 and Annex 4 of Appendix 30;

e)	that protection of the FSS in Region 2 from the FSS in that Region is assured either by Article 9 (Nos. 9.7 or 9.12) or Article 22;

f)	that protection of terrestrial services in Regions 1, 2 and 3 from non-geostationary- satellite (non-GSO) systems in the FSS in Region 2 is assured by Article 21;

g)	that there is a need to protect terrestrial services in Regions 1, 2 and 3 from GSO networks in the FSS in Region 2;

h) 	that this Conference has modified No. 5.488 by revising the regulatory limitations on the operation of GSO networks in the FSS in Region 2 in the band 11.7-12.2 GHz,

recognizing

that ITU-R has developed Recommendation ITU-R SF.674-1, dealing with sharing between the FSS and the fixed service in the band 11.7-12.2 GHz in Region 2,

resolves

that, before an administration notifies to the Radiocommunication Bureau or brings into use, in Region 2, a frequency assignment for a GSO network in the FSS in the 11.7-12.2 GHz band, it shall effect coordination, using the procedures in Articles 9 and 11 associated with No. 9.14, with any administration of Regions 1, 2, and 3 having a primary allocation to terrestrial services in the same frequency band if the power flux-density (pfd) produced on its territory exceeds the following thresholds:

–124 		dB(W/(m2 · MHz))	for 	0°   	5

–124  0.5 (– 5) 	dB(W/(m2 · MHz))	for 	5° <  	25

–114 		dB(W/(m2 · MHz))	for	         >	25

where  is the angle of arrival of the incident wave above the horizontal plane (degrees)1,

instructs the Radiocommunication Bureau

in its examination of requests for coordination for any geostationary space station in the FSS operating in the band 11.7-12.2 GHz in Region 2, to determine if the pfd thresholds under resolves above are exceeded on the territory of any administration, other than the notifying administration, having a primary allocation to terrestrial services and, if so, to so notify both the notifying and the affected administrations.

Reasons:	There is a requirement to clarify the resolves of Resolution 77 to identify a coordination mechanism that maintains, intact, the procedural flexibility initially intended by the Resolution through the use of ITU-R developed power flux-density thresholds.

1	 	These values relate to the pfd and angles of arrival which would be obtained under free-space propagation conditions.