MOD	IAP/5/30

## 1.1	Channel spacing

In a mixed DSB, SSB and digital environment (see Resolution 517 (Rev.WRC-03)), the channel spacing shall be 10 kHz. In the interest of spectrum conservation, it is also permissible to interleave SSB emissions midway between two adjacent DSB channels, i.e., with 5 kHz separation between carrier frequencies, provided that the interleaved emission is not to the same geographical area as either of the emissions between which it is interleaved.

In an all inclusive SSB environment, the channel spacing and carrier frequency separation shall be 5 kHz.

Reasons:	Updating this text to reflect digital and SSB use and deleting text concerning the transition period. No change in the carrier reduction levels.