MOD	IAP/5/31

## 2.6	Carrier reduction (relative to peak envelope power)

In a mixed DSB, SSB and digital environment, the carrier reduction shall be 6 dB to allow SSB emissions to be received by conventional DSB receivers with envelope detection without significant deterioration of the reception quality.

In an all inclusive SSB environment, the carrier reduction shall be 12 dB.

Reasons:	Updating this text to reflect digital and SSB use and deleting text concerning the transition period. No change in the carrier reduction levels.