MOD	IAP/5/51

5.444A	Additional allocation:  the band 5 091-5 150 MHz is also allocated to the fixed-satellite service (Earth-to-space) on a primary basis. This allocation is limited to feeder links of non-geostationary mobile-satellite systems and is subject to coordination under No. 9.11A.

In the band 5 091-5 150 MHz, the following conditions also apply:

–	prior to 1 January 2018, the use of the band 5 091-5 150 MHz by feeder links of non-geostationary-satellite systems in the mobile-satellite service shall be made in accordance with Resolution 114 (Rev.WRC-03);

–	prior to 1 January 2018, the requirements of existing and planned international standard systems for the aeronautical radionavigation service which cannot be met in the 5 000-5 091 MHz band, shall take precedence over other uses of this band;

–	after 1 January 2016, no new assignments shall be made to stations providing feeder links of non-geostationary mobile-satellite systems;

–	after 1 January 2018, the fixed-satellite service will become secondary to the aeronautical radionavigation service.