ADD	IAP/5/68

DRAFT  RESOLUTION  [5150]  (WRC-03)

Use of the band 5 150-5 250 MHz by the mobile service for the implementation of wireless access systems including RLANs

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that FSS (Earth-to-space) is allocated worldwide on a primary basis in the band 5 150-5 250 MHz, this allocation being limited to feeder links of non-geostationary-satellite systems in the mobile-satellite service (No. 5.447A);

b)	that there is a need for continued operation of non-GSO FSS systems used for feeder links in the mobile-satellite service in this band;

c)	that results of studies in ITU-R indicate that sharing in the band 5 150-5 250 MHz between wireless access systems, including RLANs, and the FSS is feasible under specified conditions;

d)	that very large numbers of wireless access system, including RLAN, transmitters are expected to be deployed in individual countries, and can result in global deployment of such devices;

e)	that the interference caused by very large numbers of transmitters in the mobile service which individually meet the emission limits contained in this Resolution can cause impairment to the non-GSO FSS feeder-link transmissions due to the aggregate level of interference at the satellite;

f)	that the aggregate effect will be due to the global deployment of RLAN devices that accrue from emissions from multiple administrations,

recognizing

a)	that maximum power flux-density levels have been developed in Recommendations ITU-R M.1454 and ITU-R S.1426;

b)	that means may exist to measure or calculate the aggregate power flux-density levels caused by very large numbers of wireless access system including RLAN, transmitters at the non-GSO FSS satellite used to provide feeder links in the mobile-satellite service to a precision sufficient for determining whether those in this Resolution have been or are close to being exceeded,

resolves

1	that the use of this band by the mobile service shall be limited to wireless access systems including RLANs (WAS/RLANs) as described in Recommendation ITU-R M.1450;

2	that the WAS/RLAN stations must be operated in accordance with the following conditions:

–	that the deployment of wireless access systems (WAS), including RLANs shall be restricted to indoor use only;

–	that the maximum mean e.i.r.p. density of any WAS stations, including RLAN stations, shall not exceed –20 dBW (10 mW) in any 1 MHz band;

–	that the maximum mean1 e.i.r.p. of any WAS station shall not exceed –7 dBW (200 mW);

3	that administrations encourage the determination of the aggregate power flux-density levels caused by stations of the wireless access systems, including RLANs, in the mobile service using ITU-R recommendations where appropriate;

4	that, in the event of exceedence of the following aggregate PFD limits:

–	–124 - 20 log10 (hSAT /1 414) dB(W/(m2 · 1 MHz)), or equivalently,

–	–148 - 20 log10 (hSAT/1 414) dB(W/(m2 · 4 kHz)), at the FSS satellite orbit and where hSAT is the altitude of the satellite (km);

a future competent conference may review the findings made pursuant to resolves 3, with a view to taking appropriate action including review of the emission limits in this Resolution (See Recommendation ITU-R S.1426),

invites ITU-R

to continue developing methods for calculating and measuring aggregate power flux-density levels at non-GSO FSS space stations caused by transmitters of wireless access system, including RLANs operating in the band 5 150-5 250 MHz.

“Mean power” refers here to the e.i.r.p. during the transmission burst which corresponds to the 	highest power, if power control is implemented. NOTE – Mean is arithmetically equivalent to 	average.

1