MOD	IAP/5/78

Section II  –  Spurious domain emission limits for transmitters installed after 1 January 2003 
and for all transmitters after 1 January 2012

Application of these limits

7	The frequency range of the measurement of spurious domain emissions is from 9 kHz to 110 GHz or the second harmonic if higher.

8	Guidance regarding the methods of measuring spurious domain emissions is given in the most recent version of Recommendation ITU-R SM.329. The e.i.r.p. method specified in that Recommendation should be used when it is not possible to accurately measure the power supplied to the antenna transmission line (for example, radars), or for specific applications where the antenna is designed to provide significant attenuation in the spurious domain. Additionally, the e.i.r.p. method may need some modification for special cases, e.g. beam forming radars.     (WRC-03)

9	Except as provided in No. 9bis and No. 10, the spurious domain emission levels are specified in the following reference bandwidths:

–	1 kHz between 9 kHz and 150 kHz

–	10 kHz between 150 kHz and 30 MHz

–	100 kHz between 30 MHz and 1 GHz

–	1 MHz above 1 GHz.

9bis	As a special case, the reference bandwidth of all space service spurious domain emissions should be 4 kHz.

10	Guidance regarding the methods of measuring unwanted emissions from radar systems, in particular, spurious domain emissions, is given in the most recent version of Recommendation ITU-R M.1177. The reference bandwidths required for proper measurement of radar spurious domain emissions should be calculated for each particular radar system according to the guidance given in the most recent version of Recommendation ITU-R M.1177.

For those radar systems for which acceptable methods of measurement do not exist, the lowest practicable power of spurious domain emission should be achieved.

Reasons:	The provisions of this Section apply to unwanted emissions in the spurious domain, here called “spurious domain emissions,” as opposed to the spurious emissions addressed in Section I. Recommendation ITU-R M.1177 is currently being amended by WP 8B to cover all the unwanted emissions (i.e. falling in the OOB domain and the spurious domain). To cover all the cases covered in this Recommendation in terms of measurement bandwidths, the easiest way is to add a reference to this Recommendation in Appendix 3.