ADD	IAP/5/83

ANNEX 1

Determination of the boundary between the
 out-of-band and spurious domains

1	Except as provided below, the boundary between the out-of-band and spurious domains occurs at frequencies that are separated from the centre frequency of the emission by the values shown in Table 1. For most systems, the centre frequency of the emission is the centre of the necessary bandwidth. For multichannel or multicarrier transmitters/transponders, where several carriers may be transmitted simultaneously from a final output amplifier or an active antenna, the centre frequency of the emission is taken to be the centre of the 3 dB bandwidth of the transmitter or transponder and the transmitter or transponder bandwidth is used in place of the necessary bandwidth for determining the boundary. Some systems specify unwanted emissions relative to channel bandwidth, or channel spacing. These may be used as a substitute for the necessary bandwidth in Table 1, provided they are found in ITU–R recommendations.

TABLE 1

Values for frequency separation between the centre frequency 
and the boundary of the spurious domain

| Frequency range | Narrow-band case |  | Normal separation | Wideband case |  |
| --- | --- | --- | --- | --- | --- |
|  | for BN < | Separation |  | for BN > | Separation |
| 9 kHz < fc < 150 kHz | 250 Hz | 625 Hz | 2.5 BN | 10 kHz | 1.5 BN + 10 kHz |
| 150 kHz < fc < 30 MHz | 4 kHz | 10 kHz | 2.5 BN | 100 kHz | 1.5 BN + 100 kHz |
| 30 MHz < fc < 1 GHz | 25 kHz | 62.5 kHz | 2.5 BN | 10 MHz | 1.5 BN + 10 MHz |
| 1 GHz < fc < 3 GHz | 100 kHz | 250 kHz | 2.5 BN | 50 MHz | 1.5 BN + 50 MHz |
| 3 GHz < fc < 10 GHz | 100 kHz | 250 kHz | 2.5 BN | 100 MHz | 1.5 BN + 100 MHz |
| 10 GHz < fc < 15 GHz | 300 kHz | 750 kHz | 2.5 BN | 250 MHz | 1.5 BN + 250 MHz |
| 15 GHz < fc < 26 GHz | 500 kHz | 1.25 MHz | 2.5 BN | 500 MHz | 1.5 BN + 500 MHz |
| fc > 26 GHz | 1 MHz | 2.5 MHz | 2.5 BN | 500 MHz | 1.5 BN + 500 MHz |
| NOTE – In Table 1, fc is the centre frequency of the emission and BN is the necessary bandwidth. If the assigned frequency band of the emissions extends across two frequency ranges, then the values corresponding to the higher frequency range shall be used for determining the boundary. |  |  |  |  |  |

Example 1: The necessary bandwidth of an emission at 26 MHz is 1.8 kHz. Since 2.5 BN  is only 4.5 kHz, the minimum separation applies. The spurious domain begins 10 kHz each side of the centre of the necessary bandwidth.

Example 2: The necessary bandwidth of an emission at 8 GHz is 200 MHz. Since the wideband case applies for BN  > 100 MHz at that frequency, the spurious domain begins 400 MHz each side of the centre of the necessary bandwidth. Using the general separation formula, the out-of-band domain would have extended to 2.5 × 200 MHz = 500 MHz either side of the centre frequency.

2	Tables 2 and 3 show exceptions to Table 1 for narrow-band and wideband cases, respectively, applicable to particular systems or services and frequency bands.

TABLE 2

Narrow-band variations for systems or services and frequency bands

| System or service | Frequency range |  | Narrow-band case |  |
| --- | --- | --- | --- | --- |
|  |  |  | for BN < (kHz) | Separation (kHz) |
| Fixed service | 14 kHz-1.5 MHz |  | 20 | 50(1) |
|  | 1.5-30 MHz | PT ≤ 50 W | 30 | 75(2) |
|  |  | PT > 50 W | 80 | 200(2) |
| (1) The separation value is based on an assumption that the maximum value of the necessary bandwidth is about 3 kHz for the frequency range 14 kHz-1.5 MHz. The value of 50 kHz separation is extremely large as compared with the necessary bandwidth. It is because unwanted emissions of high power transmitters under modulated conditions have to be below the spurious limit (70 dBc) at the boundary between the OoB and spurious domains. (2) PT is the transmitter power. The separation values are based on an assumption that the maximum value of the necessary bandwidth is about 12 kHz for the frequency range 1.5-30 MHz. The value of 200 kHz separation for PT > 50 W is extremely large as compared with the necessary bandwidth. It is because unwanted emissions of high power transmitters under modulated conditions have to be below the spurious limit, 70 dBc, at the boundary between the OoB and spurious domains. Also, if future systems in the fixed service operating in this frequency range require a necessary bandwidth larger than 12 kHz, it may become necessary to review the 200 kHz separation. These separation values were derived from the Category A spurious domain emission limits in Recommendation ITU-R SM.329. |  |  |  |  |

TABLE 3

Wideband variations for systems or services and frequency bands

| System or service | Frequency range | Wideband case |  |
| --- | --- | --- | --- |
|  |  | For BN > | Separation |
| Fixed service | 14-150 kHz | 20 kHz | 1.5 BN + 20 kHz |
| Fixed-satellite service (FSS) | 3.4-4.2 GHz | 250 MHz | 1.5 BN + 250 MHz |
| FSS | 5.725-6.725 GHz | 500 MHz | 1.5 BN + 500 MHz |
| FSS | 7.25-7.75 GHz and 7.9-8.4 GHz | 250 MHz | 1.5 BN + 250 MHz |
| FSS | 10.7-12.75 GHz | 500 MHz | 1.5 BN + 500 MHz |
| Broadcasting-satellite service (BSS) | 11.7-12.75 GHz | 500 MHz | 1.5 BN + 500 MHz |
| FSS | 12.75-13.25 GHz | 500 MHz | 1.5 BN + 500 MHz |
| FSS | 13.75-14.8 GHz | 500 MHz | 1.5 BN + 500 MHz |

3	For primary radar stations, the boundary between the out-of-band and spurious domains is the frequency at which the out-of-band limits specified in applicable ITU-R Recommendations are equal to the spurious limit defined in Table II of Appendix 3. Further studies need to be conducted within ITU-R to determine the appropriate spurious domain boundary for these systems.

Reasons:	Annex 1 is added for the following reasons:

•	Section II of this Appendix states that the emission limits apply to unwanted emissions in the spurious domain. This Annex is needed to determine the boundary between the out-of-band and spurious domains, and thus the frequencies to which the emission limits of Section II apply.

•	Table 1, taken from Recommendation ITU-R SM.1539-1, shows the normal boundary of 2.5BN, along with the narrow-band and wideband exception. The information in the Recommendation, along with the text of existing paragraphs 11 and 11bis, have been included, though they have been shortened to bring them in line with the form of other appendices.

•	Tables 2 and 3 are also taken from Recommendation ITU-R SM.1539-1.

•	Studies regarding the frequencies to which the Section II limits for primary radar apply will not be completed in time for WRC-03. This text is similar to that of Annex 8, § 5 of Recommendation ITU-R SM.1541.

RECOMMENDATION  66  (Rev.WRC-2000)

Studies of the maximum permitted levels of unwanted emissions

The World Radiocommunication Conference (Istanbul, 2000),

...

recommends that ITU-R

...