MOD	USA/12/181

4.2.8	The Bureau shall determine, on the basis of Annex 1, the administrations whose frequency assignments are considered to be affected within the meaning of § 4.2.2 and § 4.2.2bis. The Bureau shall publish10, in a Special Section of its BR IFIC, the complete information received under § 4.2.6, together with the names of the affected administrations, the corresponding fixed-satellite service networks and the corresponding feeder links to broadcasting-satellite service assignments, as appropriate. The Bureau shall immediately send the results of its calculations to the administration proposing the modification to the Region 2 feeder-link Plan.

4.2.9	The Bureau shall send a telegram/fax to the administrations listed in the Special Section of its BR IFIC drawing their attention to the information it contains and shall send them the results of its calculations.

4.2.10	An administration which considers that it should have been included in the list of administrations whose services are considered to be affected may, giving the technical reasons for so doing, request the Bureau to include its name in the list. The Bureau shall study this request on the basis of Annex 1 and shall send a copy of the request, with an appropriate recommendation, to the administration proposing the modification to the Region 2 feeder-link Plan.

1	0 	If the payments are not received in accordance with the provisions of Council Decision 482, as amended, on the implementation of cost recovery for satellite network filings, the Bureau shall cancel the publication, after informing the administration concerned. The Bureau shall inform all administrations of such action and that the network specified in the publication in question no longer has to be taken into consideration by the Bureau and other administrations. The Bureau shall send a reminder to the notifying administration, not later than 60 days prior to due date of the payment if payment has not been received by that date. This provision was identified in reply to Resolution 88 (Minneapolis, 1998) of the Plenipotentiary Conference and shall enter into force at a date to be determined by the forthcoming Plenipotentiary Conference.