MOD	USA/12/192

4.1.19	Should the assignments that were the basis of the disagreement not be brought into use within the period specified in No. 11.44 (for non-planned services), then the status of the assignment in the List shall be reviewed accordingly.

4.1.20	Should harmful interference be caused by an assignment included in the List under § 4.1.18 to any recorded assignment in the Master Register which was the basis of the disagreement, the administration using the frequency assignment included in the List under § 4.1.18 shall, upon receipt of advice thereof, immediately eliminate this harmful interference.

4.1.21	An administration may, at any stage in the procedure described, or before applying it, request the assistance of the Bureau.

4.1.22	The relevant provisions of Article 5 shall be applied when frequency assignments are notified to the Bureau.

4.1.23	When a frequency assignment included in the List is no longer required, the administration concerned shall immediately so inform the Bureau. The Bureau shall publish this information in a Special Section of its BR IFIC and delete the assignment from the List.

4.1.24	No assignment in the List shall have a period of operation exceeding 15 years, counted from the date of bringing into use, or 2 June 2000, whichever is later. Upon request by the responsible administration received by the Bureau at the latest three years before the expiry of this period, this period may be extended by up to 15 years, on condition that all the characteristics of the assignment remain unchanged.

4.1.25	Where an administration already having included in the List two assignments (not including those systems notified on behalf of a group of named administrations and included in the List by WRC-2000), in the same channel and covering the same service area, proposes to include in the List a new assignment in the same channel over this same service area, it shall apply the following in respect of another administration which has no assignment in the List in the same channel and which proposes to include in the List a new assignment:

a)	if the agreement of the former administration is required following the application of § 4.1 by the latter administration, in order to protect the new assignment proposed by the former administration from interference caused by the assignment proposed by the latter administration, both administrations shall make every possible effort to resolve the difficulties by means of mutually acceptable adjustments to their networks;

b)	in case of continuing disagreement, and if the former administration has not communicated to the Bureau the information specified in Annex 2 to Resolution 49 (Rev.WRC-2000), this administration shall be deemed to have given its agreement to inclusion in the List of the assignment of the latter administration.

4.1.26	This procedure may be applied by the administration of a new ITU Member State in order to include new assignments in the List. Upon completion of the procedure, the next World Radiocommunication Conference may be requested to consider, among the assignments included in the List after the successful completion of this procedure, the inclusion in the Plan of up to 10 channels (for Region 1) and up to 12 channels (for Region 3), over the national territory of the new Member State.

4.1.27	When an administration has successfully applied this procedure and received all the agreements5 required to include in the List assignments over its national territory, at an orbital location and/or in channels different from those appearing in the Plan for its country, it may request the next world radiocommunication conference to consider the inclusion in the Plan of up to 10 (for Region 1) and up to 12 (for Region 3) of these assignments, in replacement of its assignments appearing in the Plan.

4.1.28	The List, as updated, shall be published periodically by the Bureau.

4.1.29	New or modified assignments in the List shall be limited to digital modulation.

## 4.2	Provisions applicable to Region 2

4.2.1	When an administration intends to make a modification6 to the Region 2 Plan, i.e.:

a)	to modify the characteristics of any of its frequency assignments to a space station in the broadcasting-satellite service which are shown in the Region 2 Plan, or for which the procedure in this Article has been successfully applied, whether or not the station has been brought into use; or

b)	to include in the Region 2 Plan a new frequency assignment to a space station in the broadcasting-satellite service; or

c)	to cancel a frequency assignment to a space station in the broadcasting-satellite service,

the following procedure shall be applied before any notification of the frequency assignment is made to the Bureau (see Article 5).

4.2.2	The term “frequency assignment in conformity with the Plan” used in this and the following Articles is defined in Article 1.

5	In such a case, § 4.1.18 does not apply.

6	For assignments using analogue modulation, the intention not to employ energy dispersal in accordance with § 3.18 of Annex 5 shall be treated as a modification and thus subject to the appropriate provisions of this Article.