ADD	USA/12/245

draft  RESOLUTION  [NSL]  (WRC-03)

Protection of radio astronomy spectral line observations 
in the 42.5-42.77 GHz band

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that there are primary allocations to the fixed-satellite service (FSS) (space-to-Earth) and to the broadcasting-satellite service (BSS) in the 42.0-42.5 GHz band;

b)	that the band 42.5-43.5 GHz  is allocated to the radio astronomy service (RAS) on a co-primary basis, and that spectral line observations are conducted in this band;

c)	that the threshold level of detrimental interference to spectral line observations by the RAS in the 42.5-43.5 GHz band is a power flux-density level of 153 dB(W/(m2 · 500 kHz)), and that this level relates to the emissions produced by all the space stations in any non-geostationary-satellite orbit (non-GSO) system or by any geostationary-satellite orbit (GSO) station;

d)	that an FSS or BSS satellite or system operating in the 42-42.5 GHz band would encounter the greatest difficulty in meeting the detrimental interference threshold for single dish radio telescope spectral line observations near the 42.5 GHz band edge of the 42.5-43.5 GHz band, even when all practicable technical or operational measures to reduce the potential for detrimental interference to the RAS receiver sites are employed;

e)	that because there are relatively few RAS sites operating with single dish telescopes in the band 42.5-43.5 GHz, and because there are expected to be relatively few FSS or BSS earth stations operating in the 42-42.5 GHz band, it may be feasible for either or both services to employ technical or operational measures, including but not limited to such interference mitigation techniques as geographical isolation, time sharing, etc., in order to reduce the potential for detrimental interference to the RAS receiver sites operating in this band;

f)	that, taking into account the above considerings, it should be feasible to rely on arrangements between concerned RAS and FSS/BSS administrations to ensure that the unwanted emissions for FSS and BSS satellites and systems at 42-42.5 GHz do not cause detrimental interference to RAS stations conducting spectral line observations in the 42.5-42.77 GHz band,

resolves

1	that an administration that plans to operate a GSO FSS or BSS satellite or a non-GSO FSS or BSS system in the 42-42.5 GHz band shall take all practicable steps, to avoid exceeding the detrimental interference threshold of –153 dB(W/(m2 · 500 kHz)) in the 42.5-42.77 GHz band, for more than 2% of the time, at the site of a radio astronomy station registered as a single dish telescope;

2	that, in the event that an administration planning a GSO FSS or BSS satellite or a non-GSO FSS or BSS system in the band 42-42.5 GHz has taken all practicable steps to avoid exceeding the detrimental interference threshold and percentage of time criterion in resolves 1 in the 42.5-42.77 GHz band, but that nevertheless would not meet them, the administration planning such a satellite or systems shall enter into discussions with the administration operating the affected radio astronomy station to arrive at a mutually satisfactory arrangement with respect to the unwanted emissions to be produced into the band 42.5-42.77 GHz;

3	that resolves 1 and 2 shall apply with respect to any radio astronomy station registered as a single dish telescope in the band 42.5-43.5 GHz that was in operation prior to the end of WRC-03 and that has been notified to the ITU before 4 January 2004, or that was notified before the date of receipt of the advance publication information of an FSS or BSS satellite or system to which the limits in No. 5.551G apply;

4	that any radio astronomy station that is notified as a single dish telescope after the dates provided in resolves 3 may seek an agreement with administrations authorizing FSS or BSS satellites or systems to which the limits in No. 5.551G apply;

5	that the obligations on administrations under this Resolution shall apply to any FSS or BSS network and system in the band 42-42.5 GHz for which complete coordination (GSO) or notification (non-GSO) information has been received after 3 June 2000,

instructs the Director of the Radiocommunication Bureau

to review, on the basis of this Resolution and the modification to No. 5.551G made by this Conference, any findings made pursuant to No. 5.551G (WRC-2000) and Resolution 128 (Rev.WRC-2000) in the interval between the end of WRC-2000 and the end of WRC-03.

Reasons:	To provide a coordination mechanism for the protection of radio astronomy stations conducting spectral line observations in the 42.5-42.77 GHz band from FSS and BSS satellites and systems operating in the 42-42.5 GHz band.