MOD	USA/12/249

TABLE  21-4 (continued)

| Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° |  | 25°-90° |  |
| ... |  |  |  |  |  |  |
| 37.5-40 GHz | Fixed-satellite (non-geostationary-satellite orbit) Mobile-satellite (non-geostationary-satellite orbit) | –120 10 | –120 + 0.75( – 5) 10 |  | –105 10 | 1 MHz |
| 37.5-40 GHz | Fixed-satellite (geostationary-satellite orbit) Mobile-satellite (geostationary-satellite orbit) | –127 | 5°-20° | 20°-25° | –105 | 1 MHz |
|  |  |  | –127 + (4/3) ( – 5) | –107 + 0.4 ( – 20) |  |  |
| 40-40.5 GHz | Fixed-satellite | –115 | –115 + 0.5( – 5) |  | –105 | 1 MHz |
| 40.5-42 GHz | Fixed-satellite (non-geostationary-satellite orbit) Broadcasting-satellite (non-geostationary-satellite orbit) | –115 10 | –115 + 0.5( – 5) 10 |  | –105 10 | 1 MHz |
| 40.5-42 GHz | Fixed-satellite (geostationary-satellite orbit) Broadcasting-satellite (geostationary-satellite orbit) | –120 | 5°-15° | 15°-25° | –105 | 1 MHz |
|  |  |  | –120 + ( – 5) | –110 + 0.5 ( – 15) |  |  |