MOD	USA/12/119

RESOLUTION  82  (Rev.WRC-03)

Provisions relating to earth stations located on board vessels 
operating in fixed-satellite service networks in the 
bands 5 925-6 425 MHz and 14.0-14.5 GHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that there is a demand for global wideband satellite communication services on vessels;

b)	that --ESVs are currently operating through FSS networks in the bands 3 700-4 200 MHz, 5 925-6 425 MHz, 10.7-12.75 GHz and 14.0-14.5 GHz;

c)	that ESVs have the potential to cause unacceptable interference to other services in the 5 925-6 425 MHz and 14.0-14.5 GHz (Earth-to-space) bands;

d)	that ESVs operating in these bands require considerably less than the full bandwidth in this FSS allocation and only a portion of the visible geostationary arc;

e)	that the number of vessels equipped with ESVs may be such that the procedures could place a heavy processing burden on some administrations, especially those in developing countries;

f)	that in order to ensure the protection and future growth of other services, ESVs should operate with requisite technical and operational constraints;

g)	that a minimum distance has been identified beyond which an ESV will not have the potential to cause unacceptable interference to other services in the bands 5 925-6 425 MHz and 14-14.5 GHz,

noting

a)	that ESVs may be assigned frequencies to operate in FSS networks in the bands- 5 925-6 425 MHz, 10.7-12.75 GHz and 14-14.5 GHz pursuant to No. 4.4 of the Radio Regulations and shall not claim protection from, nor cause interference to, other services having allocations in these bands;

b)	that existing regulatory procedures provide for ESVs operating at specified fixed points,

recognizing

a)	that progress has been made within ITU-R in determining the technical and operational provisions under which ESVs could operate; that the reference to the distances in resolves 2 is solely for the purpose of facilitating avoidance of radio interference and does not confer any territorial rights on administrations;

b)	that further studies are needed,

resolves

1	that transmissions from ESVs within the distances identified in resolves 2 of this Resolution be based upon the prior agreement of the concerned administrations;

2

to develop Recommendations on methods for coordination between terrestrial services and ESVs;

3	--that the minimum distances from the baseline (“low water mark”, as defined by the United Nations Convention on the Law of the Sea, 1982 (UNCLOS, 1982)) beyond which ESV stations will not have the potential to cause unacceptable interference to stations of other services of any administration and beyond which no agreement is necessary, are 300 km for the 5 925-6 425 MHz band and 125 km for the 14.0-14.5 GHz band;

4	that, until a decision is adopted for ESVs by WRC-03, agreement between the administrations licensing ESVs and affected administrations should be reached on a bilateral or multilateral basis, in accordance with the guidelines in Annexes 1 and 2;

5	that, until a decision is adopted for ESVs by WRC-03, administrations licensing ESVs that enter into bilateral or multilateral agreements under resolves 4 above should ensure that, as part of the licensing process, ESVs operate in compliance with such agreements, taking into consideration the interests of concerned neighbouring countries,

encourages concerned administrations

to cooperate with administrations that license ESVs and seek agreement underthe provisions of draft Recommendation [FSS/ESV] (WRC-03),

encourages ESV licensing administrations

to consider registering their ESV frequency assignments in the Master International Frequency Register, for information purposes only,

instructs the Secretary-General

to bring this Resolution to the attention of the Secretary-General of the International Maritime Organization.