(MOD)	USA/12/141

NOTE 4 – In all cases involving sharing with analogue systems for telephony in the FS, further coordination is only required when the pfd values are greater than or equal to the coordination threshold values in both reference bandwidths.

NOTE 5 – The pfd values specified for the band 2 160-2 200 MHz provide full protection for analogue radio-relay systems using the sharing criteria established by Recommendation ITU-R SF.357, for operation with a non-GSO MSS system employing narrow-band time division multiple access/frequency division multiple access techniques.

NOTE 6 – The pfd values specified for the band 2 483.5-2 500 MHz provide full protection for analogue radio-relay systems using the sharing criteria established by Recommendation ITU-R SF.357, for operation with multiple non-GSO MSS systems employing code division multiple access techniques. The pfd values specified will not provide full protection for existing digital fixed systems in all cases. However, these pfd values are considered to provide adequate protection for digital fixed systems designed to operate in this band, where high-power industrial, scientific and medical equipment and possible low-power applications are expected to produce a relatively high interference environment.

Reasons:	Proposing no change to these Notes, only consequential numbering changes due to the deletion of NOTE 4.