MOD	USA/12/143

TABLE  21-4     (WRC-03)

| Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| ... |  |  |  |  |  |
| 3 400-4 200 MHz 4 500-4 800 MHz 5 670-5 725 MHz (Nos. 5.453 and 5.455) 7 250-7 850 MHz | Fixed-satellite (space-to-Earth, (geostationary-satellite orbit)) Meteorological-satellite (space-to-Earth) Mobile-satellite Space research | –152 | –152 + 0.5( – 5) | –142 | 4 kHz |
| 3 700-4 200 MHz | Fixed-satellite (space-to-Earth, (non-geostationary-satellite orbit)) | –160 | –160 + 0.5( – 5) | –150 | 4 kHz |
| ... |  |  |  |  |  |

Reasons:	The FS in the 3.7-4.2 GHz band would be adequately protected by the adoption of limits on pfd produced by highly-elliptical orbit non-GSO satellites. As the levels would also adequately protect the FS from other types of non-GSO satellites, they are proposed for application to all non-GSO FSS satellites, in order to avoid having to introduce a definition of HEO satellites or otherwise subcategorize non-GSO satellites. The levels for non-GSO FSS satellites have been converted to a 4 kHz reference bandwidth from the levels of –126/–136 dB(W/m2) in 1 MHz that are reflected in the CPM Report.