NOC	USA/12/144

TABLE  21-4 (continued)

| Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| ... |  |  |  |  |  |
| 10.7-11.7 GHz | Fixed-satellite (space-to-Earth) (non-geostationary-satellite orbit) | –126 | –126 + 0.5(– 5) | –116 | 1 MHz |
| 11.7-12.5 GHz (Region 1) 12.5-12.75 GHz (Region 1 countries listed in Nos. 5.494 and 5.496) 11.7-12.7 GHz (Region 2) 11.7-12.75 GHz (Region 3) | Fixed-satellite (space-to-Earth) (non-geostationary-satellite orbit) | –124 | –124 + 0.5( – 5) | –114 | 1 MHz |
| ... |  |  |  |  |  |
| 17.7-19.3 GHz 7, 8 | Fixed-satellite (space-to-Earth) Meteorological-satellite (space-to-Earth) | –115 13 or –115 – X 12 | –115 + 0.5( – 5) 13 or –115 – X + ((10 + X)/ 20)( – 5) 12 | –105 13 or –105 12 | 1 MHz |
| ... |  |  |  |  |  |

Reasons:	The current limits and associated provisions in Section V of Article 21 that were finalized at WRC-2000 for all non-GSO FSS systems in certain bands between 10 and 30 GHz apply in full to non-GSO FSS systems in highly-elliptical orbits. No additional regulatory provisions are needed for HEO systems in these bands.