ADD	USA/12/40

1.146bis		out-of-band domain (of an emission):  The frequency range, immediately outside the necessary bandwidth but excluding the spurious domain, in which out-of-band emissions generally predominate.

Out-of-band emissions, defined based on their source, occur in the out-of-band domain and, to a lesser extent, in the spurious domain. Spurious emissions likewise may occur in the out-of-band domain as well as in the spurious domain.

1.146ter		spurious domain (of an emission):  The frequency range beyond the out-of-band domain in which spurious emissions generally predominate.

Reasons:	Adoption of these two definitions will provide a means to distinguish between frequency ranges within which the emission limits of Appendix 3, Section II either apply or do not apply.