MOD	USA/12/65

RESOLUTION  344  (Rev.WRC-03)

Management of the maritime mobile service identity numbering resource

The World Radiocommunication Conference (Geneva, 2003),

noting

a)	that the installation of digital selective calling equipment or Inmarsat B, C or M ship earth station equipment on ships participating in the Global Maritime Distress and Safety System (GMDSS) on a mandatory or voluntary basis requires the assignment of a unique nine-digit maritime mobile service identity (MMSI);

b)	that such equipment offers the possibility to connect with public telecommunication networks;

c)	that only mobile-satellite systems have been able to resolve the various billing, routing, charging and signalling requirements needed to provide full two-way automatic connectivity between ships and the international public correspondence service;

d)	that ships using the present generation of mobile-satellite ship earth stations have to be assigned an MMSI ending with three trailing zeroes in order to support automatic access to public telecommunication networks through a diallable ship telephone number whose format is compliant with ITU-T Recommendation E.164, but can only accommodate the first six digits of the MMSI;

e)	that the first three digits of a ship station MMSI form the maritime identification digits (MID), which denote the ship’s administration or geographical area of origin;

f)	that each MID only has sufficient capacity to identify 999 ships using the three trailing zero number format, with the result that the widespread use of MMSIs with three trailing zeroes rapidly exhausts the capacity of each MID,

considering

a)	that digital selective calling distress alerts require valid identities recognizable by search and rescue authorities in order to ensure a timely response;

b)	that Recommendation ITU-R M.585 contains guidance for the assignment of MMSIs, including to non-compulsory ships which communicate only with domestic radio stations; and

c)	that Recommendation ITU-R M.585 was derived from ITU-T Recommendation E.210,

recognizing

a)	that even domestic ships which install the present generation of ship earth stations operating to Inmarsat B, C or M standards will require the assignment of MMSI numbers from those numbers originally intended for ships communicating worldwide, further depleting the resource;

b)	that future growth of Inmarsat B, C or M ship earth station use by non-compulsory ships may further deplete the MMSI and MID resources;

c)	that future generations of mobile-satellite systems offering access to public telecommunication networks and participating in the Global Maritime Distress and Safety System will employ a free-form numbering system that need not include any part of the MMSI,

noting further

a)	that ITU-T has recommended that ITU-R assumes sole responsibility for managing the MMSI and MID numbering resources;b)	that ITU-R can monitor the status of the MMSI resource by monitoring, through regular reviews of the spare capacity available within the MIDs already in use, and the availableility of spare maritime identification digits (first three digits of the MMSI), taking account of regional variations,

instructs the Director of the Radiocommunication Bureau

1	to manage the allotment and distribution of the MID resource within the MMSI numbering format, taking into account:

–	Sections II, V and VI of Article 19;

–	regional variations in MMSI use;

–	spare capacity within the MID resource; and

–	the guidelines on MID and MMSI management contained in the most recent version of Recommendation ITU-R M.585, in particular as regards the reuse of MMSIs;

2	to report to each world radiocommunication conference on the use and status of the MMSI resource, noting in particular the anticipated reserve capacity and any indications of rapid exhaustion of the resource,

resolves to invite ITU-R

1	to keep under review the Recommendations for assigning MMSIs, with a view to:

–	improving the management of the MID and MMSI resources; and

–	identifying alternative resources before the resources are exhausted;

2	to consult ITU-T when addressing changes to any of the Recommendations affecting the MMSI numbering resources;

3	to complete studies on an urgent basis when a future world radiocommunication conference identifies the impending exhaustion of the MMSI resource,

instructs the Secretary-General

to communicate this Resolution to the International Maritime Organization.

Reasons:	Changes needed to Resolution 344 (WRC-97) in order to implement the new resource management responsibilities.