MOD	USA/12/79

| Receiving space radiocommunication service designation |  |  | . . . | Space research | . . . |
| --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  |  |  | 14.8-15.35 |  |
| Transmitting terrestrial service designations |  |  |  | Fixed, mobile |  |
| Method to be used |  |  |  | § 2.1, 2.2 |  |
| Modulation at earth station 1 |  |  |  | N |  |
| Earth station interference parameters and criteria | p0 (%) |  |  | 0.1 |  |
|  | n |  |  | 2 |  |
|  | p (%) |  |  | 0.05 |  |
|  | NL (dB) |  |  | 0 |  |
|  | Ms (dB) |  |  | 1 |  |
|  | W (dB) |  |  | 0 |  |
| Terrestrial station parameters | E (dBW) in B 2 | A |  | 255 |  |
|  |  | N |  | –8 |  |
|  | Pt (dBW) in B | A |  | –205 |  |
|  |  | N |  | –53 |  |
|  | Gx (dBi) |  |  | 45 |  |
| Reference band- width 6 | B (Hz) |  |  | 1 |  |
| Permissible interference power | Pr( p) (dBW) in B |  |  | –216 |  |

Reasons:	Provides the characteristics of the receiving SRS earth station in the 14.8-15.35 GHz band for coordination with transmitting fixed and mobile service stations. No change to the Table 8c notes is required.