ADD	USA/12/8

PART  C  –  Digital system

# 1	System parameters

## 1.1	Channel spacing

The initial spacing for digitally modulated emissions use shall be 10 kHz. However, interleaved channels with a separation of 5 kHz may be used in accordance with the appropriate protection criteria appearing in Recommendation [DAB] (WRC-03), provided that the interleaved emission is not to the same geographical area as either of the emissions between which it is interleaved.

## 1.2	Channel utilization

Channels using digitally modulated emissions may be commingled with analogue emissions in the same HFBC band provided the protection to the analogue emissions is at least as great as that which currently is in force with analogue-to-analogue protection. To accomplish this may require that the digital spectral power density (and total power) be lower by several dB than is currently used for the same emission circuit using either DSB or SSB emissions.

# 2	Emission characteristics

## 2.1	Bandwidth and centre frequency

A full digitally modulated emission will have a 10 kHz bandwidth with its centre frequency at any of the 5 kHz possibilities within the HFBC bands.

There are “simulcast” modes, which are a combination of analogue and digital emissions of the same programme in the same channel, that may use a digital emission of 5 kHz or 10 kHz bandwidth, next to either a 5 kHz or 10 kHz analogue emission. In all cases of this type, the 5 kHz interleaved raster used in HFBC shall be adhered to in placing the emission within the HFBC bands.

## 2.2	Frequency tolerance

The frequency tolerance shall be 10 Hz.

## 2.3	Audiofrequency band

Digital source coding within a 10 kHz bandwidth, taking account of the need for various levels of error avoidance, detection and correction coding emission mitigation, can range from the equivalent of monophonic FM (approximately 15 kHz) to low-level speech codec performance of the order of 3 kHz. The choice of audio quality is connected to the needs of the broadcaster/listener, and includes such characteristics to consider as the propagation channel conditions expected. There is no single specification, only the upper and lower bounds noted in this paragraph.

## 2.4	Modulation

Quadrature amplitude modulation (QAM) with orthogonal frequency division multiplexing (OFDM) shall be used. 64-QAM is feasible under many propagation conditions; factors of 1/2, 1/4 and perhaps 1/8 of this are specified for use when needed.

Reasons:	Part C is added to address the requirements of digital systems. Specific channelization values, audiofrequency bandwidths and “modes” of digital modulation conform to ITU-R system Recommendation ITU-R BS.1514.