ADD	USA/12/9

DRAFT  RECOMMENDATION  [DAB]  (WRC-03)

RF protection ratios associated with digitally modulated emissions in the
HF bands allocated exclusively to the broadcasting service

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that this Conference has resolved to encourage the introduction of digitally modulated emissions in the high-frequency broadcast bands allocated exclusively to the broadcasting service;

b)	that the current use of the spectrum is based on the use of double-sideband (DSB) emissions;

c)	that RF co-channel and adjacent channel protection ratios are among the fundamental parameters when determining compatibility;

d)	that this Conference has adopted Resolution 517 (Rev.WRC-03) relating to the introduction of digitally modulated emissions in the HF bands allocated exclusively to the broadcasting service;

e)	that Part C of Appendix 11 contains digital system specifications that refer to this Recommendation for matters dealing with appropriate protection ratios,

recommends

that in the application of Article 12, the protection ratios specified in the Annex to this Recommendation be used for all those cases where digitally modulated emissions operate in the same bands as double-sideband analogue emissions.

Reasons:	This Resolution provides protection ratios associated with digital emissions in the same manner as that of Recommendation 517, originally from HFBC-87, which deals with SSB and DSB only. It, along with its Annex, specifies the levels of protection required both to protect the amplitude modulation transmissions from digital modulation emissions, digital modulation emissions from other digital modulation emissions, and digital modulation emissions from amplitude modulation emissions.

ANNEX  TO  DRAFT  RECOMMENDATION  [DAB]  (WRC-03)

RF protection ratio values

1	In accordance with Resolution 517 (Rev.WRC-03) digital modulation may be used in any of the HF bands allocated exclusively to the broadcasting service. This accommodation has to be made with the appropriate amounts of protection given to both analogue and digital emissions. RF protection ratios are part of the overall regulation of these emissions. Their values appear in Table 1 in this Annex.

2	Table 1 consists of RF protection ratios for co-channel and adjacent channel conditions. The independent variable in the Table is the centre frequency separation (kHz) of any pair of emissions, wanted versus unwanted. The ratio data are in decibels.

3	The digital modulation governing these protection ratios is that which appears in summary in Part C of Appendix 11, as revised at this Conference, and the analogue modulation is DSB modulation or SSB modulation as summarized in Parts A and B, respectively, of the same Appendix.

TABLE 1

Relative RF protection ratios (dB) between broadcasting systems below 30 MHz, and digital 
(64-QAM, protection level No. 1) interfered with by digital (identical robustness modes and 
spectrum occupancy types)

| Wanted signal | Unwanted signal | Frequency separation funwanted-fwanted (kHz) |  |  |  |  |  |  |  |  | Parameters |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | BDRM (kHz) | S/I (dB) |
|  |  | –20 | –15 | –10 | –5 | 0 | 5 | 10 | 15 | 20 |  |  |
| AM | DRM_B3 | –47.2 | –41.9 | –32 | 3 | 6 | 3 | –32 | –41.9 | –47.2 | 10 | – |
| DRM_B3 | AM | –53.9 | –48 | –39.9 | –3.1 | 0 | –3.1 | –39.9 | –48 | –53.9 | 10 | 7.3 |
| DRM_B3 | DRM_B3 | –52.7 | –47 | –37.7 | –3.1 | 0 | –3.1 | –37.7 | –47 | –52.7 | 10 | 15.9 |

AM:			DSB AM signal

DRM_B3:		DRM signal, robustness mode B, spectrum occupancy type 3

BDRM:			Nominal bandwidth of DRM signal

S/I:				Signal-to-interference ratio for a BER of 10–4.

Reasons:	This Annex provides the appropriate amounts of protection given to both analogue and digital emissions and supports the text of draft Recommendation DAB (WRC-03).