MOD	EUR/13A12/6

29.9-34.2 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 32-32.3 FIXED 5.547A RADIONAVIGATION SPACE RESEARCH (deep space) (space-to-Earth) 5.547 5.547C 5.548 |  |  |

Reasons:	An incompatibility between the space research service (deep space) (space-to-Earth) and the inter-satellite service has been identified and confirmed by relevant ITU-R and CEPT studies. It has been concluded that unacceptable interference could be caused by the inter-satellite service to the space research service. The band is not used yet by the inter-satellite service so that the allocation to this service could be suppressed. There are still 700 MHz of spectrum available for the inter-satellite service in the adjacent band 32.3-33 GHz.