NOC	EUR/13A12/9

14.25-15.63 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.8-15.35 FIXED MOBILE Space research 5.339 |  |  |

Reasons:	CEPT supports an allocation to the SRS in the 26 GHz band, which satisfies the spectrum requirement for this service, and therefore there is no need to have a second additional allocation. Moreover, ITU-R studies have shown that an upgrade of the status of the allocation to the SRS in the 14.8-15.35 GHz band would induce additional constraints to the fixed service in this band. No study related to the level of unwanted emissions falling into the adjacent passive frequency band 15.35-15.4 GHz has been provided either to CEPT or to ITU.