NOC	EUR/13A16/1

1 350-1 525 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 350-1 400 FIXED MOBILE RADIOLOCATION | 1 350-1 400 RADIOLOCATION |  |
| 5.149 5.338 5.339 | 5.149 5.334 5.339 |  |
| 1 400-1 427 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 5.341 |  |  |
| 1 427-1 429 SPACE OPERATION (Earth-to-space) FIXED MOBILE except aeronautical mobile 5.341 |  |  |
| 1 429-1 452 FIXED MOBILE except aeronautical mobile | 1 429-1 452 FIXED MOBILE 5.343 |  |
| 5.341 5.342 | 5.341 |  |

Reasons:	During the WRC-03 preparation, no evidence for the need of feeder links around 1.4 GHz has been shown. Furthermore, studies in ITU-R have shown that the introduction of feeder links around 1.4 GHz would imply great difficulties for the protection of both the active and passive services having an allocation in the frequency bands between 1 350 MHz and 1 452 MHz, and that it is very unlikely that these can be solved.