| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 17 to Document 13-E |
|  |  |  | 13 March 2003 |
|  |  |  | Original: English/ French/ Spanish |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| european common proposals for the work of the conference |  |
| part 17 |  |

Agenda item 1.17  –  Radiolocation at 2 900-3 100 MHz

Introduction

The frequency band 2 900-3 100 MHz is allocated to the radionavigation service on a primary basis and to the radiolocation service on a secondary basis.

Applications in the radionavigation service in the frequency band 2 900-3 100 MHz are mainly maritime radars either on ships or on shore. Some aeronautical radar systems are operated only in a limited number of countries, in the lower part of this frequency band. Various military radar applications account for use of the radiolocation service in this band. Typical radar systems are described in Recommendation ITU-R M.1460.

There has been satisfactory co-existence for many decades, of various radar systems in the band 2 900-3 100 MHz, which include systems operating under the radionavigation and the radiolocation service. Taking this into account, together with the results of the studies undertaken within ITU-R, the following changes are proposed to the Radio Regulations, which will serve to satisfy the agenda item.

Proposal

ARTICLE  5

MOD	EUR/13A17/1

2 700-4 800 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 2 900-3 100 RADIONAVIGATION 5.426 RADIOLOCATION 5.425 5.427 |  |  |

Reasons:	Upgrading the allocation for the radiolocation service helps to fulfil the requirements of radiolocation operations.

_________________