MOD	EUR/13A24/6

TABLE  7b

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  |  | ... |  | Fixed- satellite |  | ... |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  |  |  |  | 13.75-14.3 |  |  |  |  |
| Receiving terrestrial service designations |  |  |  |  | Radiolocation radionavigation (land only) |  |  |  |  |
| Method to be used |  |  |  |  | 2.1 |  |  |  |  |
| Modulation at terrestrial station 1 |  |  |  |  | – |  |  |  |  |
| Terrestrial station interference parameters and criteria | p0 (%) |  |  | 0.01 |  |  |  |  |  |
|  | n |  |  | 1 |  |  |  |  |  |
|  | p (%) |  |  | 0.01 |  |  |  |  |  |
|  | NL (dB) |  |  | 0 |  |  |  |  |  |
|  | Ms (dB) |  |  | 1 |  |  |  |  |  |
|  | W (dB) |  |  | 0 |  |  |  |  |  |
| Terrestrial station parameters | Gx (dBi) 4 |  |  | 36 |  |  |  |  |  |
|  | Te (K) |  |  | 2 636 |  |  |  |  |  |
| Reference bandwidth | B (Hz) |  |  | 107 |  |  |  |  |  |
| Permissible interference power | Pr( p) (dBW) in B |  |  | –131 |  |  |  |  |  |
| 1 A: analogue modulation; N: digital modulation. 2 The parameters for the terrestrial station associated with transhorizon systems have been used. Line-of-sight radio-relay parameters associated with the frequency band 5 725-7 075 MHz may also be used to determine a supplementary contour with the exception that Gx  37 dBi. 3 Feeder links of non-geostationary-satellite systems in the mobile-satellite service. 4 Feeder losses are not included. 5 Actual frequency bands are 7 100-7 155 MHz and 7 190-7 235 MHz for space operation service and 7 145-7 235 MHz for the space research service. |  |  |  |  |  |  |  |  |  |