MOD	EUR/13A25/23

ARTICLE  22

TABLE  22-1B     (WRC-03)

Limits to the epfd radiated by non-geostationary-satellite systems
in the fixed-satellite service in certain frequency bands3, 6, 8

| Frequency band (GHz) | epfd (dB(W/m2)) | Percentage of time during which epfd may not be exceeded | Reference bandwidth (kHz) | Reference antenna diameter and reference radiation pattern7 |
| --- | --- | --- | --- | --- |
| 17.3-17.8 GHz in Regions 1 and 3; 17.8-18.6 in all Regions | –175.4 –175.4 –172.5 –167 –164 –164 | 0 90 99 99.714 99.971 100 | 40 | 1 m Recommendation ITU-R S.1428 |
|  | –161.4 –161.4 –158.5 –153 –150 –150 | 0 90 99 99.714 99.971 100 | 1 000 |  |
|  | –178.4 –178.4 –171.4 –170.5 –166 –164 –164 | 0 99.4 99.9 99.913 99.971 99.977 100 | 40 | 2 m Recommendation ITU-R S.1428 |
|  | –164.4 –164.4 –157.4 –156.5 –152 –150 –150 | 0 99.4 99.9 99.913 99.971 99.977 100 | 1 000 |  |
|  | –185.4 –185.4 –180 –180 –172 –164 –164 | 0 99.8 99.8 99.943 99.943 99.998 100 | 40 | 5 m Recommendation ITU-R S.1428 |
|  | –171.4 –171.4 –166 –166 –158 –150 –150 | 0 99.8 99.8 99.943 99.943 99.998 100 | 1 000 |  |

Reasons:	These changes are intended to quantify No. 22.2. Moreover, as far as Table 22-3 is concerned, the modification proposed also aims at protecting the feeder-links operating in accordance with the Plans of Appendix 30A (see No. 22.5F.2). The extension of the limits to the 17.7-17.8 GHz band in Regions 1 and 3 aims at keeping the consistency in the whole 17.3-18.6 GHz band. With the new allocation of FSS (space-to-Earth), the regulatory situation with respect to FSS (space-to-Earth) and BSS would be identical in the whole 17.3-18.6 GHz band: there would be, then, no justification to have no limit only in the 17.7-17.8 GHz band.