MOD	EUR/13A26/3

14.25-15.63 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.25-14.3 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.ESV ADD 5.ESV3 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite ADD 5.ESV2 Space research 5.505 5.508 5.509 |  |  |
| 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.ESV ADD 5.ESV3 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite ADD 5.ESV2 Radionavigation-satellite | 14.3-14.4 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.ESV ADD 5.ESV3 Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite ADD 5.ESV2 Radionavigation-satellite | 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.ESV ADD 5.ESV3 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite ADD 5.ESV2 Radionavigation-satellite |
| 14.4-14.47 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.ESV ADD 5.ESV3 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite ADD 5.ESV2 Space research (space-to-Earth) |  |  |
| 14.47-14.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.ESV ADD 5.ESV3 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite ADD 5.ESV2 Radio astronomy 5.149 |  |  |

Reasons:	To apply footnotes 5.ESV, 5.ESV2 and 5.ESV3 in the band 14.25-14.50 GHz.