MOD	EUR/13A27/36

ANNEX  7     (WRC-03)

Orbital position limitations

A	In applying the procedure of Article 4 for proposed modifications to the Region 2 Plan or for proposed new or modified assignments in the Regions 1 and 3 List, administrations should observe the following criteria:

1)	No broadcasting satellite serving an area in Region 1 and using a frequency in the band 11.7-12.2 GHz shall occupy a nominal orbital position further west than 37.2 W or further east than 146° E.

2)	No broadcasting satellite serving an area in Region 2 that involves an orbital position different from that contained in the Region 2 Plan shall occupy a nominal orbital position:

a)	further east than 54 W in the band 12.5-12.7 GHz; or

b)	further east than 44 W in the band 12.2-12.5 GHz; or

c)	further west than 175.2 W in the band 12.2-12.7 GHz.

However, modifications necessary to resolve possible incompatibilities during the incorporation of the Regions 1 and 3 feeder-link Plan into the Radio Regulations shall be permitted.

3)	The purpose of the following orbital position and e.i.r.p. limitations is to preserve access to the geostationary-satellite orbit by the Region 2 fixed-satellite service in the band 11.7-12.2 GHz. Within the orbital arc of the geostationary-satellite orbit between 37.2 W and 10° E, the orbital position associated with any proposed new or modified assignment in the Regions 1 and 3 List of additional uses shall lie within one of the portions of the orbital arc listed in Table 1. The e.i.r.p. of such assignments shall not exceed 56 dBW, except at the positions listed in Table 2.

TABLE  1

Allowable portions of the orbital arc between 37.2°W and 10° E for new or modified 
assignments in the Regions 1 and 3 Plan and List

| Orbital position | 37.2° W to 36° W | 33.5° W to 32.5° W | 30° W to 29° W | 26° W to 24° W | 20° W to 18° W | 14° W to 12° W | 8° W to 6° W | 4° W 1 | 2° W to 0° | 4° E to 6° E | 9° E 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 Proposed new or modified assignments in the List which involve this orbital position shall not exceed the pfd limit –138 dB(W/(m2  27 MHz)) at any point in Region 2. |  |  |  |  |  |  |  |  |  |  |  |

TABLE  2

Nominal positions in the orbital arc between 37.2° W and 10° E at which the 
e.i.r.p. may exceed the limit of 56 dBW

| Orbital position | 37° W  0.2° | 33.5° W | 30° W | 25° W  0.2° | 19° W  0.2° | 13° W  0.2° | 7° W  0.2° | 4° W 1 | 1° W  0.2° | 5° E  0.2° | 9° E 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 Proposed new or modified assignments in the List which involve this orbital position shall not exceed the pfd limit –138 dB(W/(m2  27 MHz)) at any point in Region 2. |  |  |  |  |  |  |  |  |  |  |  |

B	The Region 2 Plan is based on the grouping of the space stations in nominal orbital positions of  0.2º from the centre of the cluster of satellites. Administrations may locate those satellites within a cluster at any orbital position within that cluster, provided they obtain the agreement of administrations having assignments to space stations in the same cluster. (See § 4.13.1 of Annex 3 to Appendix 30A.)

Reasons:	The current text of Annex 7 to Appendix 30 requires some alignments with other provisions contained in Appendix 30, in particular with regard to required references to the Regions 1 and 3 List.