MOD	EUR/13A27/56

4.1.20	Should harmful interference be caused by an assignment included in the feeder-link List under § 4.1.18 to any recorded assignment in the Master Register which was the basis of the disagreement and is not subject to a Plan or is operated in conformity with the feeder-link List, as appropriate, the administration using the frequency assignment included in the feeder-link List under § 4.1.18 shall, upon receipt of advice thereof, immediately eliminate this harmful interference. Should interference in excess of the applicable levels in Annex 1 be caused by an assignment included in the feeder-link List under § 4.1.18 to an assignment which was the basis of the disagreement and is operated in conformity with the feeder-link Plan, the administration using the frequency assignment included in the feeder-link List under § 4.1.18 shall, upon receipt of advice thereof, immediately eliminate this interference.

Reasons:	To replace a non-quantified undertaking (eliminate “harmful interference”) by a quantified one (the criterion used in applying § 4.1.5).

Sub-Part 27D – Difficulties encountered by BR in the application of 
Appendices 30 and 30A

Introduction

In reviewing the Preliminary Report of BR to WRC-03 under agenda item 7.1 (part dealing with the experiences of BR in applying the Radio Regulations, including the difficulties and inconsistencies encountered in the application of the relevant provisions), the Special Committee and the CPM addressed a number of issues and reached some conclusions which are reported in Sections 3.2.3.6.1 to 3.2.3.6.10 of the CPM Report.

Europe proposes the following modifications to Appendices 30 and 30A to solve the some of the issues mentioned above.

Proposals

APPENDIX  30*  (WRC-2000)

Provisions for all services and associated Plans and List for
the broadcasting-satellite service in the frequency bands
11.7-12.2 GHz (in Region 3), 11.7-12.5 GHz (in Region 1)
             and 12.2-12.7 GHz (in Region 2)     (WRC-2000)

ARTICLE  2

Frequency bands