MOD	EUR/13A27/61

2.2	The use of the guardbands of the Plans in this Appendix, as defined in § 3.1 and 4.1 of Annex 3, to provide space operations functions in accordance with No. 1.23 in support of the operation of geostationary-satellite networks broadcasting-satellite service, shall be coordinated with the assignments subject to these Plans using the provisions of Article 7. Coordination among assignments intended to provide these functions and services not subject to a Plan shall be effected using the provisions of No. 9.7 and the associated provisions of Articles 9 and 11. Advance publication information is not required. These assignments shall be brought into use within eight years from the date of receipt by the Bureau of the complete Appendix 4 information3bis. Coordination of modifications to the Region 2 feeder-link Plan or assignments to be included in the Regions 1 and 3 feeder-link Lists, with assignments intended to provide these functions shall be effected using § 4.1.1 d) of Article 4.     (WRC-03)