MOD	EUR/13A27/12

# 5	Limits applicable to protect a frequency assignment in the bands 17.3-18.1 GHz (Regions 1 and 3) and 17.3-17.8 GHz (Region 2) to a receiving space station in the fixed-satellite service (Earth-to-space)

An administration in Region 1 or 3 is considered affected by a proposed modification in Region 2, with respect to § 4.2.2 a) or 4.2.2 b) of Article 4, or an administration in Region 2 shall be considered affected by a proposed new or modified assignment in the Regions 1 and 3 feeder-link List, with respect to § 4.1.1 c) of Article 4, when the power flux-density arriving at the receiving space station of a broadcasting-satellite feeder-link would cause an increase in the noise temperature of the feeder-link space station which exceeds the threshold value of  T / T corresponding to 6%, where  T / T is calculated in accordance with the method given in Appendix 8, except that the maximum power densities per hertz averaged over the worst 1 MHz are replaced by power densities per hertz averaged over the necessary bandwidth of the feeder-link carriers.     (WRC-03)

Interim systems of Region 2 in accordance with Resolution 42 (Rev.Orb-88) shall not be taken into consideration when applying this provision to proposed new or modified assignments in the Regions 1 and 3 feeder-link List. However, this provision shall be applied to Region 2 interim systems with respect to Regions 1 and 3 administrations in accordance with § 5.2 b) of Resolution 42.     (WRC-03)