MOD	EUR/13A27/13

# 6	Limits applicable to protect a frequency assignment in the band 17.8-18.1 GHz (Region 2) to a receiving feeder-link space station in the fixed-satellite service (Earth-to-space)     (WRC-2000)

With respect to § 4.1.1 d) of Article 4, an administration in Region 2 is considered affected by a proposed new or modified assignment in the Regions 1 and 3 feeder-link List when the power flux-density arriving at the Region 2 receiving space station of a broadcasting-satellite feeder-link would cause an increase in the noise temperature of the receiving feeder-link space station which exceeds the threshold value of T/T corresponding to 6%, where T/T is calculated in accordance with the method given in Appendix 8, except that the maximum power densities per hertz averaged over the worst 1 MHz are replaced by power densities per hertz averaged over the necessary bandwidth of the feeder-link carriers.     (WRC-03)

Reasons:	Harmonization of the inter-Regional criteria with a common T/T value of 6%.

ANNEX  4     (WRC-2000)

Criteria for sharing between services