MOD	EUR/13A27/14

# 1	Threshold values for determining when coordination is required between transmitting space stations in the fixed-satellite service or the broadcasting-satellite service and a receiving space station in the feeder-link Plan or List, or a proposed new or modified receiving space station in the List in the frequency bands 17.3-18.1 GHz (Regions 1 and 3) and in the feeder-link Plan or a proposed modification to the Plan in the frequency band 17.3-17.8 GHz (Region 2)

With respect to § 7.1, Article 7, coordination of a transmitting space station in the fixed-satellite service or in the broadcasting-satellite service with a receiving space station in a broadcasting-satellite service feeder link in the Regions 1 and 3 feeder-link Plan or List, or a proposed new or modified receiving space station in the List, or in the Region 2 feeder-link Plan or proposed modification to the Plan is required when the power flux-density arriving at the receiving space station of a broadcasting-satellite service feeder link of another administration would cause an increase in the noise temperature of the feeder-link space station which exceeds a threshold value of Ts / Ts corresponding to 6%. Ts / Ts is calculated in accordance with Case II of the method given in Appendix 8.