MOD	EUR/13A27/16

TABLE  5-1 (end)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.19 Terrestrial, GSO, non-GSO/ GSO, non-GSO | Any transmitting station of a terrestrial service or a transmitting earth station in the FSS (Earth-to-space) in a frequency band shared on an equal primary basis with the BSS, with respect to typical earth stations included in the service area of a space station in the BSS | Bands listed in No. 9.11 and the band 11.7-12.7 GHz | i) Necessary bandwidths overlap; and ii) the transmitting terrestrial station or the transmitting FSS earth station is located within the coordination area | i) Check by using the assigned frequencies and bandwidths ii) Appendix 7 (§ 1.4.5) | See also Article 6 of Appendix 30 |
| No. 9.21 Terrestrial, GSO, non-GSO/ terrestrial, GSO, non-GSO | A station of a service for which the requirement to obtain the agreement of other administrations is included in a footnote to the Table of Frequency Allocations referring to No. 9.21 | Band(s) indicated in the relevant footnote | Incompatibility established by the use of Appendices 7, 8, technical Annexes of Appendices 30 or 30A, pfd values specified in some of the footnotes, other technical provisions of the Radio Regulations or ITU-R Recommendations, as appropriate | Methods specified in, or adapted from, Appendices 7, 8, 30, 30A, other technical provisions of the Radio Regulations or ITU-R Recommendations |  |

APPENDIX  7  (WRC-2000)

Methods for the determination of the coordination area around an earth station in frequency bands between 100 MHz and 105 GHz