ADD	EUR/13A28/3

Draft  RESOLUTION  [EUR/13A28-1]  (WRC-03)

Use of the band 108-117.975 MHz by aeronautical services other than 
the aeronautical radionavigation service

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	the current allocation of the frequency band 108-117.975 MHz to the aeronautical radionavigation service;

b)	the current requirements of the broadcasting service operating in the frequency band 87-108 MHz;

c)	the need for the aeronautical community to provide additional services in order to enhance navigation and surveillance systems through a communications datalink,

recognizing

a)	that precedence must be given to the aeronautical radionavigation service operating in the frequency band 108-117.975 MHz;

b)	that, in accordance with Annex 10 of the Convention of the International Civil Aviation Organization (ICAO), all aeronautical systems must meet Standards and Recommended Practices (SARPs) requirements;

c)	that within ITU-R, compatibility criteria between the broadcasting service operating in the frequency band 87-108 MHz and the aeronautical radionavigation service operating in the frequency band 108-117.975 MHz already exist as indicated in Recommendation ITU-R SM.1009;

d)	that all compatibility issues between FM broadcasting systems and ICAO standard ground-based systems for the transmission of radionavigation-satellite differential correction signals have been addressed,

noting

a)	that aeronautical systems are converging towards a communications datalink environment to support aeronautical navigation and surveillance functions, which need to be accommodated in existing radio spectrum;

b)	that no compatibility criteria currently exist between the broadcasting service operating in the frequency band 87-108 MHz and the planned additional aeronautical systems in the adjacent band 108-117.975 MHz using aircraft transmission;

c)	that studies by ICAO have confirmed that the planned additional aeronautical systems are compatible with the existing use of the band 108-117.975 MHz,

resolves

1	that the provisions of this Resolution and of No. 5.YYY shall enter into force on [4 July 2003];

2	that any additional aeronautical systems planned to operate in the frequency band 108-117.975 MHz shall, as a minimum, meet the FM broadcasting immunity requirements contained in Annex 10 of the Convention of the International Civil Aviation Organization (ICAO) for existing aeronautical radionavigation systems operating in this frequency band;

3	that additional aeronautical systems operating in the band 108-117.975 MHz shall place no additional constraints on stations of the broadcasting service operating in the bands allocated to this service in the frequency range 87-108 MHz;

4	that frequencies below 112 MHz shall not be used for these additional aeronautical systems until all compatibility issues with the lower adjacent frequency band 87-108 MHz have been considered,

invites ITU-R

1	to study any compatibility issues between the broadcasting and aeronautical services that may arise from the introduction of these additional aeronautical systems as referenced in noting b) and to develop new or revised ITU-R Recommendations as appropriate;

2	to bring the results of these studies, if necessary, to the attention of a future competent WRC to determine any further action required,

requests the Secretary-General

to bring this Resolution to the attention of ICAO.