MOD	EUR/13A30/2

9.5D		If the information under No. 9.30 has not been received by the Bureau within a period of 24 months after the date of receipt by the Bureau of the relevant complete information under No. 9.1, the information published under No 9.2B and not covered by a coordination request under No. 9.30 shall be cancelled, after the administration concerned has been informed at least three months before the end of the 24-month period. The Bureau shall also publish the cancellation in its BR IFIC.

Sub-part 30C

Provisions of No. 9.21

Introduction

The provisions of No. 9.21 significantly differ from their current application through the associated Rules of Procedures. Namely, the Rule of Procedure on No. 11.31.1 allows an assignment subject to No. 9.21 to be recorded with a favourable finding under No. 11.31 even when all agreements have not been obtained, except with respect to the administrations still objecting to the request for agreement. Therefore, pursuant to this Rule of Procedure, the meaning of No. 11.31.1 has been considerably altered, as well as some other provisions of Article 11 and Appendix 5.

The Rules of Procedure have introduced the concept of partial conformity with the Table of Frequency Allocations. If this concept is to be kept, there is a need to reflect it in the Radio Regulations. This requires modifications to Article 11 and Appendix 5.

Proposals

ARTICLE  11

Notification and recording of frequency assignments1, 2, 3, 4

Section II  –  Examination of notices and recording of frequency assignments
in the Master Register