MOD	EUR/13A31/17

TABLE  5-2

| Frequency band (MHz) | Terrestrial service to be protected | Coordination threshold values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | GSO space stations |  | Non-GSO space stations |  |  |
|  |  | pfd (per space station) calculation factors (NOTE 2) |  | pfd (per space station) calculation factors (NOTE 2) |  | % FDP (in 1 MHz) (NOTE 1) |
|  |  | P | r dB/ degrees | P | r dB/ degrees |  |
| 1 518-1 525 | Analogue FS telephony (NOTE 5) | – 146 dB(W/m2) in 4 kHz and – 128 dB(W/m2) in 1 MHz | 0.5 | – 146 dB(W/m2) in 4 kHz and – 128 dB(W/m2) in 1 MHz | 0.5 |  |
|  | All other cases (NOTE 4) | – 128 dB(W/m2) in 1 MHz | 0.5 | – 128 dB(W/m2) in 1 MHz | 0.5 | 25 |

NOTE 4 – Exceptions for the band 1 518-1 525 MHz are as follows: 4.1 For the land mobile service on the territory of Japan (No. 5.348A): – 150 dB(W/m2) in 4 kHz at all angles of arrival is applicable to all satellite space-to-Earth emissions. 4.2 For the aeronautical mobile service for telemetry on the territory of the United States, the requirement for coordination is determined by the following pfd thresholds, in 4 kHz: –156 dB(W/m2) for 0    4 –156.4  0.111 dB(W/m2) for 4º <   60º –150 dB(W/m2) for 60   90 where  is the angle of arrival (degrees). 4.3 For the aeronautical mobile service for telemetry on the territory of the administrations listed in No. 5.342: –140 dB(W/m2) in 4 kHz at all angles of arrival.

Reasons:	To define the pfd coordination thresholds for the protection of terrestrial services.

APPENDIX  7  (WRC-2000)