MOD	EUR/13A31/19

TABLE  8a

Parameters required for the determination of coordination distance for a receiving earth station

| Receiving space radiocommunication service designation |  |  | ... | Mobile-satellite, land-mobile satellite, maritime mobile- satellite |
| --- | --- | --- | --- | --- |
| Frequency bands (MHz) |  |  |  | 1 518-1 530 1 555-1 559 2 160-2 200 1 |
| Transmitting terrestrial service designations |  |  |  | Fixed, mobile |
| Method to be used |  |  |  | § 1.4.6 |
| Modulation at earth station 2 |  |  |  | N |
| Earth station interference parameters and criteria | p0 (%) |  |  | 10 |
|  | n |  |  | 1 |
|  | p (%) |  |  | 10 |
|  | NL (dB) |  |  | 0 |
|  | Ms (dB) |  |  | 1 |
|  | W (dB) |  |  | 0 |
| Terrestrial station parameters | E (dBW) in B 3 | A |  | 37 4 |
|  |  | N |  | 37 |
|  | Pt (dBW) in B | A |  | 0 |
|  |  | N |  | 0 |
|  | Gx (dBi) |  |  | 37 |
| Reference bandwidth | B (Hz) |  |  | 4  103 |
| Permissible interference power | Pr( p) (dBW) in B |  |  | –176 |
| 1 In the band 2 160-2 200 MHz, the terrestrial station parameters of line-of-sight radio-relay systems have been used. If an administration believes that, in this band transhorizon systems need to be considered, the parameters associated with the frequency band 2 500-2 690 MHz may be used to determine the coordination area. 2 A: analogue modulation; N: digital modulation. 3 E is defined as the equivalent isotropically radiated power of the interfering terrestrial station in the reference bandwidth. 4 This value is reduced from the nominal value of 50 dBW for the purposes of determination of coordination area, recognizing the low probability of high power emissions falling fully within the relatively narrow bandwidth of the earth station. 5 The fixed-service parameters provided in the column for 163-167 MHz and 272-273 MHz are only applicable to the band 163-167 MHz. |  |  |  |  |

Reasons:	Consequential modifications to the frequency range in the tables of Appendix 7.