ADD	EUR/13A31/23

DRAFT  RESOLUTION  [EUR/13A31-1]  [MMM]  (WRC-03)

Sharing between the mobile-satellite service (Earth-to-space) and 
the space research (passive) service in the band 1 668-1 668.4 MHz 
and between the mobile-satellite service (Earth-to-space) and 
the mobile service in the band 1 670-1 675 MHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that WRC-03 made a new global allocation to the MSS (Earth-to-space) in the band 1 668-1 675 MHz;

b)	that the band 1 660.5-1 668.4 MHz is allocated to the space research (passive) service;

c)	that in the band 1 668-1 668.4 MHz, mobile earth stations and space research (passive) stations are subject to coordination under No. 9.11A;

d)	that the band 1 670-1 675 MHz is allocated to the mobile service and that No. 5.380 identifies the band for aeronautical public correspondence systems but there are no existing or future plans for implementation of such systems;

e)	that sharing between mobile systems other than those referred to in No. 5.380 and the mobile-satellite service in the band 1 670-1 675 MHz has not been studied since mobile service systems characteristics were not available;

f)	that MSS systems in the band 1 668-1 675 MHz are not expected to become operational prior to 2007,

resolves to invite ITU-R

1	to complete, as a matter of urgency and in time for WRC-07, studies relating to provisions to protect space research (passive) space stations from harmful interference from mobile earth stations in the band 1 668-1 668.4 MHz, while avoiding undue constraints on either service;

2	to investigate, as a matter of urgency and in time for WRC-07, the use of the band 1 670-1 675 MHz by the mobile service and to complete any relevant sharing studies between the mobile service and the mobile-satellite service in this band;

3	to bring the results of these studies to the attention of WRC-07,

urges

administrations and interested parties to participate actively in these studies,

recommends

that WRC-07 takes appropriate action based on the results of those studies.

Reasons:	To initiate the necessary ITU-R studies into sharing between MSS (Earth-to-space) and the space research (passive) service and between MSS (Earth-to-space) and the mobile service, and to enable WRC-07 to take appropriate action.