MOD	EUR/13A34/4

TABLE  21-4     (WRC-03)

| Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| .... |  |  |  |  |  |
| 2 500-2 690 MHz 2 520-2 670 MHz 2 500-2 516.5 MHz (No. 5.404) | Fixed-satellite Broadcasting-satellite9A Radiodetermination-satellite | –152 9 | –152 + 0.75( – 5) 9 | –137 9 | 4 kHz |
| 2 630-2 655 MHz (No. 5.418) | Broadcasting-satellite (sound) (non-geostationary-satellite orbit) | –128 9B, 9C | –128 + 0.75( – 5) 9B, 9C | –113 9B, 9C | 1 MHz |
| 2 630-2 655 MHz (No. 5.418) | Broadcasting-satellite (sound) (non-geostationary-satellite orbit) | –132 9B, 9D | –132 + 0.5( – 5) 9B, 9D | –122 9B, 9D | 1 MHz |