MOD	EUR/13A35/1

5	This assignment shall be brought into use only when the limits given in Annex 1 to this Appendix are not exceeded or with the agreement of the administrations identified in Table 2, whose networks/beams listed in this Table may be affected, with respect to (see also the Note to § 11.2):

a)	assignments in the Region 2 Plan on 12 May 2000; or

b)	assignments in the terrestrial services which are recorded in the Master Register with a favourable finding or received by the Bureau prior to 12 May 2000 for recording in the Master Register and which subsequently receive a favourable finding based on the Plan as it existed on 12 May 2000; or

c)	assignments in the fixed-satellite service which:

–	are in conformity with No. 11.31; and

–	either are recorded in the Master Register prior to 12 May 2000 with a favourable finding; or

–	have been coordinated under the provisions of No. 9.7 (or No. 1060) or § 7.2.1 of Article 7 prior to 12 May 2000; or

–	are in process of coordination under the provisions of No. 9.7 (or No. 1060) or § 7.2.1 of Article 7 prior to 31 July 2000 for which complete Appendix 4 data (or Appendix 3 data, as appropriate) have been received by the Bureau under the relevant provisions of Article 9 (or Article 11, as appropriate):

–	filings received by the Bureau prior to 12 May 2000 at 1700 h (Istanbul time) shall be taken into account in the pertinent compatibility analysis to be carried out by the Bureau after WRC-2000 by applying the pfd criteria shown in Table 1; or

–	filings received by the Bureau after 12 May 2000 at 1700 h (Istanbul time), but before 31 July 2000, shall be taken into account by applying the sharing criteria of _138 dB(W/(m2 · 27 MHz)) or the pfd criteria shown in Table 1, whichever is higher.

6	This assignment shall not claim protection from interference caused by the assignments which pertain to network/beams identified in Table 3 which are in conformity with the Region 2 Plan on 12 May 2000 (see also the Note to § 11.2).

7	This assignment shall not claim protection from interference caused by assignments in the fixed-satellite service which pertain to networks/beams identified in Table 2 (see also the Note to § 11.2) and:

a)	are in conformity with No. 11.31; and

b)	either are recorded in the Master Register with a favourable finding prior to 12 May 2000 or;

c)	for which complete Appendix 4 data (or Appendix 3 data, as appropriate) under the relevant provisions of Article 9 (or No. 1060, or § 7.2.1 of Article 7, as appropriate) have been received prior to 12 May 2000, which have been brought into use prior to 12 May 2000 and for which the complete due diligence information, in accordance with Annex 2 to Resolution 49 (WRC-97), has been received prior to 12 May 2000.

8	This assignment shall not claim protection from the assignments of administrations for terrestrial services which are recorded in the Master Register with a favourable finding, or received by the Bureau prior to 12 May 2000 for recording in the Master Register and which subsequently receive a favourable finding based on the Plan as it existed on 12 May 2000 (see also the Note to § 11.2).

9	Provisional beam. These assignments have been included in the Plan by WRC-2000. These assignments are for exclusive use by East Timor.

TABLE  1

Criteria used in Note 5 c) to identify affected assignments/administrations

| Symbol | Criteria |
| --- | --- |
| c | For Regions 1 and 3 broadcasting-satellite service  Region 2 fixed-satellite service: –160 dB(W/(m2 · 27 MHz)) 0    0.054 –137.46 + 17.74 log  dB(W/(m2 · 27 MHz)) 0.054    3.67 –141.56 + 25 log  dB(W/(m2 · 27 MHz)) 3.67    11.54 –115 dB(W/(m2 · 27 MHz)) 11.54   For Region 1 broadcasting-satellite service  Region 3 fixed-satellite service: –160 dB(W/(m2 · 27 MHz)) 0    0.054 –137.46 + 17.74 log  dB(W/(m2 · 27 MHz)) 0.054    3.67 –141.56 + 25 log  dB(W/(m2 · 27 MHz)) 3.67    24.12 1 –107 dB(W/(m2 · 27 MHz)) 24.12   1 where  corresponds to the minimum geocentric angular separation taking into account the pertinent station-keeping accuracy of the interfering broadcasting-satellite service and the interfered-with fixed-satellite service space stations. |
| 1 For the purpose of analysing the WRC-2000 Plan. |  |