MOD	EUR/13A37/16

Section II  –  Control of interference to geostationary-satellite systems

22.5C	§ 6	1)	The equivalent power flux-density2, epfd, at any point on the Earth’s surface visible from the geostationary-satellite orbit, produced by emissions from all the space stations of a non-geostationary-satellite system in the fixed-satellite service in the frequency bands listed in Tables 22-1A to 22-1E, including emissions from a reflecting satellite, for all conditions and for all methods of modulation, shall not exceed the limits given in Tables 22-1A to 22-1E for the given percentages of time. These limits relate to the equivalent power flux-density which would be obtained under free-space propagation conditions, into a reference antenna and in the reference bandwidth specified in Tables 22-1A to 22-1E, for all pointing directions towards the geostationary-satellite orbit.     (WRC-03)

Reasons:	To insert the reference to the new epfd limits in the 6/4 GHz range.